***Engineer***

1. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

2. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above

3. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

4. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

5. A simply supported wooden beam 150 cm long and having a cross section 16 cm x 24 cm carries a concentrated load, at the centre. If the permissible stress ft = 75 kg/cm2 and fs = 10 kg/cm2 the safe load is

a. 3025 kg

b. 3050 kg

c. 3075 kg

d. 3100 kg.

6. A relatively fixed point of known elevation above datum, is called

a. bench mark

b. datum point

c. reduced level

d. reference point.

7. To orient a plane table at a point P roughly south of the mid-point of two inaccessible conical hill stations A and B in the plains, a point C is selected in line with AB and table is oriented at C by bringing ab in line with AB. A ray is then drawn towards P and at P the table is oriented by back ray method. The orientation so obtained, is

a. unique and correct

b. incorrect

c. manifold and correct

d. not reliable.

8. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

9. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

10. The difference in the lengths of an arc and its subtended chord on the earth surface for a distance of 18.2 km, is only

a. 1 cm

b. 5 cm

c. 10 cm

d. 100 cm.

11. Flint glass

a. has slightly the greater refracting power than crown glass

b. has roughly double refracting power than that of crown

c. and crown glass proportions yield the required focal length and neutralise the dispersion produced by the convex lens at the emergence from the concave

d. all the above.

12. During secular variation of magnetic meridian at different places

a. range of oscillations is constant

b. period of oscillation is constant

c. range and period of oscillation both vary

d. period of oscillation only varies.

13. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

14. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

15. A badly mixed cement concrete results in

a. segregation

b. bleeding

c. honey combing

d. none to these.

16. Pick up the correct statement from the following:

a. Steel produced by open hearth process is milder than that obtained by the bessemer process

b. Engineers prefer open hearth steel for structural purpose as it is more homogenous

c. Basic Bessemer process is suitable for converting poor ore containing a large proportion of sulphur and phosphorus into steel

d. all the above.

17. Commonly used thinner in

a. cellulose paints is ethyle acetate

b. oil paints, is naptha

c. distemper, is water

d. all the above.

18. In paints, the pigment is responsible for

a. colour

b. smoothness

c. glassy face

d. none of these.

19. According to I.S. : 456, the number of grades of concrete mixes, is

a. 4

b. 5

c. 6

d. 7

20. The impurity of mixing water which affects the setting time and strength of concrete, is

a. sodium chlorides

b. sodium carbonates and bicarbonates

c. calcium chorides

d. calcium bicarbonates

21. If the effective plan area of a warehouse is 54 sq. m, and maximum height of piles permitted is 270 cm, the number of cement bags to be stored, is

a. 2200 bags

b. 2400 bags

c. 2700 bags

d. 3000 bags

22. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

23. Expansion joints are provided if the length of concrete structures exceeds

a. 15 m

b. 15 m

c. 35 m

d. 45 m

24. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

25. A decrease in water content results in a reduction of the volume of a soil in

a. liquid state

b. plastic state

c. semi solid state

d. all of these.

26. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

27. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

28. A soil has bulk density 2.30 g/cm3 and water content 15 per cent, the dry density of the sample, is

a. 1.0 g/cm2

b. 1.5 g/cm3

c. 2.0 g/cm3

d. 2.5 g/cm3

29. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

30. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

31. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

32. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

33. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

34. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

35. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

36. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

37. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

38. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

39. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

40. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

41. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

42. The paper size of A, is

a. 297 x 420

b. 210 x 297

c. 420x 850

d. none of the above

43. Vertical lines are drawn

a. top to bottom

b. bottom to top

c. both (a) and (b) above

d. none of the above

44. A 100 m long line making 30° with the vertical, show its projection on horizontal plane

a. 100 m

b. 50 m

c. 86.6 m

d. 70.71 m

45. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

46. Pick up the correct statement from the following:

a. An annuity is a series of equal payments occurring at equal period of time.

b. Annuity is called an equal payment or uniform payment series.

c. An annuity may have periods of time of any length but should always be of equal length.

d. All the above.

47. Which one of the following statements is correct?

a. The number of years required to recover the initial cash investment in a project, is called Pay Back period (PBP).

b. The discount rate that equates the present value of the expected Net Cash Flows (CFs) with the Initial Cash Outflow (ICO) is known as internal rate of return.

c. The present value of the proposal's net cash flows, less the proposal's initial cash outflow is known as the Net Present Value (NPV)

d. All of these

48. The minimum and maximum no. of members in the user's committee should be

a. 15&17

b. 17&19

c. 9& 15

d. 7&9

49. Any Non-Nepali Engineer desiring to work in Nepal may apply for registration under:

a. Category A

b. Category B

c. Category C

d. Category D

50. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid