***Engineer***

1. The equivalent length of a column of length L, having both the ends hinged, is

a. 2L

b. L

c. L/2

d. 3L

2. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse.

3. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

4. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

5. The moment diagram for a cantilever which is subjected to a uniformly distributed load will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

6. In setting up a plane table at any station

a. levelling is done first

b. centering is done first

c. both levelling and centering are done simultaneously

d. orientation is done first.

7. In case of a direct vernier scale

a. graduations increase in opposite direction in which graduations of the main scale increase

b. smallest division is longer than smallest division of the main scale

c. graduations increase in the same direction in which graduations of the main scale increase

d. none of these.

8. A transit is oriented by setting its vernier A to read the back azimuth of the preceding line. A back sight on the preceding transit station taken and transit is rotated about its vertical axis. The vernier A reads

a. azimuth of the forward line

b. bearing of the. forward line

c. back bearing of the forward line

d. equal to 360°-azimuth of the forward line.

9. The representation of general topography of a very flat terrain is possible only

a. by drawing contours at large interval

b. by drawing contours at small interval

c. by giving spot levels at large interval

d. by giving spot levels to salient features at close interval.

10. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

11. While working on a plane table, the correct rule is :

a. Draw continuous lines from all instrument stations

b. Draw short rays sufficient to contain the points sought

c. Intersection should be obtained by actually drawing second rays

d. Take maximum number of sights as possible from each station to distant objects.

12. Deviation of the actual road gradient from the proposed contou'r gradient up hill side, involves

a. embankment on the centre line

b. excavation on the centre line

c. earth work on the centre line

d. none of these.

13. The rock generally used for roofing, is

a. granite

b. basalt

c. slate

d. pumice.

14. German silver is an alloy of

a. silver, gold and lead

b. copper, nickel and zinc

c. copper, brass and zinc

d. brass, silver and zinc.

15. Stones used for rubble masonry must be

a. soft

b. hard

c. light

d. heavy.

16. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

17. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

18. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

19. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

20. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

21. In slump test, each layer of concrete is compacted by a steel rod 60 cm long and of 16 mm diameter for

a. 25 times

b. 30 times

c. 40 times

d. 50 times

22. Pick up the correct statement from the following:

a. According to the petrological characteristics, concrete aggregates are classified as heavy weight, normal weight and light weight

b. According to the shape of the particles, concrete aggregates are classified as rounded irregular, angular and flaky

c. According to the surface texture of the particles, the concrete aggregates are classified as glassy, smooth, granular, rough, crystalline, honey combed and porous

d. All the above.

23. An ordinary Portland cement when tested for its fineness, should not leave any residue on I.S. seive No. 9, more than

a. 10%

b. 15%

c. 20%

d. 25%

24. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

25. The bearing capacity of a soil depends upon

a. shape of the particles

b. cohesive properties of particles

c. internal frictional resistance of particles

d. all the above.

26. The void ratio of a soil sample decreases from 1.50 to 1.25 when the pressure is increased from 25 tonnes/m2 to 50 tonnes/m2, the coefficient of compressibility is

a. 0.01

b. 0.02

c. 0.05

d. 0.001

27. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

28. Chemical weathering of soil is caused due to

a. carbonation

b. hydration

c. leaching

d. all the above.

29. In a liquid limit test, the moisture content at 10 blows was 70% and that at 100 blows was 20%. The liquid limit of the soil, is

a. 35%

b. 50%

c. 65%

d. none of these.

30. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

31. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

32. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

33. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

34. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

35. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

36. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

37. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

38. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

39. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

40. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

41. The point of intersection of co-ordinate axis is called

a. zero-zero

b. origin

c. cross point

d. all of the above

42. Free hand sketch is normally used for

a. concept

b. final

c. discussion

d. none of the above

43. When a pyramid or cone is cut by a cutting plane parallel to its base, the remaining portion thus obtained after removing the top portion is called :..

a. a prism .

b. frustum

c. a cylinder

d. a cone

44. In case of aligned method of dimensioning, they can be readable form

a. bottoms and right side of drawing sheet

b. bottoms and left side of drawing sheet

c. bottoms and top side of drawing sheet

d. all of the above

45. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

46. Pick up the correct statement from the following:

a. The capital required to get a project started is the first cost.

b. The first cost is a single cash flow or a series of cash flows that are made in the beginning of the activity's life span

c. The first cost of purchasing a car is the sum of the down payment, taxes and dealers charges.

d. All of these

47. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

48. Ethics as a normative science needs code of conduct and guidelines to maintaining high level of standard of good behavior in the society. This is applicable to :

a. Lawers

b. doctors

c. Engineers

d. all of the above

49. What is EPC contract ?

a. Engineering, Planning and costing

b. Evaluation, Planning and consulting

c. Engineering, procurement and Construction

d. none of above

50. The bid security shall be forfeited if

a. Bidder withdraws its bid during the period of bid validity specified by the bidder on the Bid form

b. in case of successful bidder, if the bidder fails within the specified time limit either sign the contract or furnish performance security

c. both (a) and (b) of above

d. none of the above