***Engineer***

1. Pick up the correct statement from the following:

a. In a loaded beam, the moment at which the entire section of the beam becomes fully plastic, is called plastic moment

b. In a fully plastic stage of the beam, the neutral axis divides the section in two sections of equal area

c. The ratio of plastic moment to the yield moment, is called shape factor

d. All the above.

2. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

3. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

4. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

5. Pick up the correct statement from the following :

a. A ductile material has large plastic zone

b. A brittle material has no plastic zone

c. A rigid material has no plastic zone

d. All the above.

6. A back sight

a. is always taken on a point of known elevation or can be computed

b. is added to the known level to obtain the instrument height

c. taken on an inverted staff is treated as negative

d. all the above.

7. Determining the difference in elevation between two points on the surface of the earth, is known as

a. levelling

b. simple levelling

c. differential levelling

d. longitudinal levelling.

8. The best method of interpolation of contours, is by

a. estimation

b. graphical means

c. computation

d. all of these.

9. In a perfect prismatic compass

a. ends of the needle and pivot are in same vertical and horizontal planes

b. pivot is vertically over the centre of the graduated circle

c. needle is always kept sensitive

d. all the above.

10. The co-ordinate of a point measured perpendicular to the parallel, is called

a. total latitude

b. meridian distance

c. total departure

d. consecutive co-ordinate.

11. Prolongation of chain line across an obstruction in chain surveying, is done by

a. making angular measurements

b. drawing perpendiculars with a chain

c. solution of triangles

d. all the above.

12. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

13. Pick up the correct characteristic of Pyroxene from the following:

a. It converts to chlorine by hydration

b. Its density is 2.3 to 3.6 g/cm2

c. Its hardness is 5 to 6

d. All the above.

14. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

15. Cement is said to be of good quality if

a. one feels cool by thrusting one's hand in the cement bag

b. it is not smooth when rubbed in between fingers

c. a handful of cement thrown into a bucket of water does not float

d. none of these.

16. Dry rot

a. reduces the strength of timber

b. reduces the timber to powder

c. shrinks the timber

d. spoils the appearance of timber.

17. Pick up the correct statement from the following:

a. In stone arches, the stones are placed with their natural beds radial

b. In cornices, the stones are placed with their natural beds as vertical

c. In stone walls, the stones are placed with their natural beds as horizontal

d. All the above.

18. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

19. Permissible compressive strength of M 200 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

20. For given water content, workability decreases if the concrete aggregates contain an excess of

a. flat particles

b. elongated particles

c. flaky paticles

d. all the above.

21. Specified compressive strengh of concrete is obtained from cube tests at the end of

a. 7 days

b. 14 days

c. 21 days

d. 28 days.

22. You are asked to construct a massive dam, the type of cement you will use, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. white cement.

23. To ensure constant moisture content in aggregates

a. height of each aggregate pile should not exceed 1.50 m

b. aggregate pile should be left for 24 hours before aggregates are used

c. conical heaps of aggregates should be avoided to prevent moisture variation

d. all the above.

24. The maximum pressure which a soil can carry without shear failure, is called

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

25. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

26. Coulomb's wedge theory assumes that

a. slip surface is the plane which passes through the heel of the wall

b. sliding wedge itself acts as a rigid body and the value of earth pressure is obtained by considering the limiting equilibrium of the wedge

c. position and direction of the resultant earth pressure, are known

d. all the above.

27. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

28. The angle between the directions of the failure and the major principal plane, is equal to

a. 90° + effective angle of shearing resistance

b. 90° + half of the angle of shearing resistance

c. 45° - half of the angle of shearing resistance

d. 45° + half of the angle of shearing resistance.

29. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

30. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

31. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

32. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

33. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

34. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

35. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

36. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

37. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

38. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

39. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

40. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

41. The letter 'H' lebelled in a Drawing Pencil indicates -

a. relative hardness

b. thickness of line

c. both of above

d. none of the above

42. Axonometric view/drawing is what type of view?

a. one plane

b. two plane

c. three plane

d. six plane

43. The purpose of making the drawing board with seasoned soft wood

a. easy to carry

b. for fixing of drawing paper with the help of drawing pin

c. to match with T-square

d. all of the above

44. Find the RF for scale 1cm= 25 km

a. 1/250

b. 1/2500

c. 1/25000

d. 1/2500000

45. Pick up the correct statement from the following:

a. The ratios which show profitability in relation to sales and those which show profitability in relation to investment, are called profitability ratios.

b. The ratio of gross profit and net sales, is called profitability in relation to sales ratio.

c. The ratio of net profit after taxes to total assests is known as profitability in relation to investment ratio

d. All of these

46. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

47. The alternatives which are standalone solutions for given situations in engineering involve :

a. the anticipated life of the assest

b. the yearly costs of maintaining the assest (annual maintenance and operating cost)

c. the anticipated resaleable value (salvage value) and the interest return (rate of return)

d. All of these

48. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr

49. The employer's have right to

a. accept the bid

b. reject the bid

c. both (a) and (b) of above

d. none of the above

50. Which is the professional body of architectural and engineering firms is Nepal?

a. Society of Nepalese Architects (SONA)

b. Society of Public Health Engineers (SOPHEN)

c. Sociality of Consulting Engineering and Architectural firms

d. None of above