***Engineer***

1. The strain energy stored in a spring when subjected to greatest load without being permanently distorted, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

2. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

3. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

4. The phenomenon of slow growth of strain under a steady tensile stress, is called

a. yielding

b. creeping

c. breaking

d. none of these.

5. The following assumption is not true in the theory of pure torsion :

a. The twist along the shaft is uniform

b. The shaft is of uniform circular section throughout

c. Cross-section of the shaft, which is plane before twist remains plane after twist

d. All radii get twisted due to torsion.

6.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

7. In levelling operation,

a. The first sight on any change point is a back sight

b. The second sight on any change point is a fore sight

c. The line commences with a fore sight and closes with a back sight

d. The line commences with a back sight and closes with a foresight.

8. The slope correction for a 3° slope for a length of 100 m, is

a. -0.11 m

b. -0.12 m

c. -1.87 m

d. -0.137 m

9. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

10. Hydrographic surveys deal with the mapping of

a. large water bodies

b. heavenly bodies

c. mountaineous region

d. canal system

11. The 10 mm markings on a levelling staff placed at 20 m are separated by

a.  radian

b.  radian

c.  radian

d.  radian.

12. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

13. Pick up the correct statement from the following:

a. Air bubbles in casting produce a dull sound by tapping their surfaces lightly with a hammer

b. Cupola furnace is used for the manufacture of cast iron

c. Red short iron is of no value for welding purpose

d. All the above.

14. During smelting process, the combination of fuel in the furnace

a. forms carbon dioxide

b. carbon dioxide with carbon forms carbon mono-oxide

c. carbon mono-oxide reacts with Fe2O3 to form iron and liberates CO2

d. all the above.

15. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

16. Plastic

a. consists of natural or synthetic binders

b. finished products are rigid and stable at normal temperature

c. is capable of flow when necessary heat and pressure are applied

d. All the above.

17. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

18. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

19. A concrete having a slump of 6.5 cm, is said to be

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

20. The following proportion of the ingredients of concrete mix, is not in conformation to arbitrary method of proportioning

a. 1 : 2 : 4

b. 1 : 3 : 6

c. 1 : 2 : 8

d. 1 : 4 : 10

21. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

22. An ordinary Portland cement when tested for its fineness, should not leave any residue on I.S. seive No. 9, more than

a. 10%

b. 15%

c. 20%

d. 25%

23. Pick up the correct statement from the following:

a. The concrete cured at a temperature below 23°C, gains strength up to 28 days

b. The concrete does not set at freezing point

c. The strength of concrete increases with its age

d. All the above.

24. A grillage foundation

a. is provided for heavily loaded isolated columns

b. is treated as spread foundation

c. consists of two sets of perpendicularly placed steel beams

d. all the above.

25. Factor of safety against sliding of a slope, is the ratio of

a. actual cohesion to that required to maintain stability of slope

b. shear strength to shear stress along the surface

c. neither (a) nor (b)

d. both (a) and (b).

26. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

27. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

28. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

29. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

30. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

31. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

32. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

33. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

34. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

35. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

36. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

37. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

38. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

39. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

40. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

41. The angle between two perpendicular line is

a. 0°

b. 45°

c. 90°

d. 180°

42. 9H pencil is ......than the 7H pencil

a. softer

b. harder

c. medium

d. none of the above

43. The angle between two horizontal line is

a. 0°

b. 180°

c. both (a) and (b) of above

d. none of the above

44. ....... is used for setting of short equal distances.

a. compass

b. bow divider

c. scale

d. lengthening bar

45. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

46. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

47. Pick up the correct statement from the following:

a. The ability of a company to meet obligations which are likely to mature in short term, is called liquidity.

b. The liquidity ratio may be defined as a relationship of current liabilities and current assests and advances.

c. The liquidity ratios are used to indicate the financial position of the firm.

d. All of these

48. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer

49. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

50. As per the public Procurement Regulation, 2064 of Nepal, what is the maximum age for a technical proposal for consulting services ?

a. 90%

b. 80%

c. 70%

d. 60%