***Engineer***

1. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

2. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

3. A cantilever of length 2 cm and depth 10 cm tapers in plan from a width 24 cm to zero at its free end. If the modulus of elasticity of the material is 0.2 x 106 N/mm2, the deflection of the free end, is

a. 3 mm

b. 4 mm

c. 5 mm

d. 6 mm

4. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

5. When two plates butt together and are riveted with two cover plates with two rows of rivets, the joint is known as

a. lap joint

b. butt join

c. single riveted single cover butt joint

d. double riveted double cover butt joint.

6. Surveys which are carried out to depict mountains, rivers, water bodies, wooded areas and other cultural details, are known as

a. city surveys

b. topographical surveys

c. guide map surveys

d. plane surveys.

7. A dumpy level is set up with its eye-piece vertically over a peg A. The height from the top of peg A to the centre of the eye-piece is 1.540 m and the reading on peg B is 0.705 m. The level is then setup over B. The height of the eye-piece above peg B is 1.490 m and a reading on A is 2.195 m. The difference in level between A and B is

a. 3.030 m

b. 0.770 m

c. 0.785 m

d. 1.770 m.

8. Two concave lenses of 60 cm focal length are cemented on either side of a convex lens of 15 cm focal length. The focal length of the combination is

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

9. Locating the position of a plane table station with reference to three known points, is known as

a. intersection method

b. radiation method

c. resection method

d. three point problem.

10. The staff reading at a distance of 80 m from a level with the bubble at its centre is 1.31 m. When the bubble is moved by 5 divisions out of the centre, the reading is 1.39 m. The angular value of the one division of the bubble, is

a. 41.25 secs

b. 14.52 secs

c. 25.05

d. none of these

11. The radius of curvature of the arc of the bubble tube is generally kept

a. 10 m

b. 25 m

c. 50 m

d. 100 m

12. The 'fix' of a plane table station with three known points, is bad if the plane table station lies

a. in the great triangle

b. outside the great triangle

c. on the circumference of the circumscribing circle

d. none of these.

13. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

14. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

15. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

16. The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

17. Smith's test of stones is performed to find out

a. the presence of soluble matter of stone

b. the compressive strength of the stone

c. the hardness of the stone

d. the toughness of the stone.

18. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

19. Construction joints are generally provided in concrete

a. retaining walls

b. lining of canals

c. lining of tunnels

d. all the above.

20. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

21. The size of fine aggregates does not exceed

a. 3.00 mm

b. 3.75 mm

c. 4.75 mm

d. 5.75 mm

22. Non-uniform compaction may cause the concrete

a. porous

b. non-homogeneous

c. reduced strength

d. all the above.

23. M 150 grade of concrete approximates

a. 1 : 1 :2 mix

b. 1 : 2 : 4 mix

c. 1 : 1.5 : 3 mix

d. none of these.

24. The plasticity index is the numerical difference between

a. liquid limit and plastic limit

b. plastic limit and shrinkage limit

c. liquid limit and shrinkage limit

d. none of these.

25. The equation τ = C + σ tan φ is given by

a. Rankine

b. Coulomb

c. Culaman

d. Mohr.

26. Tergazhi's theory of one dimensional consolidation assumes

a. water and soil particles are incompressible

b. deformation of the soil, is entirely due to change in volume

c. Darcey's law for the velocity of flow of water through soil, is perfectly valid

d. all the above.

27. Number of piles required to support a column, is

a. 1

b. 2

c. 3

d. 4

28. The phreatic line in an earth dam may be

a. elliptical

b. parabolic

c. a straight line

d. all the above.

29. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

30. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

31. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

32. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

33. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

34. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

35. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

36. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

37. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

38. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

39. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

40. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

41. Which of the following is not a drawing hand tool?

a. alidade

b. straight edge

c. french curve

d. set squares

42. Circle and arcs of circles are drawn by means of a .......

a. lengthening bar

b. divider

c. compass

d. T-square

43. Number of equal pieces of A4 size paper from A0 is equal to

a. 2

b. 4

c. 8

d. 16

44. The area occupied by Ao (drawing sheet) is ......... times the area occupied by A3 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

45. Current assests less inventories divided by current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Earning per share is the most important ratio for

a. share holders

b. banks

c. company's management

d. All of these

47. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

48. If the registration of an engineers is cancealed from NEC his name will be registerered or not if engineer is ready to register his name (CT) -18 NEC, regulution

a. yes

b. no

c. may be

d. all of the above

49. Integrity is

a. state or quality of being complete, undivided or unbroken moral soundness, honesty & uprightness

b. the wholeness of deriving quality

c. both a) and b) of above

d. none of the above

50. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid