***Engineer***

1. Stress may be defined as

a. force per unit length

b. force per unit volume

c. force per unit area

d. none of these.

2.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

3. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

4. If the stress produced by a prismatic bar is equal to the working stress, the area of the cross-section of the prismatic bar, becomes

a. zero

b. infinite

c. maximum

d. minimum.

5. The principal stresses at a point are 100, 100 and-200 kgf/cm2, the octo hedral shear stress at the point is :

a. 100 2 kg/cm2

b. 200 2 kg/cm2

c. 300 2 kg/cm2

d. 400 2 kg/cm2

6. If vertical angles of inclined sights do not exceed 10° and non-verticality of the staff remains within 1°, stadia system of tacheometric observations are made on

a. staff normal

b. staff vertical

c. staff normal as well as vertical

d. none of these.

7. Geodetic surveying is undertaken

a. for developing the science of geodesy

b. making use of most accurate instruments and methods of observation

c. for determination of accurate positions on the earth's surface of system of control points

d. all the above.

8. If Δ is the angle of deflection of a simple curve of radius R, the length of its long chord, is

a. R cos

b. 2R cos

c. R sin

d. 2R sin .

9. The true meridian of a place is the line in which earth's surface is intersected by a plane through

a. east and west points

b. zenith and nadir points

c. north and south geographical poles

d. north and south magnetic poles.

10. For true difference in elevations between two points A and B, the level must be set up

a. at any point between A and B

b. at the exact mid point of A and B

c. near the point A

d. near the point B.

11. In case of reduction of levels by the height of instrument method,

a. ∑ B.S. - ∑ F.S. = difference in R.L.S of the first station and last station

b. ∑ (R.L. + I + F.S.) - first R.L = ∑ (H.I. + No. of R.L.s.)

c. both (a) and (b) above

d. neither (a) nor (b).

12. The bubble tube is nearly filled with

a. alcohol or chloroform

b. a liquid which is very mobile

c. a liquid having low freezing point

d. all the above.

13. Stones used for rubble masonry must be

a. soft

b. hard

c. light

d. heavy.

14. Fibre glass

a. has a higher strength to weight ratio

b. is shock proof and fire retardent

c. does not decay

d. all the above.

15. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

16. The commonly used drying oil for oil paints, is

a. olive oil

b. linseed oil

c. kerosine oil

d. accetate of lead.

17. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

18. A 1st class brick immersed in water for 24 hours, should not absorb water (by weight) more than

a. 15%

b. 20%

c. 25%

d. 5%.

19. For batching 1:3:6 concrete mix by volume, the ingredients required per bag of 50 kg cement, are:

a. 70 kg of sand and 140 litres of aggregates

b. 105 litres of sand and 140 litres of aggregates

c. 105 litres of sand and 210 litres of aggregates

d. none of these.

20. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

21. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

22. Pick up the correct statement from the following:

a. Concrete using sand stones, cracks due to excessive shrinkage

b. Very hard and close grained crystallined lime stones are suitable aggregates but provide low strength

c. Broken bricks produce a concrete having good fire resisting qualities

d. All the above.

23. The maximum percentage of chemical ingredient of cement is that of

a. iron oxide

b. aluminium

c. lime

d. silica.

24. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

25. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

26. A critical hydraulic gradient may occur when

a. flow is in upward direction

b. seepage pressure is in upward direction

c. effective pressure is zero

d. all the above.

27. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

28. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

29. The specific gravity of sands, is approximately

a. 2.0

b. 2.2

c. 2.4

d. 2.6

30. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

31. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

32. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

33. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

34. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

35. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

36. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

37. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

38. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

39. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

40. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

41. Parallel rullig straight edge machine is used to draw

a. inclined lines

b. curved lines

c. irregular lines

d. horizontal lines

42. The purpose of making the drawing board with seasoned soft wood

a. easy to carry

b. for fixing of drawing paper with the help of drawing pin

c. to match with T-square

d. all of the above

43. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

44. ”Title Block” is placed at bottom right hand corner of the drawing which gives information about

a. title of drawing

b. scale of the drawing

c. serial no of drawing

d. all of the above

45. Pick up the correct statement from the following:

a. The manifestation of the time value of money is termed as interest.

b. Interest on borrowing = present amount owed - original loan

c. The original investment (or loan) is referred to as principal.

d. All of these.

46. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

47. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

48. Which of the following category of Engineer is not mentioned in Engineering Council rule

a. Charted Engineer

b. General Engineer

c. Professional Engineer

d. Non-Nepali Regd. Engineer

49. The document announcing to prospective bidder that the owner is ready to receive bids is called

a. notice to bidders

b. instruction to bidders

c. bill of quantities

d. intension to acceptance

50. Any Non-Nepali Engineer desiring to work in Nepal may apply for registration under:

a. Category A

b. Category B

c. Category C

d. Category D