***Engineer***

1. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

2. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4.

3. If ΣH and ΣV are the algebraic sums of the forces resolved horizontally and vertically respectively, and ΣM is the algebraic sum of the moments of forces about any point, for the equilibrium of the body acted upon

a. ΣH = 0

b. ΣV = 0

c. ΣM = 0

d. all the above.

4. The property of a material by which it can be drawn to a smaller section, due to tension, is called

a. plasticity

b. ductility

c. elasticity

d. malleability.

5. Strain energy of any member may be defined as work done on it

a. to deform it

b. to resist elongation

c. to resist shortening

d. all the above.

6. If the length of a chain line along a slope of θ° is l, the required slope correction is

a. 2l cos2 θ/2

b. 2l sin2 θ/2

c. l tan2 θ/2

d. l cos2 θ/2.

7. If the plane table is not horizontal in a direction at right angles to the alidade, the line of sight is parallel to the fiducial edge only for

a. horizontal sights

b. inclined sights upward

c. inclined sight downward

d. none of these.

8. The instrument which is used in plane tabling for obtaining horizontal and vertical distances directly without resorting to chaining, is known as

a. Plane alidade

b. telescopic alidade

c. clinometer

d. tacheometer.

9. The operation of revolving a plane table about its vertical axis so that all lines on the sheet become parallel to corresponding lines on the ground, is known

a. levelling

b. centering

c. orientation

d. setting.

10. The line normal to the plumb line is known as

a. horizontal line

b. level line

c. datum line

d. vertical line.

11. Offsets are measured with an accuracy of 1 in 40. If the point on the paper from both sources of error (due to angular and measurement errors) is not to exceed 0.05 cm on a scale of 1 cm = 20 m, the maximum length of offset should be limited to

a. 14.14

b. 28.28 m

c. 200 m

d. none of these.

12. True meridians are generally preferred to magnetic meridians because

a. these converge to a point

b. these change due to change in time

c. these remain constant.

d. None of these.

13. Dolomite is a lime stone which contains carbonate of magnesia upto

a. 20%

b. 25%

c. 35%

d. 45%.

14. In stone masonry, if stones are so placed that their layers are parallel to the direction of load, they

a. split easily

b. are affected by moisture

c. both (a) and (b)

d. none of these.

15. Pick up the correct statement from the following:

a. The silica in excess makes the cement stronger but its setting time also increases

b. The excess amount of alumina weakens the cement

c. The addition of gypsum increases the initial setting time of cement

d. All the above.

16. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

17. Minimum required water cement ratio for a workable concrete, is

a. 0.40

b. 0.50

c. 0.60

d. 1.0.

18. Mastic asphalt is

a. acid resisting material

b. non-corrosive material

c. corrosive material

d. heating-resisting material.

19. Sand generally contains salt if it is obtained from:

a. river beds

b. sea beds

c. all the above

d. none of these.

20. To obtain a very high strength concrete, use very fine grained

a. Granite

b. Magnetite

c. Barite

d. Volcanic scoria.

21. The entrained air in concrete

a. increases workability

b. decreases workability

c. decreases resistance to weathering

d. increases strength

22. Efflorescence in cement is caused due to an excess of

a. iron oxide

b. silica

c. alkalis

d. magnesium oxide.

23. The impurity of mixing water which affects the setting time and strength of concrete, is

a. sodium chlorides

b. sodium carbonates and bicarbonates

c. calcium chorides

d. calcium bicarbonates

24. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

25. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

26. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

27. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

28. The seepage exit gradient in a soil is the ratio of

a. flow line to slope

b. head upstream to that at downstream

c. head loss to the length of the seepage

d. none of these.

29. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

30. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

31. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

32. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above

33. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

34. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

35. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

36. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

37. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

38. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

39. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

40. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

41. In third angle projection top view is put

a. above front view

b. below font view

c. side view

d. none of the above

42. In case of third angle projection, plan is kept at

a. top

b. below

c. side

d. none

43. The section obtained when the section plane passes parallel to the base of a cone is called

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

44. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

45. The ratio obtained by dividing 'quick assests' by current liabilities is called

a. Turnover ratio

b. Acid test ratio

c. Solvency ratio

d. None of these

46. Which one of the following questions is relevant to the construction estimates :

a. Did the estimators precisely evaluate site conditions ?

b. Did the estimators use short cut methods which may be un realistic in their situation ?

c. How much money will the contractor's risk, loosing if he Were to submit bid on the raw estimate of cost.

d. All of these

47. Pick up the correct statement from the following:

a. A NPV profile graph shows the curvilinear relationship between the net present value of the project and discount rate employed.

b. In a NPV profile, if discount rate is zero, then net present value is simply total cash inflows less the total cash outflows of the project.

c. As the discount rate increases, the net present value profile slopes downward to the right.

d. All of these

48. Prequalification is done

a. after the invitation of bids

b. before the invitation of bids

c. during the invitation of bids

d. none of the above

49. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer

50. The number of members in dispute resolution committee are

a. 2

b. 3

c. 4

d. 5