***Engineer***

1. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

2. Stress may be defined as

a. force per unit length

b. force per unit volume

c. force per unit area

d. none of these

3. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

4. To ascertain the maximum permissible eccentricity of loads on circular columns, the rule generally followed, is

a. middle half rule of columns

b. middle third rule of columns

c. middle fourth rule of columns

d. none of these.

5. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

6. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

7. Greater accuracy in linear measurements, is obtained by

a. tacheometry

b. direct chaining

c. direct taping

d. all the above.

8. If the whole circle bearing of a line is 270°, its reduced bearing is

a. N 90° W

b. S 90° W

c. W 90°

d. 90° W.

9. The slope correction for a 3° slope for a length of 100 m, is

a. -0.11 m

b. -0.12 m

c. -1.87 m

d. -0.137 m

10. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

11. To avoid large centering error with very short legs, observations are generally made

a. to chain pins

b. by using optical system for centering the theodolite

c. to a target fixed on theodolite tripod on which theodolite may be fitted easily

d. all the above.

12. If the rate of gain of radial acceleration is 0.3 m per sec3 and full centrifugal ratio is developed. On the curve the ratio of the length of the transition curve of same radius on road and railway, is

a. 2.828

b. 3.828

c. 1.828

d. 0.828.

13. Generally wooden moulds are made from

a. shisham wood

b. deodar wood

c. teak wood

d. hard wood.

14. A piece of timber whose thickness and width are respectively 5 cm and 10 cm is called

a. slate

b. plank

c. board

d. strip.

15. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

16. Pick up the correct statement from the following:

a. Quick lime contains calcium oxide

b. Slaked lime may be obtained from quick lime

c. Slaked lime is obtained by adding water to quick lime

d. All the above.

17. Good quality cement contains higher percentage of

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite

18. Stainless steel resists corrosion due to

a. sulphur

b. vanadium

c. chromium

d. maganese.

19. Concrete gains strength due to

a. chemical reaction of cement with sand and coarse aggregates

b. evaporation of water from concrete

c. hydration of cement

d. All the above.

20. Vicat apparatus is used for

a. consistency test

b. test for setting time

c. test for tensile strength

d. none of these.

21. The top diameter, bottom diameter and the height of a slump mould are :

a. 10 cm, 20 cm, 30 cm

b. 10 cm, 30 cm, 20 cm

c. 20 cm, 10 cm, 30 cm

d. 20 cm, 30 cm, 10 cm

22. Pick up the correct statement from the following:

a. According to the petrological characteristics, concrete aggregates are classified as heavy weight, normal weight and light weight

b. According to the shape of the particles, concrete aggregates are classified as rounded irregular, angular and flaky

c. According to the surface texture of the particles, the concrete aggregates are classified as glassy, smooth, granular, rough, crystalline, honey combed and porous

d. All the above.

23. Pick up the correct statement from the following:

a. Silica in excess, causes the cement to set slowly

b. Alumina in excess, reduces the strength of the cement

c. Magnesium oxide in excess, remains in free state and makes the cement unsound

d. All the above.

24. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

25. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

26. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

27. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

28. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

29. Degree of saturation of a natural soil deposit having water content 15%, specific gravity 2.50 and void ratio 0.5, is

a. 50%

b. 60%

c. 75%

d. 80%

30. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

31. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

32. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

33. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

34. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same

35. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

36. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

37. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

38. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

39. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

40. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

41. Set squares are used for drawing ........ ....... .. and ........... lines.

a. vertical

b. inclined

c. parallel

d. all of the above

42. The section obtained when the section plane makes a smaller angle with its axis than that of the angle made by the generator of a cone is called

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

43. The polyhedra  having two equal and similar end bases, parallel to each other and are other faces which may be rectangle, or parallelograms is called ......

a. a prism

b. a pyramid

c. a cylinder

d. a cone

44. The most important material for drawing is

a. pencil and eraser

b. eraser & paper

c. paper & pencil

d. none of the above

45. Probabilistic estimating of a construction project includes:

a. Labour

b. Productivity

c. wage scale

d. All of these

46. If a is the base amount expenditure, b is the increase in the operation cost each year over a period of n years, the total cost of maintenance is :

a. a + (n + 1) b

b. a + (n - 1) b

c. a x (n - 1) b

d. a - (n - 1) b

47. Which one of the following statements is correct?

a. The number of years required to recover the initial cash investment in a project, is called Pay Back period (PBP).

b. The discount rate that equates the present value of the expected Net Cash Flows (CFs) with the Initial Cash Outflow (ICO) is known as internal rate of return.

c. The present value of the proposal's net cash flows, less the proposal's initial cash outflow is known as the Net Present Value (NPV)

d. All of these

48. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation

49. Who is the Chairperson of House Rent Fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

50. In general, Nepal Engineering Council mecting per year should be held:

a. 2times

b. 3times

c. 4times

d. 5times