***Engineer***

1. By applying the static equations i.e. ΣH = 0, ΣV = 0 and ΣM = 0, to a determinate structure, we may determine

a. shear forces only

b. bending moments only

c. internal forces only

d. all the above.

2.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

3. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

4. The stress at which extension of a material takes place more quickly as compared to the increase in load, is called

a. elastic point

b. plastic point

c. breaking point

d. yielding point.

5. In a solid arch, shear force acts

a. along the axis of the arch

b. perpendicular to the axis of arch

c. tangentially to the arch

d. none of these.

6. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

7. The direction of steepest slope on a contour, is

a. along the contour

b. at an angle of 45° to the contour

c. at right angles to the contour

d. none of these.

8. Pick up the correct statement from the following :

a. The horizontal angle between magnetic meridian and true meridian at a place is called magnetic declination or variance of the compass

b. the imaginary lines which pass through points at which the magnetic declinations are equal at a given time are called isogonic lines

c. the isogonic lines through places at which the declination is zero are termed agonic lines

d. all the above.

9. The radius of curvature of the arc of the bubble tube is generally kept

a. 10 m

b. 25 m

c. 50 m

d. 100 m

10. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

11. In a perfect prismatic compass

a. ends of the needle and pivot are in same vertical and horizontal planes

b. pivot is vertically over the centre of the graduated circle

c. needle is always kept sensitive

d. all the above.

12. The surface of zero elevation around the earth, which is slightly irregular and curved, is known as

a. mean sea level

b. geoid surface

c. level surface

d. horizontal surface.

13. The most important constituent of an oil paint, is

a. vehicle

b. pigment

c. base

d. all the above.

14. Galvanising means covering iron with a thin coat of

a. tin

b. zinc

c. glaze

d. coal tar.

15. Basalt is

a. sedimentary rock

b. metamorphic rock

c. extrusive igneous rock

d. intrusive igneous rock.

16. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

17. The low voltage porcelain is mainly used for

a. switch block

b. insulating tubes

c. lamp sockets

d. All of these.

18. The specific gravity of marble, is

a. 2.60

b. 2.66

c. 2.72

d. 3.00.

19. Pick up the incorrect statement from the following:

a. The process of changing cement paste into hard mass, is known as setting of cement

b. The phenomenon by virtue of which cement does not allow transmission of sound, is known as soundness of cement

c. The heat generated during chemical reaction of cement with water, is known as heat of hydration

d. None of these.

20. For a good concrete

a. cement should be sufficient to produce the required strength

b. water should be free from organic materials

c. mixing of ingredients should be done thoroughly so as to produce homogeneity

d. All the above.

21. The commercial name of white and coloured cement in India, is

a. rainbow cement

b. silvicrete

c. snowcem

d. all the above.

22. Pick up the incorrect statement from the following:

a. Concrete for which preliminary tests are conducted, is called controlled concrete

b. Bulking of sand depends upon the fineness of grains

c. Concrete mix 1 : 6 : 12, is used for mass concrete in piers.

d. All the above.

23. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

24. The water content of soil is defined as the ratio of

a. volume of water to volume of given soil

b. volume of water to volume of voids in soil

c. weight of water to weight of air in voids

d. weight of water to weight of solids of given mass of soil.

25. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

26. For determing the specific gravity of soil solids, using a pycnometer of 500 cc., the following data is available : Weight of dry empty pycnometer = 125 g Weight of dried soil and pycnometer = 500 g Weight of dried soil and distilled = 850 g water filled in pycnometer up to top The specific gravity of soil soilds, is

a. 2.0

b. 2.25

c. 2.50

d. 2.75

27. Pick up the cohesive soil from the following:

a. Red earth

b. Clay

c. Black cotton soil

d. Compacted ground.

28. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

29. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

30. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

31. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

32. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

33. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

34. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

35. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

36. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

37. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

38. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

39. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

40. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

41. A 100 m long line making 30° with the vertical, show its projection on horizontal plane

a. 100 m

b. 50 m

c. 86.6 m

d. 70.71 m

42. Topographic map is used to represent the

a. artificial detail

b. natural detail

c. construction detail

d. all of the above

43. The paper size of A, is

a. 297 x 420

b. 210 x 297

c. 420x 850

d. none of the above

44. The free hand drawing of a ordinary room is generally:

a. one point perspective

b. two pint perspective

c. three point perspective

d. all of the above

45. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

46. The estimate based on a detailed quantity survey and furnishes the most accurate and reliable estimate possible is known as

a. Conceptual estimate

b. Definitive estimate

c. Probabilistic estimate

d. None of these

47. The person desires to pay off the amount in 10 equal annual instalments. The amount of each instalment is :

a. Rs 5638

b. Rs 6638

c. Rs 7738

d. None of these

48. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer

49. Code of conduct defines

a. what should be

b. what should not be

c. what had been

d. what will be

50. NEC publishes the name of registered engineers each year as well as every forth year. The name of foreigner engineers publish

a. each year

b. every forth year

c. both (a) and (b)

d. none of the above