***Engineer***

1. The assumption in the theory of bending of beams, is:

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

2. For calculating the permissible stress is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

3. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

4. A three hinged arch is loaded with an isolated load 1000 kg at a horizontal distance of 2.5 m from the crown, 1 m above the level of hinges at the supports 10 metres apart. The horizontal thrust is

a. 1250 kg

b. 125 kg

c. 750 kg

d. 2500 kg

5. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

6. Pick up the correct statement from the following :

a. The directions of plumb lines suspended at different points in a survey are not strictly parallel

b. In surveys of small extent, the effect of curvature may be ignored and the level surface of the earth is assumed as horizontal

c. In surveys of large extent, the effect of curvature of the earth must be considered

d. All the above.

7. If the smallest division of a vernier is longer than the smallest division of its primary scale, the vernier is known as

a. direct vernier

b. double vernier

c. retrograde vernier

d. simple vernier.

8. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

9. An internal focussing type surveying telescope, may be focussed by the movement of

a. objective glass of the telescope

b. convex-lens in the telescope

c. concave lens in the telescope

d. plano-convex lens in the telescope.

10. The area of any irregular figure of the plotted map is measured with

a. sextant

b. clinometer

c. planimeter

d. optical square

11. If the chain line which runs along N-S direction is horizontal and the ground in E-W direction is sloping

a. it is possible to set offsets correctly on east side

b. it is possible to set offsets correctly on east side

c. it is not possible to set offsets correctly on west side

d. it is possible to set offsets correctly on both sides.

12. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

13. Pick up the correct statement from the following:

a. Impurities float on the molten iron as slag

b. The slag contains lime about 45%

c. The molten iron is made to run, in a long channel formed in sand called 'sow'

d. All the above.

14. Wrought iron contains carbon about

a. 1.5% to 5.5%

b. 0.5% to 1.75%

c. 0.1% to 0.25%

d. none to these.

15. The process of manufacturing steel by heating short lengths of wrought iron bars mixed with charcoal in fire clay crucibles and collecting the molten iron into moulds, is known as

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

16. The melting point of silica is :

a. 1570°C

b. 1630°C

c. 1730°C

d. 1850°C.

17. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

18. The main constituent of fly-ash, is

a. aluminium oxide

b. silica

c. ferrous oxide

d. All of these.

19. Bulking of sand is

a. mixing of different sizes of sand particles

b. mixing of lime with sand

c. maximum water with sand

d. swelling of sand when wetted.

20. Slump test is done for

a. clay

b. sand

c. lime

d. concrete.

21. Curing

a. reduces the shrinkage of concrete

b. preserves the properties of concrete

c. prevents the loss of water by evaporation

d. all of the above.

22. Pick up the incorrect statement from the following:

a. Tricalcium silicate (C3S) generates more heat of hydration

b. Tricalcium silicate (C3S) develops early strength

c. Tricalcium silicate (C3S) has more resistance to sulphate attack

d. None of these.

23. The bulk density of aggregates, depends upon

a. shape

b. grading

c. compaction

d. all the above.

24. A pycnometer is used to determine

a. voids ratio

b. dry density

c. water content

d. density index.

25. The angle between the directions of the failure and the major principal plane, is equal to

a. 90° + effective angle of shearing resistance

b. 90° + half of the angle of shearing resistance

c. 45° - half of the angle of shearing resistance

d. 45° + half of the angle of shearing resistance.

26. For a clay slope of height of 10 m, the stability number is 0.05, γ = 2.0 t/m3, C = 2.5 t/m2, the critical height of the slope of the soil, is

a. 4.0 m

b. 12.5 m

c. 25.0 m

d. 15.0 m

27. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

28. The quantity of seepage of water through soils is proportional to

a. coefficient of permeability of soil

b. total head loss through the soil

c. neither (a) nor (b)

d. both (a) and (b).

29. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

30. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

31. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

32. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

33. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

34. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

35. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

36. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

37. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

38. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

39. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

40. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

41. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

42. The purpose of sectional view is to show the

a. surface of the object

b. internal of the object

c. shape of the object

d. none of the above

43. The drawing for municipality is done at scale

a. 1" = 8"

b. 1:50

c. 1:100

d. both (a) and (c)

44. Which of the following regarding scales in drawing is NOT true?

a. the name of the scale (e.g. scale 1:10) should be mentioned below the scale

b. a diagonal scale consists of a primary scale and a venirer

c. comparative scales may be plain or diagonal

d. when an unusual scale is used it is constructed on the drawing sheet

45. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

46. The construction manager uses the estimate of the project

a. to advise the architect/engineer regarding design cost parameter especially in value engineering analysis.

b. to develop bids on the project.

c. to control the project during its construction.

d. All of these

47. A project construction cost estimate includes:

a. the labour and material cost

b. the equipment and over head cost

c. the profit of the contractor

d. All of these

48. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-

49. Which one is not concerned with NEC

a. acredition of academic certificate

b. authority of opening engineering college.

c. appointment of teacher

d. all of the above

50. As per the public Procurement Regulation, 2064 of Nepal, what is the maximum age for a technical proposal for consulting services ?

a. 90%

b. 80%

c. 70%

d. 60%