***Engineer***

1.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

2. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

3. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

4. The shear stress at any section of a shaft is maximum

a. at a distance r/2 from the centre

b. at the top of the surface

c. at a distance 3/4 r from the centre

d. none of these.

5. If the depth of a simply supported beam carrying an isolated load at its centre, is doubled, the deflection of the beam at the centre will be changed by a factor of

a. 1/2

b. 8

c. 1/8

d. 4

6. The constant vertical distance between two adjacent contours, is called

a. horizontal equivalent

b. vertical equivalent

c. contour interval

d. contour gradient.

7. The real image of an object formed by the objective, must lie

a. in the plane of cross hairs

b. at the centre of the telescope

c. at the optical centre of the eye-piece

d. anywhere inside the telescope.

8. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

9. The imaginary line passing through the intersection of cross hairs and the optical centre of the objective, is known as

a. line of sight

b. line of collimation

c. axis of the telescope

d. none of these.

10. Surveys which are carried out to provide a national grid of control for preparation of accurate maps of large areas, are known

a. plane surveys

b. geodetic surveys

c. geographical surveys

d. topographical surveys.

11. Pick up the correct statement from the following :

a. the system of a series of lines which forms a circuit which ends at the starting point, is called a closed traverse

b. the traverse that starts from a point already fixed in some survey system and ends on another such point, is called a controlled traverse

c. the traverse that is not controlled is called a fly traverse

d. all the above.

12. Offsets are measured with an accuracy of 1 in 40. If the point on the paper from both sources of error (due to angular and measurement errors) is not to exceed 0.05 cm on a scale of 1 cm = 20 m, the maximum length of offset should be limited to

a. 14.14

b. 28.28 m

c. 200 m

d. none of these.

13. Pick up the correct statement from the following:

a. The bulking of fine sand is more than that of coarse sand

b. If the percentage content of moisture exceeds 10%, increase in bulk of sand starts increasing

c. The volume of fully saturated sand equals that of dry sand

d. All the above.

14. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

15. According to IS 399-1963, the weight of the timber is specified at

a. 8% moisture content

b. 10% moisture content

c. 12% moisture content

d. 14% moisture content.

16. The rocks having alumina or clay as their major constituents, are known as

a. argillaceous rocks

b. calcareous rocks

c. sedimentary rocks

d. igneous rocks.

17. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

18. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

19. Bulking of sand is

a. mixing of different sizes of sand particles

b. mixing of lime with sand

c. maximum water with sand

d. swelling of sand when wetted.

20. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

21. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

22. The maximum amount of dust which may be permitted in aggregates is

a. 5% of the total aggregates for low workability with a coarse grading

b. 10% of the total aggregates for low workability with a fine grading

c. 20% of the total aggregates for a mix having high workability with fine grading

d. all the above.

23. For given workability the grading requiring the least amount of water is one that gives

a. greatest surface area for the given cement and aggregates

b. least surface area for the given cement and aggregates

c. least weight for the given cement and aggregates

d. greatest weight for the given cement and aggregates

24. A coarse-grained soil has a voids ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

25. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

26. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

27. Pick up the incorrect statement from the following:

a. Compaction has no effect on the structure of a soil

b. Permeability decreases with increase in the dry density of a compacted soil

c. A wet side compacted soil is more compressible than a dry side compacted soil

d. Dry side compaction soils swell more when given access to moisture

28. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

29. The ratio of the volume of voids to the volume of soil solids in a given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

30. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

31. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

32. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

33. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

34. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

35. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

36. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

37. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

38. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

39. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

40. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

41. In one plane drawing, which one does not falls under

a. orthographic

b. axonometric

c. oblique

d. dimetric

42. Set squares are not used to draw

a. straight line

b. vertical line with T-square

c. horizontal line

d. all of the above

43. The shades and shadows are :

a. the same

b. the different

c. the result of object on the ground

d. the result of light on the ground

44. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

45. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

46. Pick up the correct statement regarding financial statement analysis from the following.

a. Final analysis always involves the use of various financial statements i.e., balance sheet and income statement.

b. The balance sheet is the summary of assests, liabilities and owner's equity of business at a point in time.

c. The income statement is the summary of revenues and expenses of a firm over a particular period of time.

d. All the above

47. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

48. For the registration of general engineer tha amount of fee required is:

a. NRS 2000/-

b. NRS 3000/-

c. NRS 3500/-

d. NRS 4000/-

49. What percent of interest is taken for the advance amount paid to the contractor by employer?

a. 1% of advance amount

b. 2% of advance amount

c. 3% of advance amount

d. none of the above

50. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid