***Engineer***

1. In plastic analysis, the shape factor for a triangular section, is

a. 1.5

b. 1.34

c. 2.34

d. 2.5

2. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50.

3. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

4. To ascertain the maximum permissible eccentricity of loads on circular columns, the rule generally followed, is

a. middle half rule of columns

b. middle third rule of columns

c. middle fourth rule of columns

d. none of these.

5. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

6. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

7. The  lens  equation   is applicable

a. when the thickness of the lens is small

b. only to conjugate distances along the principal axis

c. the computed quantities are accurate enough for geometrical optics of simple distances

d. all the above.

8. If the chain line which runs along N-S direction is horizontal and the ground in E-W direction is sloping

a. it is possible to set offsets correctly on east side

b. it is possible to set offsets correctly on east side

c. it is not possible to set offsets correctly on west side

d. it is possible to set offsets correctly on both sides.

9. Location of contour gradient for a high way is best set out from

a. ridge down the hill

b. saddle down the hill

c. bottom to the ridge

d. bottom to the saddle.

10. The distance between steps for measuring down hill to obtain better accuracy

a. decreases with decrease of slope

b. increases with increase of slope

c. decreases with increase of slope

d. decreases with decrease of weight of the chain.

11. Accuracy of 'fix' by two point problem, is

a. bad

b. good

c. not reliable

d. unique.

12. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

13. The base material for distemper, is

a. chalk

b. lime

c. lime putty

d. cement wash.

14. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

15. Plastics are compounds of carbon with element

a. hydrogen

b. nitrogen

c. oxygen

d. All of these.

16. Good quality cement contains higher percentage of

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite

17. The variety of pig iron used for the manufacture of steel by Bessemer process, is

a. Bessemer pig

b. Grey pig

c. White forge pig

d. Mottled pig.

18. Pick up the correct statement from the following:

a. soft stones are required for carving

b. light stones are required for arches

c. hard stones are required to stand high pressure

d. All the above.

19. For compacting plain concrete road surface of thickness less than 20 cm, we use

a. internal vibrator

b. screed vibrator

c. form vibrator

d. none of these.

20. Pick up the correct statement from the following:

a. Consistency does not affect the workability of concrete

b. If the slump increases, workability decreases

c. If the concrete mix is dry, the slump is maximum

d. None of these.

21. For given workability the grading requiring the least amount of water is one that gives

a. greatest surface area for the given cement and aggregates

b. least surface area for the given cement and aggregates

c. least weight for the given cement and aggregates

d. greatest weight for the given cement and aggregates

22. Pick up the incorrect statement from the following:

a. The process of changing cement paste into hard mass, is known as setting of cement

b. The phenomenon by virtue of which cement does not allow transmission of sound, is known as soundness of cement

c. The heat generated during chemical reaction of cement with water, is known as heat of hydration

d. None of these.

23. The main object of compaction of concrete, is:

a. to eliminate air holes

b. to achieve maximum density

c. to provide initimate contact between the concrete and embedded materials

d. all the above.

24. The shearing strength of a cohesion-less soil depends upon

a. dry density

b. rate of loading

c. confining pressure

d. nature of loading.

25. Pick up the correct statement from the following:

a. An unconfined compression test is a special case of triaxial compression test

b. An unconfined compression test is a special case of direct shear test

c. The confining pressure is maximum during an unconfined compression test

d. The cylindrical specimen of a soil is subjected to major principal stress till it fails due to shearing along the plane of the failure.

26. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

27. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

28. The density of soil can be increased

a. by reducing the space occupied by air

b. by elastic compression of soil grains

c. by expelling water from pores

d. All the above.

29. The ultimate bearing capacity of a soil, is

a. total load on the bearing area

b. safe load on the bearing area

c. load at which soil fails

d. load at which soil consolidates.

30. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

31. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

32. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

33. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

34. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

35. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

36. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

37. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

38. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

39. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

40. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

41. Axonometric view/drawing is what type of view?

a. one plane

b. two plane

c. three plane

d. six plane

42. The area occupied by Ao (drawing sheet) is ........... times the area occupied by A1 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

43. The maps which are on sufficient large scale to enable the individual features shown on the map to be identified on the ground by their shape and position are called :

a. topographical maps

b. geographical maps

c. geological maps

d. site plan

44. In orthographic projection, projector lines are

a. vertical lines

b. inclined lines

c. horizontal lines

d. none of above

45. Probabilistic estimating of a construction project includes:

a. Labour

b. Productivity

c. wage scale

d. All of these

46. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

47. If P is principal amount, i is the rate of interest per annum and n is the number of periods in years, the compound amount factor (CAF) is :

a. (1 + i)n

b. (1 + i)(1/2n)

c. (n + i)

d. None of these

48. The registered engineer in Nepal Engineering Council should have to renew their registration in:

a. every year

b. every two years

c. every four years

d. none of the above

49. For procurement of works up to 2 million, buy which of the following method is to be considered?

a. by sealed tender

b. by direct negotiation

c. by bidding

d. by direct labour or direct negotiation

50. NEC act and its regulation........

a. 55 and 57

b. 54 and 55

c. 55 and 56

d. 57 and 58