***Engineer***

1. The normal component of a force inclined through θ° is obtained by multiplying the force by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

2. Principal planes are subjected to

a. normal stresses only

b. tangential stresses only

c. normal stresses as well as tangential stresses

d. none of these

3. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

4. In a shaft rotated by a couple, the shear force varies

a. from zero at the centre to a maximum at the circumference

b. from minimum at the centre of maximum at the circumference

c. from maximum at the centre to zero at the circumference

d. equally throughout the section.

5. Beams of uniform strength are preferred to those of uniform section because these are economical for

a. large spans

b. heavy weights

c. light weights

d. short spans.

6. You have to observe an included angle with better accuracy than what is achievable by a vernier, you will prefer the method of

a. repetition

b. reiteration

c. double observations

d. exactness.

7. True meridian of different places

a. converge from the south pole to the north pole

b. converge from the north pole to the south pole

c. converge from the equator to the poles

d. run parallel to each other.

8. If the chain line which runs along N-S direction is horizontal and the ground in E-W direction is sloping

a. it is possible to set offsets correctly on east side

b. it is possible to set offsets correctly on east side

c. it is not possible to set offsets correctly on west side

d. it is possible to set offsets correctly on both sides.

9. A lens or combination of lenses in which the following defect is completely eliminated is called aplanatic

a. spherical aberration

b. chromatic aberration

c. coma

d. astigmatism.

10. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

11. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

12.In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

13. Pick up the correct statement from the following:

a. Catalysts are added to assist and accelerate the hardening of resin,

b. The fillers are inert materials and they impart strength and hardness

c. Fibrous fillers increase thermal resistance

d. All the above.

14. The thermosetting plastic

a. becomes rigid when moulded at suitable pressure and temperature

b. at 127°C to 177°C permanently set and further application of heat does not soften it

c. charrs at 343°C

d. All the above.

15. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

16. Pig iron is manufactured from the ores by

a. dressing

b. calcination and roasting

c. smelting

d. all the above.

17. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

18. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

19. Workability of concrete mix with low water cement ratio is determined by

a. slump test

b. compaction factor test

c. flexural strength test

d. none of these.

20. Sand requiring a high water cement ratio, belongs to

a. Zone I

b. Zone II

c. Zone III

d. Zone IV.

21. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

22. If P, Y and Z are the weights of cement, fine aggregates and coarse aggregates respetively and W/C is the water cement ratio, the minimum quantity of water to be added to first batch, is obtained by the equation

a. 0.3P + 0.1Y + 0.01Z = W/C x P

b. 0.4P + 0.2Y + 0.01Z = W/C x P

c. 0.5P + 0.3Y + 0.01Z = W/C x P

d. 0.2P + 0.5Y + 0.1Z = W/C x P

23. An aggregate which passes through 25 mm I.S. sieve and is retained on 20 mm sieve, is said to be flaky if its least dimension is less than

a. 18.5 mm

b. 16.5 mm

c. 15.5 mm

d. 13.5 mm.

24. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

25. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

26.A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

27. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the degree of saturation, is

a. 95.4%

b. 95.5%

c. 95.6%

d. 95.7%

28. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

29. If water content of a soil is 40%, G is 2.70 and void ratio is 1.35, the degree of saturation is

a. 75%

b. 80%

c. 85%

d. 90%

30. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

31. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

32. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

33. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

34. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

35. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

36. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

37. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these

38. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

39. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

40. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

41. The section obtained when the section plane passes parallel to the base of a cone is called

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

42. The art of representation of an object by systemic lines on a paper is known as

a. drawing

b. art

c. dimensioning

d. profession

43. The unseen/inner edge of an object is represented by

a. solid line

b. dotted line

c. hatch line

d. none of the above

44. Number of equal pieces of A4 size paper from A0 is equal to

a. 2

b. 4

c. 8

d. 16

45. Pick up the ratio which gives us sufficient information by which to judge the financial condition and performance of the firm, from the following:

a. Financial leverage ratio

b. Activity ratio

c. Portability

d. None of these

46. Annuities involve:

a. all payments of equal amount

b. payment at equal time intervals

c. payments at the end of periods.

d. All of these

47. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

48. Prequilification is not required for any types of firm if the costing to buy an equipment is

a. less than 20 million

b. more than 20 million

c. any amount

d. all of the above

49. Fundamental principle of Professional Engineering Ethics is:

a. uploading and advancing Engineering Profession

b. maintain high level of ethical conduct

c. both of the above

d. none of the above

50. Which Ministry of Government of Nepal deals with Civil Engineering group of Nepal Engineering service?

a. Ministry of General Administration

b. Ministry of Energy

c. Ministry of Irrigation

d. Ministry of Physical Infrastructure and Transport