***Engineer***

1. Gradually applied static loads do not change with time their

a. magnitude

b. direction

c. point of application

d. all the above.

2. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

3. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

4. If two forces acting at a joint are not along the straight line, then for the equilibrium of the joint

a. one of the forces must be zero

b. each force must be zero

c. forces must be equal and of the same sign

d. forces must be equal in magnitude but opposite in sign.

5. Along the neutral axis of a simply supported beam

a. fibres do not undergo strain

b. fibres undergo minimum strain

c. fibres undergo maximum strain

d. none of these.

6. Pick up the correct statement from the following :

a. A level line lies in level surface

b. A horizontal surface is normal to the direction of gravity at only one point

c. A horizontal line is tangential to the level surface

d. All the above.

7. The ratio of the angles subtended at the eye, by the virtual image and the object, is known as telescope's

a. resolving power

b. brightness

c. field of view

d. magnification.

8. If deflection angles are measured in a closed traverse, the difference between the sum of the right-hand and that of the left hand angles should be equal to

a. 0°

b. 90°

c. 180°

d. 360°

9. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

10. The most reliable method of plotting a theodolite traverse, is

a. by consecutive co-ordinates of each station

b. by independent co-ordinates of each station

c. by plotting included angles and scaling off each traverse leg

d. by the tangent method of plotting.

11. Pick up the correct statement from the following :

a. the system of a series of lines which forms a circuit which ends at the starting point, is called a closed traverse

b. the traverse that starts from a point already fixed in some survey system and ends on another such point, is called a controlled traverse

c. the traverse that is not controlled is called a fly traverse

d. all the above.

12. Probable systematic error in precise levelling as recommended by International Geodetic Association should not exceed (where k is in kilometers.)

a. ± 0.1 k mm

b. ± 0.2 k mm

c. ± 0.1 k

d. 0.2 k mm.

13. Shingle is

a. decomposed laterite

b. crushed granite

c. water bound pebbles

d. air weathered rock.

14. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

15. With storage, strength of cement

a. increases

b. decreases

c. remains the same

d. none to these.

16. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

17. Bitumen in

a. solid state, is called asphalt

b. semi fluid state, is called mineral tar

c. fluid state, is called petroleum

d. all the above.

18. In arches, stratified stones are placed so that their planes are

a. parallel

b. perpendicular

c. radial

d. none of these.

19. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

20. Separation of coarse aggregates from mortar during transportation, is known

a. creeping

b. segregation

c. shrinkage

d. none of these.

21. If P, Y and Z are the weights of cement, fine aggregates and coarse aggregates respetively and W/C is the water cement ratio, the minimum quantity of water to be added to first batch, is obtained by the equation

a. 0.3P + 0.1Y + 0.01Z = W/C x P

b. 0.4P + 0.2Y + 0.01Z = W/C x P

c. 0.5P + 0.3Y + 0.01Z = W/C x P

d. 0.2P + 0.5Y + 0.1Z = W/C x P

22. Curing of pavements, floors, roofs and slabs, is done by

a. ponding method

b. covering surface with bags

c. sprinkling water method

d. shading concrete method.

23. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

24. Pick up the correct statement from the following:

a. At critical hydraulic gradient, the saturated sand becomes quick

b. The critical gradient depends on the void ratio and the specific gravity

c. The quick sand occurs more in fine sand and silt than coarse material

d. All the above.

25. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

26. If voids ratio is 0.67, water content is 0.188 and specific gravity is 2.68, the degree of saturation of the soil, is

a. 25%

b. 40%

c. 60%

d. 75%

27. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

28. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

29. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

30. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

31. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

32. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

33. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

34. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

35. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

36. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

37. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

38. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

39. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

40. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

41. The area occupied by Ao (drawing sheet) is ......... times the area occupied by A3 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

42. Which of the the following is not intended while making a 'sketch' in the context of technical drawings?

a. a fished product

b. free hand sketching

c. a preliminary product

d. a quick way to record an idea for work

43. The two parts of T-square are....... and .......

a. vertical and horizontal edge

b. straight and slant edge

c. stock and blade

d. blade and batten

44. The view actually appears to the observer when viewed from a fixed position is called

a. isometric drawing

b. orthographic projection

c. perspective drawing

d. none of the above

45. Pick up the correct statement from the following:

a. The capital required to get a project started is the first cost.

b. The first cost is a single cash flow or a series of cash flows that are made in the beginning of the activity's life span

c. The first cost of purchasing a car is the sum of the down payment, taxes and dealers charges.

d. All of these

46. Probabilistic estimating of a construction project includes:

a. Labour

b. Productivity

c. wage scale

d. All of these

47. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

48. In Nepal, which of the following act contains the provisions on health and safety of the workers?

a. contract act

b. industry act

c. labour act

d. all of the above

49. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

50. Labour act was introduced /founded in

a. 2038

b. 2048

c. 2043

d. 2053