***Engineer***

1. The forces in the members of simple trusses, may be analysed by

a. graphical method

b. method of joints

c. method of sections

d. all the above

2. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

3. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

4. If the width of a simply supported beam carrying an isolated load at its centre is doubled, the deflection of the beam at the centre is changed by

a. 4 times

b. 8 times

c. 1/2 times

d. 3 times.

5. The tension coefficient of any member is

a. force divided by the length

b. tension divided by the length

c. tension per unit area

d. tension in the member.

6. Pick up the correct statement from the following

a. the contour lines having the same elevation cannot unite and continue as one line

b. a contour can not end abruptly, but must ultimately close itself not necessarily within the limits of map.

c. the direction of steepest slope at a point on a contour is at right angles to the contour

d. all the above.

7. While measuring the distance between two points along upgrade with the help of a 20 m chain, the forward end of the chain is shifted forward through a distance

a. 20 (sin θ - 1)

b. 20 (cos θ - 1)

c. 20 (sec θ - 1)

d. 20 (cosec θ - 1).

8. On a diagonal scale, it is possible to read up to

a. one dimension

b. two dimensions

c. three dimensions

d. four dimensions.

9. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

10. The first reading from a level station is

a. foresight

b. intermediate sight

c. back-sight

d. any sight.

11. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

12. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

13. Pick up the correct statement from the following:

a. Impurities float on the molten iron as slag

b. The slag contains lime about 45%

c. The molten iron is made to run, in a long channel formed in sand called 'sow'

d. All the above.

14. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

15. The rocks which are formed due to cooling of magma at a relatively shallow depth from the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Ignoeous rocks.

16. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

17. Bituminous fells are used for

a. covering A.C. sheets

b. covering sloping roofs

c. D.P.C.

d. none to these.

18. Clay and silt content in a good brick earth must be at least

a. 50%

b. 40%

c. 30%

d. 25%

19. To obtain a very high strength concrete, use very fine grained

a. Granite

b. Magnetite

c. Barite

d. Volcanic scoria.

20. If the depth of moist sand in a cylinder is 15 cm and the depth of the sand when fully inundated with water is 12 cm, the bulking of the moist sand, is

a. 12%

b. 15%

c. 20%

d. 25%.

21. The ratio of the length to breadth of a wooden float, is

a. 5.5

b. 6.5

c. 7.5

d. 8.5

22. The condition not applicable to water cement ratio law, is

a. concrete specimens may be tested at any temperature

b. concrete specimens need be of same age

c. concrete specimens need be of same size

d. none of these.

23. For compacting plain concrete road surface of thickness less than 20 cm, we use

a. internal vibrator

b. screed vibrator

c. form vibrator

d. none of these.

24. A clay subjected to pressure in excess to its present over-burden, is said to be

a. pre-compressed

b. pre-consolidated

c. over-consolidated

d. all the above.

25. Pick up the correct statement from the following:

a. The void space between the soil grains, is filled partly with air and partly with water

b. In perfectly saturated soil, the voids are completely filled with water

c. In dry soil, the voids are completely filled with air

d. all the above.

26. Pick up the correct statement from the following:

a. The ratio of shear strength in natural state to the remoulded shear strength under undrained conditions, is called degree of sensitivity.

b. The difference between the undisturbed shear strength and remoulded shear strength is known remoulding loss

c. The tendency of dense sand to expand on application of shearing load, is known as dilatancy

d. All the above.

27. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

28. The specific gravity of sands, is approximately

a. 2.0

b. 2.2

c. 2.4

d. 2.6

29. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

30. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

31. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

32. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

33. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

34. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

35. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

36. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

37. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

38. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

39. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

40. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

41. For drawing thin lines of uniform thickness the pencil should be sharpened in the form of

a. chisel edge

b. conical

c. pointed

d. circular

42. The locus of centre of curvature is known as

a. space

b. evolute

c. point

d. centre

43. In drafting machine, which one does not follow :

a. levelling machine

b. T-squared

c. set square

d. divider

44. Third angle projection is the traditional ............. system.

a. American

b. British

c. Indian

d. Nepali

45. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

46. Earning per share is the most important ratio for

a. share holders

b. banks

c. company's management

d. All of these

47. The financial analysis helps to judge:

a. The operational efficiency of the firm

b. The financial position of the firm.

c. Both (a) and (b)

d. Neither (a) nor (b)

48. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr

49. For the fulfillment of his/her professional duties Engineer should:

a. Paramount the health of the public

b. paramount the safety of the public .

c. welfare of the public

d. all of the above

50. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid