***Engineer***

1. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

2. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

3. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

4. An arch with three hinges, is a structure

a. statically determinate

b. statically indeterminate

c. geometrically unstable

d. structurally sound but indeterminate

5. An arch may be subjected to

a. bending moment and shear force

b. bending moment and axial force

c. shear force and thrust

d. thrust, shear force and bending moment.

6. A dumpy level is set up with its eye-piece vertically over a peg A. The height from the top of peg A to the centre of the eye-piece is 1.540 m and the reading on peg B is 0.705 m. The level is then setup over B. The height of the eye-piece above peg B is 1.490 m and a reading on A is 2.195 m. The difference in level between A and B is

a. 3.030 m

b. 0.770 m

c. 0.785 m

d. 1.770 m.

7. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

8. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

9. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

10. Keeping the instrument height as 1  m, length of staff 4 m, the up gradient of the ground 1 in 10, the sight on the up slope must be less than

a. 25 cm

b. 20 m

c. 45 m

d. 10 m

11. The operation of making the algebraic sum of latitudes and departures of a closed traverse, each equal to zero, is known

a. balancing the sights

b. balancing the departures

c. balancing the latitudes

d. balancing the traverse.

12. In setting up a plane table at any station

a. levelling is done first

b. centering is done first

c. both levelling and centering are done simultaneously

d. orientation is done first.

13. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

14. Inner part of a timber log surrounding the pith, is called

a. sapwood

b. cambium layer

c. heart wood

d. none to these.

15. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

16. Pegmatite is a/an

a. intrusive igneous rock

b. extrusive igneous rock

c. sedimentary rock

d. metamorphic rock.

17. The rocks having alumina or clay as their major constituents, are known as

a. argillaceous rocks

b. calcareous rocks

c. sedimentary rocks

d. igneous rocks.

18. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

19. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

20. The operation of removing humps and hollows of uniform concrete surface, is known as

a. screeding

b. trowelling

c. finishing

d. none of these.

21. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

22. The standard sand now a days used in India, is obtained from

a. Jullundur (Punjab)

b. Hyderabad (Andhra Pradesh)

c. Ennore (Madras)

d. Cuttuck (Orissa).

23. The internal dimensions of a ware house are 15 m x 5.6 m, and the maximum height of piles is 2.70 m, the maximum number of bags to be stored in two piles, are

a. 2000 bags

b. 2500 bags

c. 3000 bags

d. 4000 bags

24. The fluid generally used for grouting is

a. clay suspension

b. sodium silicate

c. bitumen emulsion

d. all the above.

25. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

26. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

27. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

28. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

29. The soil which contains finest grain particles, is

a. coarse sand

b. fine sand

c. silt

d. clay.

30. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

31. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

32. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

33. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

34. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

35. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

36. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

37. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

38. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

39. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

40. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

41. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

42. ......... is used to protect the adjoining lines on the drawing when some parts of the line is being erased

a. eraser

b. erasing shield

c. pencil

d. drop compass

43. The techniques for free hand sketch to drawing circle is

a. fixing a fixed point and arc

b. fixing the perimeter

c. fixing the radius only

d. none of the above

44. The angle between isometric axes is

a. 30°

b. 60°

c. 90°

d. 120°

45. The project contractor relies on the cost of the estimate :

a. for submission of a competitive bid for a lumpsum contract

b. for a unit price contract

c. for preparation of a definitive estimate to help negotiate contract.

d. All of these

46. Pick up the element of the cost from the following:

a. direct material

b. direct labour

c. Over head

d. All of these

47. Renu Bala deposits Rs 1200 now, Rs 800 two years from now and Rs 1000 five years from now. If the savings bank's rate of interest in 5%, she will receive an amount of Rs X 10 years from now, where X is

a. Rs 3415

b. Rs 4225

c. Rs 4413

d. Rs 4826

48. Which of the following an engineer should not follow:

a. act as faithful agent for client

b. accept compensation from more than one party for same service without the consent of all

c. indicate client the adverse consequences if judgment is over ruled

d. undertake only these assignments for which he is qualified

49. The number of members in Nepal Engineers' Association are

a. 5

b. 10

c. 15

d. 20

50. Ethics as a normative science needs code of conduct and guidelines to maintaining high level of standard of good behavior in the society. This is applicable to :

a. Lawers

b. doctors

c. Engineers

d. all of the above