***Engineer***

1. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

2. Pick up the correct statement from the following:

a. The shear stress is zero at the outer fibres and maximum at the neutral axis

b. The bending stress at the outer fibres, is known as principal stress

c. The planes of principal stresses are inclined at 45° to the neutral plane

d. All the above.

3. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

4. A short masonry pillar is 60 cm x 60 cm in cross-section, the core of the pillar is a square whose side is

a. 14.14 cm

b. 20.00 cm

c. 22.36 cm

d. 25.22 cm.

5. Pick up the correct statement from the following :

a. In visco-elastic material, stress-strain relation is dependent on time

b. An instropic material has different properties is different directions

c. An orthotropic material has different properties in three mutually perpendicular directions

d. All the above.

6. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

7. The 'point of curve' of a simple circular curve, is

a. point of tangency

b. point of commencement

c. point of intersection

d. mid-point of the curve

8. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

9. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

10. Permanent adjustments of a level are

a. 2 in number

b. 3 in number

c. 4 in number

d. 6 in number

11. Systematic errors are those errors

a. which cannot be recognised

b. whose character is understood

c. whose effects are cumulative and can be eliminated

d. none of these.

12. While measuring a chain line between two stations A and B intervened by a raised ground

a. vision gets obstructed

b. chaining gets obstructed

c. both vision and chaining get obstructed

d. all the above.

13. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

14. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

15. Geologically, marble is known as

a. sedimentary rock

b. igneous rock

c. metamorphic rock

d. stratified rock.

16. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

17. The hardest rock is

a. marble

b. diamond

c. talc

d. quartz.

18. The rocks which are formed due to pouring of magma at the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks

19. Curing of pavements, floors, roofs and slabs, is done by

a. ponding method

b. covering surface with bags

c. sprinkling water method

d. shading concrete method.

20. Concrete containing

a. silicious aggregates, has higher co-efficient of expansion

b. igneous aggregates, has intermediate coefficient of expansion

c. lime stones, has lowest co-efficient of expansion

d. All the above.

21. The operation of removing humps and hollows of uniform concrete surface, is known as

a. screeding

b. trowelling

c. finishing

d. none of these.

22. For preparing a test-specimen, it is necessary

a. to mix coarse aggregates

b. to mix water to the cement, fine aggregates and coarse aggregates

c. to oil inner surface of the mould and the base plate, before placing concrete

d. all the above.

23. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

24. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

25. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

26. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

27. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

28. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

29. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

30. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

31. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

32. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

33. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

34. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

35. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

36. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

37. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

38. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

39. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

40. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

41. The area occupied by Ao (drawing sheet) is .......... times the area occupied by Az (drawing sheet).

a. 2

b. 4

c. 8

d. 16

42. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

43. Axonometric view/drawing is what type of view?

a. one plane

b. two plane

c. three plane

d. six plane

44. ”Title Block” is placed at bottom right hand corner of the drawing which gives information about

a. title of drawing

b. scale of the drawing

c. serial no of drawing

d. all of the above

45. The construction estimate of a project is used by :

a. the owner of the facility

b. the consulting architect/engineer

c. the contractor of the project

d. All of these

46. Pick up the main purpose of project cost control from the following :

a. To signal immediate warning of uneconomic operations

b. To provide a feed back to the estimator

c. To promote cost consciousness

d. All of these

47. A project construction cost estimate includes:

a. the labour and material cost

b. the equipment and over head cost

c. the profit of the contractor

d. All of these

48. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr

49. Engineer shall act in such a manner as to uphold and enhance the honor and dignity of engineering profession and shall act with......for bribery, fraud and corruption.

a. a tolerence

b. a pnnchant

c. a predilection

d. zero tolerance

50. Any Non-Nepali Engineer desiring to work in Nepal may apply for registration under:

a. Category A

b. Category B

c. Category C

d. Category D