***Engineer***

1.  is the equation of Euler's crippling load if

a. both at the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

2. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

3. A compound truss may be formed by connecting two simple rigid frames, by

a. two bars

b. three bars

c. three parallel bars

d. three bars intersecting at a point.

4. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

5. The maximum resistance against rotation, is offered by the weld at a point

a. most distant

b. least distant

c. at either end

d. centrally located.

6. An angle of deflection right, may be directly obtained by setting the instrument to read

a. zero on back station

b. 180° on back station

c. 90°

d. 270° on back station.

7. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

8. If the length of a chain line along a slope of θ° is l, the required slope correction is

a. 2l cos2 θ/2

b. 2l sin2 θ/2

c. l tan2 θ/2

d. l cos2 θ/2.

9. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

10. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

11. A lens or combination of lenses in which the following defect is completely eliminated is called aplanatic

a. spherical aberration

b. chromatic aberration

c. coma

d. astigmatism.

12. In a closed traverse, sum of south latitudes exceeds the sum of north latitudes and the sum of east departures exceeds the sum of west departures, then, the closing line will lie in

a. north-west quadrant

b. north east quadrant

c. south-east quadrant

d. south-west quadrant.

13. Clay and silt content in a good brick earth must be at least

a. 50%

b. 40%

c. 30%

d. 25%

14. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

15. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

16. Quick lime

a. reacts with carbon dioxide

b. may be used for white-washing

c. when mixed with water forms slaked lime

d. all the above.

17. Laterite is a/an

a. volcanic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

18. The material generally not used as extender in paints, is

a. powdered silica

b. gypsum

c. talc

d. zinc white.

19. Workability of concrete for a given water content is good if the aggregates, are

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregates.

20. Pick up the correct statement from the following:

a. Fine sand bulks more than coarse sand

b. With 10% moisture content by weight, the bulking of sand is increased by 50%.

c. The volume of fully saturated sand, is equal to the volume of dry and loose sand

d. All the above.

21. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

22. Water required per bag of cement, is

a. 14 kg

b. 21 kg

c. 28 kg

d. 35 kg

23. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

24. A clay subjected to pressure in excess to its present over-burden, is said to be

a. pre-compressed

b. pre-consolidated

c. over-consolidated

d. all the above.

25. The shearing force acting along the slice of a curved surface of slippage, causes the soil to slide

a. down at the centre

b. down at the toe

c. upward at the centre

d. none of these.

26. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

27. When drainage is permitted under initially applied normal stress only and full primarily consolidation is allowed to take place, the test is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

28. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

29. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

30. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

31. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

32. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

33. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

34. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

35. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

36. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

37. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

38. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

39. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

40. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

41. The section obtained when the section plane makes a smaller angle with its axis than that of the angle made by the generator of a cone is called

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

42. The space between two sentences should be left equal to

a. height of letter

b. twice the ht. of letter

c. 1.5 times ht. of letter

d. none of the above

43. The lines which are drawn to represent visible edges and surface boundaries of objects are called.

a. outlines

b. principle lines

c. both (a) and (b) of above

d. none of the above

44. Size of A4 size paper is

a. 148 x 210 mm

b. 210 x 297 mm

c. 297 x 210 mm

d. 320 X 460 mm

45. Probabilistic estimating of a construction project includes:

a. Labour

b. Productivity

c. wage scale

d. All of these

46. Pick up the element of the cost from the following:

a. direct material

b. direct labour

c. Over head

d. All of these

47. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

48. Labour act was introduced /founded in

a. 2038

b. 2048

c. 2043

d. 2053

49. The general experience required for nomination of chairperson of NEC is:

a. 10 yrs

b. 15 yrs

c. 20 yrs

d. 25 yrs

50. NEC does not approach for

a. engineer

b. overseer to engineer

c. minimum degree B.E.

d. all of the above