***Engineer***

1. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

2. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

3. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

4. A cantilever carries is uniformly distributed load W over its whole length and a force W acts at its free end upward. The net deflection of the free end will be

a. zero

b.  upward

c.  downward

d. none of these.

5. For a stable frame structure, number of members required, is

a. twice the number of joints minus three

b. twice the number of joints minus two

c. twice the number of joints minus one

d. none of these.

6. On a diagonal scale, it is possible to read up to

a. one dimension

b. two dimensions

c. three dimensions

d. four dimensions.

7. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

8. Pick up the correct statement from the following :

a. the eyepiece plays no part in defining the line of sight

b. the diaphragm plays no part in defining the line of sight

c. the optical centre of the objective plays no part in defining the line of sight

d. none of these.

9. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

10. Correction per chain length of 100 links along a slope of α radians, is

a. 100 α2

b. 100 α

c. 100 α3

d. 100 α-1.

11. Bergchrund is a topograhical feature in

a. plains

b. water bodies

c. hills

d. glaciated region

12. For the construction of highway (or railway)

a. longitudinal sections are required

b. cross sections are required

c. both longitudinal and cross sections are required

d. none of these.

13. For a good building stone, its specific gravity should the greater than

a. 1.5

b. 1.7

c. 2.2

d. 2.7.

14. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

15. With storage, strength of cement

a. increases

b. decreases

c. remains the same

d. none to these.

16. The commonly used base for iron and steel work, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

17. Varnish is a transparent or semi-transparent solution of resinuous substances in

a. alcohol

b. linseed

c. turpentine

d. all the above.

18. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

19. The internal dimensions of a ware house are 15 m x 5.6 m, and the maximum height of piles is 2.70 m, the maximum number of bags to be stored in two piles, are

a. 2000 bags

b. 2500 bags

c. 3000 bags

d. 4000 bags

20. Cement used for normal concrete construction, is obtained by burning a mixture of

a. argilaceous and calcareous materials

b. silicious and catcareous materials

c. silicious, argillaceous and calcareous materials

d. none of these.

21. Slump test of concrete is a meausre ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

22. Pick up the incorrect statement from the following:

a. In properly graded aggregates, bulk density is more

b. In single size aggregates, bulk density is least

c. In single size aggregates, bulk density is maximum

d. None of these.

23. The size of fine aggregates does not exceed

a. 3.00 mm

b. 3.75 mm

c. 4.75 mm

d. 5.75 mm

24. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

25. The ratio of the volume of voids to the total volume of the given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

26. The degree of saturation of the soil sample stated in Q. No, 216, is

a. 60%

b. 62%

c. 64%

d. 66%

27. 'Loess' is silty clay formed by the action of

a. water

b. glacier

c. wind

d. gravitational force.

28. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

29. For a homogeneous earth dam 50 m high having 2 m free broad, a flow net was constructed and the results were : Number of potential drops = 2.4 Number of flow channels = 0.4. If coefficiency of permeability of the dam mateiral is 3 x 10-3 cm3/sec, the discharge per metre length of dam, is

a. 12 x 10-5 m3/sec

b. 24 x 10-3 m3/sec

c. 6 x 10-5 m3/sec

d. 24 x 10-5 m3/sec

30. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

31. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

32. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

33. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

34. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

35. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

36. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

37. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

38. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

39. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

40. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

41. The selection of suitable scale and allotment of proper space for margin, title block, revision panel, folding marks, etc. on the drawing sheet is known as:

a. formatting

b. layout

c. folding

d. centering

42. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

43. The art of representation of an object by systemic lines on a paper is known as

a. drawing

b. art

c. dimensioning

d. profession

44. French curve is used to draw

a. regular curve

b. circle

c. irregular curve

d. none of the above

45. Both architect and engineer make use of the cost estimate of the project:

a. for site selection

b. for designing of the project

c. for choosing alternatives

d. All of these

46. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

47. The financial analysis helps to judge:

a. The operational efficiency of the firm

b. The financial position of the firm.

c. Both (a) and (b)

d. Neither (a) nor (b)

48. NEC act and its regulation........

a. 55 and 57

b. 54 and 55

c. 55 and 56

d. 57 and 58

49. In Nepal, which of the following act contains the provisions on health and safety of the workers?

a. contract act

b. industry act

c. labour act

d. all of the above

50. The minimum and maximum no. of members in the user's committee should be

a. 15&17

b. 17&19

c. 9& 15

d. 7&9