***Engineer***

1. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

2. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory.

3. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

4. Failure of riverted joints is due to

a. Tearing of plates between rivets

b. Shearing of rivets

c. Crushing of rivets

d. All the above.

5. The distance between the centres of adjacent rivets in the same row, is called

a. pitch

b. lap

c. gauge

d. staggered pitch.

6. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

7. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

8. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

9. In levelling operation

a. when the instrument is being shifted, the staff must not be moved

b. when the staff is being carried forward, the instrument must remain stationary

c. both (a) and (b)

d. neither (a) nor (b).

10. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

11. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

12. Surveys which are carried out to provide a national grid of control for preparation of accurate maps of large areas, are known

a. plane surveys

b. geodetic surveys

c. geographical surveys

d. topographical surveys.

13. Smith's test of stones is performed to find out

a. the presence of soluble matter of stone

b. the compressive strength of the stone

c. the hardness of the stone

d. the toughness of the stone.

14. Seasoning is

a. a process of removing sap

b. creosoting

c. painting with sodium silicate

d. coating with tar.

15. Permanent magnets are made of high carbon steel and

a. 15% of cobalt

b. 20% of cobalt

c. 35% of cobalt

d. 45% of cobalt.

16. For lime concrete,

a. flexural strength at 90 days is 0.2 N/mm2

b. compressive strength at 90 days is 1.5 N/mm2

c. compressive strength at 26 days is 1.2 N/mm2

d. all the above.

17. Knots in timber are

a. defects caused by crushing fibres

b. splits radiating from the centre

c. speckled strains

d. signs of branches cut off.

18. The specific gravity of marble, is

a. 2.60

b. 2.66

c. 2.72

d. 3.00.

19. Le-Chatelier's apparatus is used for testing

a. soundness of cement

b. hardness of cement

c. strength of cement

d. durability of cement.

20. In the method of voids for determination of the quantity of cement paste, it is assumed that

a. Voids in fine aggregates are filled by the cement paste

b. Volume of fine aggregates is equal to total voids in coarse aggregates plus 10% extra

c. Volume of cement paste required is equal to total volume of voids in fine aggregates plus 15% extra

d. All the above.

21. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

22. Slump test of concrete is a meausre ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

23. Shrinkage in concrete can be reduced by using

a. less cement in the concrete

b. proper concrete mix

c. presaturated aggregates

d. all the above.

24. A moist soil sample of volume 60 cc. weighs 108 g and its dried weight is 86.4 g. If its absolute density is 2.52, the degree of saturation is

a. 54%

b. 64%

c. 74%

d. 84%

25. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

26. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

27. The specific gravity of sands, is approximately

a. 2.0

b. 2.2

c. 2.4

d. 2.6

28. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

29. Stoke's law does not hold good if the size of particle is smaller than

a. 0.0002 mm

b. 0.002 mm

c. 0.02 mm

d. 0.2 mm

30. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

31. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

32. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

33. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

34. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

35. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

36. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

37. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

38. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

39. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

40. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

41. The area occupied by Ao (drawing sheet) is ........... times the area occupied by A4 (drawing sheet)

a. 2

b. 4

c. 8

d. 16

42. Which of the following is not a drawing hand tool?

a. alidade

b. straight edge

c. french curve

d. set squares

43. To measure a fraction we use

a. diagonal scale

b. T-square

c. set square

d. planimeter

44. The projection in which the length, breadth and height of an object is shown in one view is known as:

a. pictorial projection

b. isometric projection

c. cavalier projection

d. cabinet projection

45. The more critical (or severe) test of the firm's liquidity can be judged by :

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

47. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

48. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation

49. Engineering profession needs cooperation with the profession of:

a. medical professional

b. accountant

c. lawyer

d. all of the above

50. Any Non-Nepali Engineer desiring to work in Nepal may apply for registration under:

a. Category A

b. Category B

c. Category C

d. Category D