***Engineer***

1.  is the equation of Euler's crippling load if

a. both at the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

2. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

3. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

4. The cross sections of the beams of equal length are a circle and a square whose permissible bending stress are same under same maximum bending. The ratio of their flexural weights is,

a. 1.118

b. 1.338

c. 1.228

d. 1.108

5. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

6. In an internal focusing telescope

a. the objective is at a fixed distance from the diaphragm

b. the focusing is done by the sliding of a divergent lens.

c. the focusing divergent lens is situated at about the middle of the tube

d. all the above.

7. Pick up the correct statement from the following :

a. a back sight is the first sight taken after setting up the instrument in any position

b. the first sight on each change point is a fore sight

c. the second sight on each change point is a back sight

d. all the above.

8. The defect of a lens whereby rays of white light proceeding from a point get dispersed into their components and conveyed to various foci, forming a blurred and coloured image is known as

a. chromatic aberration

b. spherical aberration

c. astigmatism

d. coma.

9. Magnetic declination at any place

a. remains constant

b. does not remain constant

c. fluctuates

d. changes abruptly.

10. The operation of revolving a plane table about its vertical axis so that all lines on the sheet become parallel to corresponding lines on the ground, is known

a. levelling

b. centering

c. orientation

d. setting.

11. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

12. An angle of deflection right, may be directly obtained by setting the instrument to read

a. zero on back station

b. 180° on back station

c. 90°

d. 270° on back station.

13. Duco paints are

a. cellulose paints

b. emulsion paints

c. bituminous paints

d. oil paints.

14. Distemper is

a. a paint consisting of powdered chalk, pigments, and water

b. a water proofing agent

c. a paint consisting of coloured cement and water

d. a drying agent.

15. A good quality stone absorbs water less than

a. 5%

b. 10%

c. 15%

d. 20%

16. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

17. The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

18. Strength of cement concrete primarily depends upon

a. quality of water

b. quantity of aggregate

c. quantity of cement

d. water-cement ratio.

19. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

20. For the construction of R.C.C. slabs, columns, beams, walls, etc. the grade of concrete mix used, is

a. 1 :  : 3

b. 1 : 2 : 4

c. 1 : 1 : 2

d. 1 : 4 : 8.

21. The produce impermeable concrete

a. proper compaction of concrete is required

b. proper curing of concrete is required

c. properly graded and non-porous aggregates are requied

d. all the above.

22. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

23. The grade of concrete M 150 means that compressive strength of a 15 cm cube after 28 days, is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

24. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

25. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

26. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

27. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

28. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

29. Plasticity index is defined as the range of water content between

a. liquid and plastic limit

b. plastic limit and semi solid limit

c. semi-solid limit and liquid limit

d. liquid limit and solid limit.

30. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

31. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

32. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

33. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

34. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

35. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

36. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

37. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

38. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

39. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

40. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

41. In orthographic projection which one have all three views same

a. cube

b. circle

c. sphere

d. both (a) and (c) of above

42. Find the RF for scale 1cm= 25 km

a. 1/250

b. 1/2500

c. 1/25000

d. 1/2500000

43. Pencil of ...... grade sharpened in the form of ......... is used for sketching

a. soft & conical point

b. low and conical point

c. hard & conical point

d. chisel & conical point

44. The techniques for free hand sketch to drawing circle is

a. fixing a fixed point and arc

b. fixing the perimeter

c. fixing the radius only

d. none of the above

45. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

46. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

47. A construction estimate is used

a. to judge tentatively or approximate value of the project

b. to produce a statement of the approximate cost

c. to decide an approximation of the value of the project and not the exact cost.

d. None of these

48. Engineer shall practices:

a. in favour of client

b. in favour of contractor

c. in favour of him/herself

d. impartial andefficient professional relationship with client, contractor & fellow personnels

49. Engineers shall build their professional reputation on the ..........of their services and shall not compete unfairly with others.

a. publicity

b. basis

c. merit

d. reputation

50. Engineer shall act in such a manner as to uphold and enhance the honor and dignity of engineering profession and shall act with......for bribery, fraud and corruption.

a. a tolerence

b. a pnnchant

c. a predilection

d. zero tolerance