***Engineer***

1. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises.

2. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

3. A cantilever of length 2 cm and depth 10 cm tapers in plan from a width 24 cm to zero at its free end. If the modulus of elasticity of the material is 0.2 x 106 N/mm2, the deflection of the free end, is

a. 3 mm

b. 4 mm

c. 5 mm

d. 6 mm

4. If two forces acting at a joint are not along the straight line, then for the equilibrium of the joint

a. one of the forces must be zero

b. each force must be zero

c. forces must be equal and of the same sign

d. forces must be equal in magnitude but opposite in sign.

5. While testing a cast iron beam (2.5 cm x 2.5 cm) in section and a metre long simply supported at the ends failed when a 100 kg weight is applied at the centre. The maximum stress induced is :

a. 960 kg/cm2

b. 980 kg/cm2

c. 1000 kg/cm2

d. 1200 kg/cm2.

6. In a theodolite

a. the telescope axis is perpendicular to transit axis

b. the axis of rotation is perpendicular to transit axis

c. the telescope axis, the transit axis and the rotation axis pass through the centre of theodolite

d. all the above.

7. The  lens  equation   is applicable

a. when the thickness of the lens is small

b. only to conjugate distances along the principal axis

c. the computed quantities are accurate enough for geometrical optics of simple distances

d. all the above.

8. The 'fix' of a plane table from three known points, is good, if

a. middle station is nearest

b. middle station is farthest

c. either the right or left station is nearest

d. none of these.

9. Pick up the correct statement from the following :

a. if the image of the object does not fall on the plane of the cross-lines, parallax exists

b. parallax has nothing to do with the eyepiece

c. the eyepiece is adjusted for clear vision of the cross hairs

d. all the above.

10. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

11. Simpson's rule for calculating areas states that the area enclosed by a curvilinear figure divided into an even number of strips of equal width, is equal to

a. one third the width of a strip, multiplied by the sum of two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

b. one third the width of a strip, multiplied by the sum of two extreme offsets, four times the sum of the remaining odd offsets, and twice the sum of the even offsets

c. one sixth the width of a strip, multiplied by the sum of the two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

d. none of these

12. Pick up the correct specification of Ramsden eyepiece from the following :

a. the curved surfaces of plano-convex lenses face each other

b. the two lenses are separated by a distance equal to 2/3 of the focal length of either lens.

c. the distance between the diaphragm and the front lens of the eyepiece is kept equal to 1/4 th of the focal length of a lens so that rays from a point on the diaphragm enter the eye as a parallel beam

d. all the above.

13. The steel used for rails under heavy traffic and on sharp curves, is

a. Nickel steel

b. Chrome steel

c. Magnese steel

d. Vanadium steel.

14. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

15. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

16. Pick up the correct statement from the following: Method of sawing timber

a. tangentially to annual rings, is known as tangential method.

b. in four quarters such that each board cuts annual rings at angles not less than 45°, is known as quarter sawing method.

c. cut out of quarter logs, parallel to the medullary rays and perpendicular to annual rings, is known as radial sawing.

d. all the above.

17. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

18. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

19. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

20. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

21. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

22. Pick up the correct statement from the following:

a. Continuous grading is not necessary for obtaining a minimum of air voids

b. The omission of a certain size of aggregate is shown by a straight horizontal line on the grading curve

c. The omission of a certain size of aggregate in concrete increases the workability but also increases the liability to segregation

d. All the above.

23. The grade of concrete not recommended by I.S. : 456, is

a. M 200

b. M 300

c. M 400

d. M 500

24. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

25. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

26. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

27. If water content of a soil is 40%, G is 2.70 and void ratio is 1.35, the degree of saturation is

a. 75%

b. 80%

c. 85%

d. 90%

28. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

29. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

30. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

31. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

32. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

33. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

34. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

35. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

36. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

37. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

38. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

39. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

40. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

41. For drawing thin lines of uniform thickness the pencil should be sharpened in the form of

a. chisel edge

b. conical

c. pointed

d. circular

42. The main purpose of leader line is

a. show dimension

b. to connect a note with the feature

c. to show beauty

d. to show hatch

43. Working drawing is prepared not for

a. bill of quantities

b. estimation

c. layout

d. specification

44. During drawing stock of T-square is placed

a. adjoining the working edge of the board

b. adjoining the sliding edge of the board

c. surface of the board,

d. none of the above

45. Pick up the correct statement from the following:

a. Both the numerator and denominator of financial ratios come directly from the balance sheet.

b. Income statement ratios compare one 'flow' item from the income statement to another flow item from the income statement.

c. Income statement ratios compare a flow item from the income statement to another flow item form the income statement

d. All of these

46. In a cash flow series :

a. uniform gradient signifies that an income or disbursement changes by the same amount in each interest period.

b. Either an increase or a decrease in the amount of a cash flow is called the gradient.

c. The gradient in the cash flow may be positive or negative.

d. All of these

47. The alternatives which are standalone solutions for given situations in engineering involve :

a. the anticipated life of the assest

b. the yearly costs of maintaining the assest (annual maintenance and operating cost)

c. the anticipated resaleable value (salvage value) and the interest return (rate of return)

d. All of these

48. Who is the Chairperson of House Rent Fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

49. The registered engineer in Nepal Engineering Council should have to renew their registration in:

a. every year

b. every two years

c. every four years

d. none of the above

50. The time between ordering of the goods or materials and receiving and placing them into use at the point of need is called

a. order-interval

b. lead time

c. usage time

d. delayed time