***Engineer***

1. At any point of a beam, the section modulus may be obtained by dividing the moment of inertia of the section by

a. depth of the neutral axis

b. maximum tensile stress at the section

c. maximum compressive stress at the section

d. none of these

2. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

3. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

4. The bending moment is maximum on a section where shearing force

a. is maximum

b. is minimum

c. is equal

d. changes sign.

5. The maximum resistance against rotation, is offered by the weld at a point

a. most distant

b. least distant

c. at either end

d. centrally located.

6. Pick up the correct statement from the following:

a. the theodolite in which telescope can be rotated in vertical plane is called a transit

b. when the vertical circle is to the left of the telescope during observation, it is called to be in left face

c. when the vertical circle is to the right of the telescope during observation, it is called to be in right face

d. all the above.

7. If i is the stadia distance, f is the focal length and d is the distance between the objective and vertical axis of the techeometer, the multiplying constant, is

a. f/i

b. i/f

c. (f + d)

d. f/d.

8. In reciprocal levelling, the error which is not completely eliminated, is due to

a. earth's curvature

b. non-adjustment of line of collimation

c. refraction

d. non-adjustment of the bubble tube.

9. The surface of zero elevation around the earth, which is slightly irregular and curved, is known as

a. mean sea level

b. geoid surface

c. level surface

d. horizontal surface.

10. In case of a direct vernier scale

a. graduations increase in opposite direction in which graduations of the main scale increase

b. smallest division is longer than smallest division of the main scale

c. graduations increase in the same direction in which graduations of the main scale increase

d. none of these.

11. Two hill tops A and B 20 km apart are intervened by a third top C. If the top most contour of the three hill tops are of the same value, state whether the line of sight AB

a. passes clear of hill top C

b. passes below the hill top C

c. grazes the hill top C

d. none of these.

12. In a precision traverse, included angles are measured by setting the vernier

a. to read zero exactly on back station

b. to read 5° exactly on back station

c. somewhere near zero and reading both verniers on back station

d. all the above.

13. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

14. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

15. Pick up the correct statement from the following:

a. The plastic sheets are made by the calendering process in which the plastic material is allowed to pass between cylindrical rollers

b. The application of thermo-setting resins on sheets of paper, is called laminating process

c. The plastic articles made by placing raw material in the desired moulds, is known as moulding process

d. All the above.

16. Rapid hardening cement attains early strength due to

a. larger proportion of lime grounded finer than normal cement

b. lesser proportion of lime grounded coarser than normal cement

c. lesser proportion of lime grounded finer than normal cement

d. larger proportien of lime grounded coarser than normal cement

17. Pick up the correct statement from the following:

a. Attrition test is done to find out the rate of wear of stones which are used in road construction

b. Crushing test is done to find out the compressive strength of the stone

c. Impact test is done to determine toughness of a stone

d. All the above.

18. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

19. Workability of concrete for a given water content is good if the aggregates, are

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregates.

20. Hardening of cement occurs at

a. rapid rate during the first few days and afterwards it continues to increase at a decreased rate

b. slow rate during the first few days and afterwards it continues to increase at a rapid rate

c. uniform rate throughout its age

d. none of these.

21. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

22. Pick up the correct statement from the following:

a. The free water is the amount of water added while mixing and the amount of water held on the surface of the aggregates prior to mixing

b. The total water is the free water and the amount actually absorbed by the aggregates

c. Neither (a) nor (b)

d. Both (a) and (b).

23. For the construction of the retaining structures, the type of concrete mix to be used, is

a. 1 : 2 :4

b. 1 :  : 3

c. 1 : l : 2

d. 1 : 4 : 8.

24. Tergazhi's theory of one dimensional consolidation assumes

a. water and soil particles are incompressible

b. deformation of the soil, is entirely due to change in volume

c. Darcey's law for the velocity of flow of water through soil, is perfectly valid

d. all the above.

25. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

26. The fluid generally used for grouting is

a. clay suspension

b. sodium silicate

c. bitumen emulsion

d. all the above.

27. The property of a soil which allows it to be deformed rapidly without rupture, elastic rebound and also a volume change, is known

a. porosity

b. plasticity

c. permeability

d. ductility.

28. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

29. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

30. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

31. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

32. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

33. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

34. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

35. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

36. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

37. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

38. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

39. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

40. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

41. Which one is not drafting equipment ?

a. drafter

b. dust cloth

c. scale

d. None of the above

42. Circle and arcs of circles are drawn by means of a .......

a. lengthening bar

b. divider

c. compass

d. T-square

43. The isometric length of the edge of the cube is obtained by

a. adding 0.815 in actual length

b. dividing actual length by 0.815

c. multiplying actual length by 0.815

d. none of the above

44. Uses of the T-square, set squares, scale and protractor are combined in the

a. set-square

b. drafting machine

c. compass

d. none of the above

45. Pick up the correct statement from the following:

a. The manifestation of the time value of money is termed as interest.

b. Interest on borrowing = present amount owed - original loan

c. The original investment (or loan) is referred to as principal.

d. All of these.

46. Pick up the correct statement from the following:

a. Engineering economy is a collection of mathematical techniques which simplify economic comparisons

b. Engineering economy is a decision assistance tool by which one method will be chosen as the most economically one.

c. For understanding the engineering economy, one should be able to classify the basic terminology and fundamental concepts of economy.

d. All of these.

47. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

48. Collusive practice" means

a. offering anything to influence the action of public official

b. arrangement between two or more parties to achieve an improper purpose including influencing improperly the action of another party

c. mis representation of facts in order to influence a procurement process

d. all of the above

49. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer

50. For the registration of foregineer engineers the amount of fee required is:

a. NRS 3000/-

b. NRS 5000/-

c. NRS 10000/-

d. NRS 15000/-