***Engineer***

1. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

2. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

3. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

4. The maximum bending moment due to a moving load on a simply supported beam, occurs

a. at the supports

b. under the load

c. anywhere on the beam

d. none of these.

5. A 8 metre long simply supported rectangular beam which carries a distributed load 45 kg/m. experiences a maximum fibre stress 160 kg/cm2. If the moment of inertia of the beam is 640 cm4, the overall depth of the beam is

a. 10 cm

b. 12 cm

c. 15 cm

d. 16 cm

6. The construction of optical square is based, on the principle of optical

a. reflection

b. refraction

c. double refraction

d. double reflection.

7. Pick up the correct statement from the following :

a. a back sight is the first sight taken after setting up the instrument in any position

b. the first sight on each change point is a fore sight

c. the second sight on each change point is a back sight

d. all the above.

8. Two contour lines, having the same elevation

a. cannot cross each other

b. can cross each other

c. cannot unite together

d. can unite together.

9. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

10. Metric chains are generally available in

a. 10 m and 20 m length

b. 15 m and 20 m length

c. 20 m and 30 m length

d. 25 m and 100 m length

11. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

12. In tangential tacheometry, an ordinary level staff is used

a. leaning towards the instrument for inclined sights upward

b. leaning away from the instrument for inclined sights downwards

c. vertical in all cases

d. none of these.

13. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

14. Pick up the hypabyssal rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

15. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

16. Quick setting cement is produced by adding

a. more amount of gypsum in very fine powdered form

b. aluminium sulphate in very fine powdered form

c. pozzolana in very fine powdered form

d. none of these.

17. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

18. Asbestos

a. is a natural fibrous mineral substance

b. is composed of hydrous silicates of calcium and magnesium (CaSiO3, 3MgSiO3)

c. contains iron oxide and alumina

d. all the above.

19. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

20. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

21. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

22. Sand generally contains salt if it is obtained from:

a. river beds

b. sea beds

c. all the above

d. none of these.

23. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

24. Factor of safety against sliding of a slope, is the ratio of

a. actual cohesion to that required to maintain stability of slope

b. shear strength to shear stress along the surface

c. neither (a) nor (b)

d. both (a) and (b).

25. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

26. The reduction in volume of soil due to squeezing out of water from the voids, is termed

a. primary consolidation

b. primary compression

c. primary time effect

d. all the above.

27. Over-consolidation of soils is caused due to

a. erosion of over burden

b. melting of ice sheets after glaciation

c. permanent rise of water table

d. all the above.

28. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

29. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

30. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

31. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

32. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

33. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

34. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

35. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

36. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

37. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

38. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

39. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

40. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

41. Site plan is a:

a. structural plan

b. architecturel plan

c. section plan

d. none of the above 1

42. Select the incorrect statement about orthographic projections :

a. the reference planes are also called the principle planes

b. the object must the situated in the first quadrant

c. the planes of projection and assumed to be transparent

d. projections are obtained by drawing perpendicular s from the object to the planes

43. The edge of drawing board where T-square is made to slide is called

a. sliding edge

b. fixing edge

c. working edge

d. all of the above

44. The isometric length of the edge of the cube is obtained by

a. adding 0.815 in actual length

b. dividing actual length by 0.815

c. multiplying actual length by 0.815

d. none of the above

45. The key to profitable operation for project cost control, is :

a. To keep the project cost equal to original cost estimate.

b. To keep the project cost equal to subsequent construction budget.

c. To keep the project cost within the cost budget and knowing when and where job costs are deviating.

d. None of these

46. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

47. Pick up the element of the cost from the following:

a. direct material

b. direct labour

c. Over head

d. All of these

48. For the fulfillment of his/her professional duties Engineer should:

a. Paramount the health of the public

b. paramount the safety of the public .

c. welfare of the public

d. all of the above

49. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation

50. As per the prevalling Public Procurement Acts & Regulations of Nepal, provision of price adjustments is normally to inculpated in civil works contract having project duration ?

a. more than 12 months

b. more than 15 months

c. more than 18 months

d. more than 24 months