***Engineer***

1. A compound truss may be formed by connecting two simple rigid frames, by

a. two bars

b. three bars

c. three parallel bars

d. three bars intersecting at a point.

2. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

3. The ratio of moments of inertia of a triangular section about its base and about a centroidal axis parallel to its base, is

a. 1.5

b. 2.0

c. 2.5

d. 3.0

4. If a three hinged parabolic arch carries a uniformly distributed load on its entire span, every section of the arch resists.

a. compressive force

b. tensile force

c. shear force

d. bending moment.

5. When a rectangular beam is loaded transversely, the maximum compressive stress develops on

a. bottom fibre

b. top fibre

c. neutral axis

d. every cross-section.

6. The properties of autogenous curve for automobiles are given by

a. true spiral

b. cubic parabola

c. Bernoulli's Lemniscate

d. clothoid spiral.

7. Planimeter is used for measuring

a. area

b. contour gradient

c. slope angle

d. none of these

8. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

9. If deflection angles are measured in a closed traverse, the difference between the sum of the right-hand and that of the left hand angles should be equal to

a. 0°

b. 90°

c. 180°

d. 360°

10. Perpendicular offset from a tangent to the junction of a transition curve and circular curve is equal to

a. shift

b. twice the shift

c. thrice the shift

d. four times the shift.

11. The first reading from a level station is

a. foresight

b. intermediate sight

c. back-sight

d. any sight.

12. For orientation of a plane table with three points A, B and C, Bessel's drill is

a. Align b through a and draw a ray towards c, align a through b and draw a ray towards c, finally align c through the point of intersection of the previously drawn rays

b. Align c through a and draw a ray towards b, align a through c and draw a ray towards b, finally align b through the point of intersection of the previously drawn rays

c. Align c through b and draw a ray towards a, align b through c and draw a ray towards a, finally align a, through the point of intersection of the previously, drawn rays

d. In the first two steps any two of the points may be used and a ray drawn towards the third point, which is sighted through the point of intersection of previously drawn rays in the final step.

13. A prime coat is given to steel work with

a. an oxide of iron paint

b. a mixture of white lead and lead paint

c. a special paint

d. cement paint.

14. The harmonious mixing of the clay ingredients, is known as

a. weathering

b. blending

c. tempering

d. None of these.

15. For light and ornamental casting, the most unsuitable pig iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

16. Lime putty

a. is made by adding lime to water

b. can be used only upto three days

c. may be obtained from drying lime water mix passing through IS sieve No. 300

d. all of above.

17. Fibre glass

a. has a higher strength to weight ratio

b. is shock proof and fire retardent

c. does not decay

d. all the above.

18. Polymerization helps to improve the property of

a. strength

b. rigidity

c. elasticity

d. all of these.

19. For an ordinary Portland cement

a. soundness varies from 5 to 10 mm

b. initial setting time is not less than 30 minutes

c. compressive stress after 7 days, is not less than 175 kg/cm2

d. all the above.

20. Ordinary Portland cement is manufactured from

a. lime stone and clay

b. gypsum and lime

c. pozzolana

d. lime, pozzolana and clay.

21. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

22. Pick up the correct statement from the following:

a. High percentage of C3S and low percentage of C2S make the cement less resistive to chemical attack

b. Low percentage of C3S and high percentage of C2S contribute to slow hardening

c. Low percentage of C3S and high percentage of C2S provide greater resistance to chemical attack

d. All the above.

23. According to Water-Cement Ratio Law, the strength of workable plastic concrete

a. depends upon the amount of water used in the mix

b. does not depend upon the quality of cement mixed with aggregates

c. does not depend upon the quanity of cement mixed with aggregates

d. all the above.

24. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

25. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

26. The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

27. A critical hydraulic gradient may occur when

a. flow is in upward direction

b. seepage pressure is in upward direction

c. effective pressure is zero

d. all the above.

28. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

29. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

30. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

31. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

32. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

33. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

34. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

35. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

36. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

37. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

38. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

39. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

40. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

41. Size of A4 size paper is

a. 148 x 210 mm

b. 210 x 297 mm

c. 297 x 210 mm

d. 320 X 460 mm

42. The set of technical drawings used during the manufacturing phase of product including cvili construction, are known as

a. structural drawing

b. as built drawing

c. structural drawing

d. working drawing

43. In general structural drawings would typically include following information:

a. north point

b. plan, section and elevation

c. notes on specification

d. all of the above

44. Number of equal pieces of A4 size paper from A0 is equal to

a. 2

b. 4

c. 8

d. 16

45. Pick up the correct statement from the following:

a. The manifestation of the time value of money is termed as interest.

b. Interest on borrowing = present amount owed - original loan

c. The original investment (or loan) is referred to as principal.

d. All of these.

46. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

47. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

48. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer

49. Who is the Chairperson of Project Co-ordination Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

50. Nepal engineering council is an autonomous body formed under NEC act in

a. 2056 BS

b. 2055 BS

c. 2054 BS

d. 2053 BS