***Engineer***

1. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

2. A masonry dam (density = 20, 000 N/m3) 6 m high, one metre wide at the top and 4 m wide at the base, has vertical water face. The minimum stress at the base of the dam when the reservoir is full, will be

a. 75 N/m2

b. 750 N/m2

c. 7500 N/m2

d. 75000 N/m2

3. Gradually applied static loads do not change with time their

a. magnitude

b. direction

c. point of application

d. all the above.

4. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

5. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

6. In an internal focusing telescope

a. the objective is at a fixed distance from the diaphragm

b. the focusing is done by the sliding of a divergent lens.

c. the focusing divergent lens is situated at about the middle of the tube

d. all the above.

7. One of the Lehmann's rules of plane tabling, is

a. location of the instrument station is always distant from each of the three rays from the known points in proportion to their distances

b. when looking in the direction of each of the given points, the instrument station will be on the right side of one and left side of the other ray

c. when the instrument station is outside the circumscribing circle its location is always on the opposite side of the ray to the most distant point as the inter-section of the other two rays

d. none of these.

8.  is a conventional sign of

a. mosque

b. temple

c. church

d. idgah.

9. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

10. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

11. The zero of the graduated circle of a prismatic compass is located at

a. north end

b. east end

c. south end

d. west end.

12. Profile levelling is usually done for determining

a. contours of an area

b. capacity of a reservoir

c. elevations along a straight line

d. boundaries of property

13. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

14. Pick up the correct statement regarding low heat cement from the following:

a. Its initial setting time is about one hour

b. Its final setting time is about 10 hours

c. Its mainly used for mass concrete work

d. All the above.

15. For construction of structures under water, the type of lime used, is

a. hydraulic lime

b. fat lime

c. quick lime

d. pure lime

16. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

17. Refractory bricks are used for

a. retaining walls

b. columns

c. piers

d. combustion chambers.

18. Most commonly used solvent in oil paints, is

a. petroleum

b. spirit

c. coaltar

d. turpentine.

19. If the effective plan area of a warehouse is 54 sq. m, and maximum height of piles permitted is 270 cm, the number of cement bags to be stored, is

a. 2200 bags

b. 2400 bags

c. 2700 bags

d. 3000 bags

20. Pick up the correct statement from the following:

a. Continuous grading is not necessary for obtaining a minimum of air voids

b. The omission of a certain size of aggregate is shown by a straight horizontal line on the grading curve

c. The omission of a certain size of aggregate in concrete increases the workability but also increases the liability to segregation

d. All the above.

21. If 50 kg of fine aggregates and 100 kg of coarse agregates are mixed in a concrete whose water cement ratio is 0.6, the weight of water required for harsh mix, is

a. 10 kg

b. 12 kg

c. 14 kg

d. 15 kg.

22. If the depth of moist sand in a cylinder is 15 cm and the depth of the sand when fully inundated with water is 12 cm, the bulking of the moist sand, is

a. 12%

b. 15%

c. 20%

d. 25%.

23. Concrete containing

a. silicious aggregates, has higher co-efficient of expansion

b. igneous aggregates, has intermediate coefficient of expansion

c. lime stones, has lowest co-efficient of expansion

d. All the above.

24. Degree of saturation of a natural soil deposit having water content 15%, specific gravity 2.50 and void ratio 0.5, is

a. 50%

b. 60%

c. 75%

d. 80%

25. For testing a saturated clay for shear strength, the test recommended, is

a. direct shear test

b. triaxial compression test

c. unconfined compression test

d. all the above.

26. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

27. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

28. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

29. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

30. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

31. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

32. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

33. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

34. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

35. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

36. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

37. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

38. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

39. The following item of earth work is not measured separately.

a. Site clearance

b. dead men

c. Steps in deep excavation

d. All the above.

40. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the center line of the pipes in meters

c. Lead caulked joints are enumerated separately

d. All the above.

41. The lines which are drawn to represent visible edges and surface boundaries of objects are called.

a. outlines

b. principle lines

c. both (a) and (b) of above

d. none of the above

42. Which one is not drafting equipment ?

a. drafter

b. dust cloth

c. scale

d. None of the above

43. Picture plane is

a. transparent

b. opaque

c. semi transparent

d. all of the above

44. ......... is used to protect the adjoining lines on the drawing when some parts of the line is being erased

a. eraser

b. erasing shield

c. pencil

d. drop compass

45. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square meter method

c. Cost per cubic meter method

d. Cost per linear meter method

46. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

47. Pick up the method used for project evaluation and selection in capital budgetting from the following:

a. Internal ratio of return

b. Net present worth

c. Profitability index

d. All the above

48. The employer's have right to

a. accept the bid

b. reject the bid

c. both (a) and (b) of above

d. none of the above

49. Ethics as a normative science needs code of conduct and guidelines to maintaining high level of standard of good behavior in the society. This is applicable to :

a. Lawers

b. doctors

c. Engineers

d. all of the above

50. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr