***Engineer***

1. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these

2. The forces in the members of simple trusses, may be analysed by

a. graphical method

b. method of joints

c. method of sections

d. all the above.

3. Stress may be expressed in Newtons

a. per millimeter square (N/mm2)

b. per centimeter square (N/cm2)

c. per meter square (N/m2)

d. All of the above.

4. The deflection of any rectangular beam simply supported, is

a. inversely proportional to its width

b. inversely proportional to the cube of its depth

c. directly proportional to the cube of its length

d. none of these.

5. The moment diagram for a cantilever whose free end is subjected to a bending moment, will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

6. Pick up the correct statement from the following :

a. it is difficult to eliminate an error completely at first trial

b. instability of the instrument makes it almost impossible to adjust it satisfactorily

c. adjustment screws must be left bearing firmly but should never be forced

d. all the above.

7. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

8. Location of contour gradient for a high way is best set out from

a. ridge down the hill

b. saddle down the hill

c. bottom to the ridge

d. bottom to the saddle.

9. Offsets are measured with an accuracy of 1 in 40. If the point on the paper from both sources of error (due to angular and measurement errors) is not to exceed 0.05 cm on a scale of 1 cm = 20 m, the maximum length of offset should be limited to

a. 14.14

b. 28.28 m

c. 200 m

d. none of these.

10. Two concave lenses of 60 cm focal length are cemented on either side of a convex lens of 15 cm focal length. The focal length of the combination is

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

11. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

12. Subtense tacheometry is generally preferred to if ground is

a. flat

b. undulating

c. mountaineous

d. deserts.

13. The cracks which extend from bark towards the sap wood in the cross section of a tree, are called

a. radial shakes

b. star shakes

c. heart shakes

d. cup shakes.

14. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

15. For slaking of 10 kg of CaO, the theoretical amount of water is

a. 2.2 kg

b. 1.5 kg

c. 3.2 kg

d. None of these.

16. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

17. Polymerization helps to improve the property of

a. strength

b. rigidity

c. elasticity

d. all of these.

18. Pick up the correct statement from the following:

a. Solder material is an alloy which melts at a temperature above 400°C

b. Brazing is done at temperature above 600°C to 1100°C

c. Brazing joint is stronger than the solder joint

d. all the above.

19. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

20. According to Water-Cement Ratio Law, the strength of workable plastic concrete

a. depends upon the amount of water used in the mix

b. does not depend upon the quality of cement mixed with aggregates

c. does not depend upon the quanity of cement mixed with aggregates

d. all the above.

21. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

22. Curing of pavements, floors, roofs and slabs, is done by

a. ponding method

b. covering surface with bags

c. sprinkling water method

d. shading concrete method.

23. A concrete using an air entrained cement

a. has more resistance to weathering

b. is more plastic and workable

c. is free from segregation and bleeding

d. all the above.

24. Plasticity index is defined as the range of water content between

a. liquid and plastic limit

b. plastic limit and semi solid limit

c. semi-solid limit and liquid limit

d. liquid limit and solid limit.

25. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

26. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

27. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

28. Pick up the correct statement from the following:

a. The dry density reduces by addition of water after attaining optimum moisture content

b. The line joining the peak of three moisture content graphs obtained by using three compactive energies, is called line of optimus

c. Well graded coarse grained soils can be compacted to a very high density as compared to fine grained soils

d. All the above. .

29. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

30. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

31. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

32. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

33. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

34. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

35. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

36. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

37. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

38. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

39. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

40. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

41. The free hand drawing of a ordinary room is generally:

a. one point perspective

b. two pint perspective

c. three point perspective

d. all of the above

42. Working drawing is prepared not for

a. bill of quantities

b. estimation

c. layout

d. specification

43. The principal of oblique view drawing is to keep:

a. width axes at 90°

b. width and height axes at 90°

c. width, height and depth axes at 90°

d. all of the above

44. Axonometric drawing is a drawing like

a. parallel to the plane

b. perpendicular to the plane

c. inclined to the plane

d. all of the above

45. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

46. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

47. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

48. The Nepal Engineers' Association was established in

a. 2014 BS

b. 2024 BS

c. 2034 BS

d. 2044 BS

49. NEC does not approach for

a. engineer

b. overseer to engineer

c. minimum degree B.E.

d. all of the above

50. The space required for living the worker is 15m with considering the maximum height of room above floor level.

a. 3m

b. 3.5m

c. 4m

d. 5 m