***Engineer***

1. A steel rod of sectional area 250 sq. mm connects two parallel walls 5 m apart. The nuts at the ends were tightened when the rod was heated to 100°C. If αsteel = 0.000012/C°, Esteel = 0.2 MN/mm2, the tensile force developed at a temperature of 50°C, is

a. 80 N/mm2

b. 100 N/mm2

c. 120 N/mm2

d. 150 N/mm2

2. At any point of a beam, the section modulus may be obtained by dividing the moment of inertia of the section by

a. depth of the neutral axis

b. maximum tensile stress at the section

c. maximum compressive stress at the section

d. none of these

3. The forces in the members of simple trusses, may be analyzed by

a. graphical method

b. method of joints

c. method of sections

d. all the above.

4. During a tensile test on a ductile material

a. nominal stress at fracture is higher than the ultimate stress

b. true stress at fracture is higher than the ultimate stress

c. true stress at fracture is the same as the ultimate stress

d. none of these.

5. If the beam is supported so that there are only three unknown reactive elements at the supports. These can be determined by using the following fundamental equation of statics

a. ∑V = 0

b. ∑H = 0; ∑H = 0

c. ∑H = 0; ∑V = 0; ∑M = 0

d. ∑M = 0; ∑H = 0

6. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

7. Pick up the method of surveying in which field observations and plotting proceed simultaneously from the following

a. chain surveying

b. compass surveying

c. plan table surveying

d. tacheometric surveying.

8. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

9. A sewer is laid from a manhole A to a manhole B, 250 m away along a gradient of 1 in 125. If the reduced level of the invert at A is 205.75 m and the height of the boning rod is 3 m, the reduced level of the sight rail at B, is

a. 208.75 m

b. 202.75 m

c. 206.75 m

d. 211.75 m

10. If the plane table is not horizontal in a direction at right angles to the alidade, the line of sight is parallel to the fiducial edge only for

a. horizontal sights

b. inclined sights upward

c. inclined sight downward

d. none of these.

11. The formula for the horizontal distances for inclined sights, on staff held normalis  s cos θ + (f + d) cos θ ± h sin θ

a. minus sign is used for angle of depression

b. plus sign is used for angle of depression

c. minus sign is used for angle of elevation

d. non of these.

12. In levelling operation

a. when the instrument is being shifted, the staff must not be moved

b. when the staff is being carried forward, the instrument must remain stationary

c. both (a) and (b)

d. neither (a) nor (b).

13. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2

14. Seasoning of timber is essential to remove

a. knots from timber

b. sap from timber

c. twisted fibre from timber

d. roughness of timber.

15. If P is the percentage of water required for normal consistency, water to be added for determination of initial setting time, is

a. 0.75 P

b. 0.80 P

c. 0.85 P

d. 0.90 P.

16. The portion of the brick cut across its width and having its length equal to that of a full brick, is known as

a. queen closer

b. king closer

c. prince closer

d. none of these.

17. Spirit varnish generally consists of

a. alcohol, wax and turpentine

b. pigment and synthetic resin

c. spirit and shellac

d. none of these.

18. Mastic asphalt is normally used for

a. sound insulation

b. water proofing

c. fire proofing

d. none to these.

19. Pick up the incorrect statement applicable to the field test of good cement.

a. The colour of the cement is bluish

b. A handful of cement thrown into a bucket of water should sink immediately

c. By rubbing cement in between fingers, one should feed rough

d. All the above.

20. For a concrete mix 1:3:6 and water cement ratio 0.6 both by weight, the quantity of water required per bag, is

a. 12 kg

b. 14 kg

c. 16 kg

d. 20 kg

21. Water cement ratio is

a. weight of water to that of cement

b. weight of concrete to that of water

c. volume of concrete to that of water

d. both (a) and (b) of the above.

22. Concrete is unsuitable for compaction by a vibrator if it is

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

23. Permissible compressive strength of M 150 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

24. The shear strength of a soil

a. is proportional to the cohesion of the soil

b. is generally known as the strength of the soil

c. is proportional to the tangent of the angle of internal friction

d. all the above.

25. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

26. The neutral stress in a soil mass is

a. force per neutral area

b. force per effective area

c. stess taken up by the pore water

d. stress taken up by solid particles.

27. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

28. Compression of soil occurs rapidly if voids are filled with

a. air

b. water

c. partly with air and partly with water

d. none of these.

29. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

30. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

31. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

32. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

33. A Milestone chart

a. shows the inter-dependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

34. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

35. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

36. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

37. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

38. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

39. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

40. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

41. Third angle projection is the traditional ............. system.

a. American

b. British

c. Indian

d. Nepali

42. Vertical lines are drawn

a. top to bottom

b. bottom to top

c. both (a) and (b) above

d. none of the above

43. The section obtained when the section plane is inclined to the axis of cone and cuts all the generators on either side of the apex, is called .......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

44. In drafting machine, which one does not follow :

a. levelling machine

b. T-squared

c. set square

d. divider

45. The key to profitable operation for project cost control, is :

a. To keep the project cost equal to original cost estimate.

b. To keep the project cost equal to subsequent construction budget.

c. To keep the project cost within the cost budget and knowing when and where job costs are deviating.

d. None of these

46. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

47. Pick up the correct statement from the following:

a. Engineering economy is a collection of mathematical techniques which simplify economic comparisons

b. Engineering economy is a decision assistance tool by which one method will be chosen as the most economically one.

c. For understanding the engineering economy, one should be able to classify the basic terminology and fundamental concepts of economy.

d. All of these.

48. Who is the chairperson of Central Labour Advisory Board?

a. Secretory of ministry of labour

b. Specialist of ministry of labour

c. Director General, Department of labour

d. Labour minister/ satate minister

49. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer

50. For the formation of users committee, members are selected

a. among the people of whole district

b. among the beneficiaries of the project

c. by nomination by the chief District officer of the district

d. none of the above