***Engineer***

1. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

2. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

3. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

4. In a simply supported beam (l + 2a) with equal overhangs (a) and carrying a uniformly distributed load over its entire length, B.M. at the middle point of the beam will be zero if

a. l = 2a

b. l = 4a

c. l < 2a

d. l > a

5. Strain energy of any member may be defined as work done on it

a. to deform it

b. to resist elongation

c. to resist shortening

d. all the above.

6. The curvature of the earth's surface, is taken into account only if the extent of survey is more than

a. 100 sq km

b. 160 sq km

c. 200 sq km

d. 260 sq km.

7. The 'fix' of a plane table station with three known points, is bad if the plane table station lies

a. in the great triangle

b. outside the great triangle

c. on the circumference of the circumscribing circle

d. none of these.

8. For high sensivity of the bubble tube

a. a liquid of low surface tension is used

b. the bubble space should be long

c. the bubble tube should not be too narrow

d. all the above.

9. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

10. Number of links per metre length of a chain are

a. 5

b. 8

c. 10

d. 20

11. The main plate of a transit is divided into 1080 equal divisions. 60 divisions of the vernier coincide exactly with 59 divisions of the main plate. The transit can read angles accurate upto

a. 10"

b. 15"

c. 20"

d. 30"

12. Diopter is the power of a lens having a focal length of

a. 50 cm

b. 75 cm

c. 100 cm

d. 125 cm

13. The steel which contains fissures and cavities, is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

14. A volatile substance added to a paint to make its application easy and smooth, is known as

a. base

b. solvent

c. vehicle

d. none to these.

15. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

16. Depending on the chemical composition and mechanical properties, iron may be classified as

a. cast iron

b. wrought iron

c. steel

d. all the above.

17. Putty is

a. made with finely powdered chalk and linseed oil

b. used for fixing glass panes

c. softened by a solution of pearl ash and quick-lime soaked in water

d. all the above.

18. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

19. Workability of concrete for a given water content is good if the aggregates, are

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregates.

20. Water required per bag of cement, is

a. 14 kg

b. 21 kg

c. 28 kg

d. 35 kg

21. The ratio of the length to breadth of a wooden float, is

a. 5.5

b. 6.5

c. 7.5

d. 8.5

22. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

23. The specifications of a cement bag for storage, are

a. height 18 cm

b. plan area 3000 sq. cm

c. volume 35 litres

d. all the above.

24. A suspended particle falls through a height H cm in water in t minutes. If the viscosity of water is η and specific gravity of the particle is G, the diameter of the particle is (where M is a constant factor)

a. 103M

b. 104M

c. 105M

d. 102M

25. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

26. Water content of a soil sample is the difference of the weight of the given sample at the given temperature and the weight determined after drying it for 24 hours at temperature ranging from

a. 90° to 95°C

b. 95° to 100°C

c. 103° to 105°C

d. 105° to 110°C

27. Number of piles required to support a column, is

a. 1

b. 2

c. 3

d. 4

28. The maximum pressure which a soil can carry without shear failure, is called

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

29. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

30. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

31. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

32. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

33. Pre-tender stage requires

a. selection of site

b. formalization of alignment of work

c. formalization of designs and preparation of estimate

d. all the above.

34. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

35. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4.

36. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

37. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

38. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

39. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

40. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

41. The plane on which perspective is formed is called:

a. horizontal plane

b. picture plane

c. ground plane

d. central plane

42. Axonometric view/drawing is what type of view?

a. one plane

b. two plane

c. three plane

d. six plane

43. The area occupied by Ao (drawing sheet) is ......... times the area occupied by A3 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

44. For drawing large-size circles ........... is attached to the compass (>150mm).

a. straight bar

b. bow compass

c. lengthening bar

d. divider

45. Ratio analysis of a construction firm is used for analysis by :

a. firm's management

b. Banks of the firm

c. financial analysts

d. All of these.

46. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

47. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

48. For the fulfillment of his/her professional duties Engineer should:

a. Paramount the health of the public

b. paramount the safety of the public .

c. welfare of the public

d. all of the above

49. Engineer shall not disclose, without...............confidential information concerning the business affairs or technical processes of any present or former client or employer or public body on which the serve:

a. editing

b. getting paid

c. remuneration

d. consent

50. What percent of interest is taken for the advance amount paid to the contractor by employer?

a. 1% of advance amount

b. 2% of advance amount

c. 3% of advance amount

d. none of the above