***Engineer***

1. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

2. For calculating the permissible stress  is the emprical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

3. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

4. A cantilever carries is uniformly distributed load W over its whole length and a force W acts at its free end upward. The net deflection of the free end will be

a. zero

b.  upward

c.  downward

d. none of these.

5. The point of contraflexture occurs in

a. continuous beams only

b. over hanging beams only

c. all types of beams

d. both (a) and (b).

6. While setting a plane table at a station it was found that the error in centering was 30 cm away from the ray of length 40 m drawn from the station. If the scale of the plan is 1 cm = 2 cm, the displacement of the end of the ray in plan from the true position will be

a. 0.02 cm

b. 0.15 cm

c. 02 cm

d. 0.1 cm

7. To orient a plane table at a point with two inaccessible points, the method generally adopted, is

a. intersection

b. resection

c. radiation

d. two point problem.

8. Diopter is the power of a lens having a focal length of

a. 50 cm

b. 75 cm

c. 100 cm

d. 125 cm

9. Keeping the instrument height as 1.5 m, length of staff 4 m, the slope of the ground as 1 in 10, the sight on the down-slope, must be less than

a. 30 m

b. 25 m

c. 20 m

d. 15 m

10. The distance between terminal points computed from a subsidiary traverse run between them, is generally known, as

a. traverse leg

b. a base

c. traverse base

d. all the above.

11. In an internal focusing telescope

a. the objective is at a fixed distance from the diaphragm

b. the focusing is done by the sliding of a divergent lens.

c. the focusing divergent lens is situated at about the middle of the tube

d. all the above.

12. The most reliable method of plotting a theodolite traverse, is

a. by consecutive co-ordinates of each station

b. by independent co-ordinates of each station

c. by plotting included angles and scaling off each traverse leg

d. by the tangent method of plotting.

13. Putty is

a. made with finely powdered chalk and linseed oil

b. used for fixing glass panes

c. softened by a solution of pearl ash and quick-lime soaked in water

d. all the above.

14. The commonly used drying oil for oil paints, is

a. olive oil

b. linseed oil

c. kerosine oil

d. accetate of lead.

15. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

16. Pick up the plutonic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

17. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

18. Strength of cement concrete primarily depends upon

a. quality of water

b. quantity of aggregate

c. quantity of cement

d. water-cement ratio.

19. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

20. Addition of pozzolana to cement causes

a. loss of heat of hydration

b. reduction in bleeding

c. increase in curing time

d. all the above.

21. The internal dimensions of a ware house are 15 m x 5.6 m, and the maximum height of piles is 2.70 m, the maximum number of bags to be stored in two piles, are

a. 2000 bags

b. 2500 bags

c. 3000 bags

d. 4000 bags

22. Pick up the correct statement from the following:

a. Silica in excess, causes the cement to set slowly

b. Alumina in excess, reduces the strength of the cement

c. Magnesium oxide in excess, remains in free state and makes the cement unsound

d. All the above.

23. Pick up the incorrect statement from the following:

a. The axis of the mould is perpendicular to the end faces

b. The internal surface of the mould is kept clean and free from set cement

c. The mould is in the form of a frustum of hexagonal pyramid

d. None of these.

24. When a cohesionless soil attains quick condition, it looses

a. shear strength

b. bearing capacity

c. both (a) and (b)

d. neither (a) nor (6).

25. The maximum pressure which a soil can carry without shear failure, is called

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

26. Cohesive soils are generally

a. plastic and also compressible

b. elastic and also compressible

c. plastic but incompressible

d. none of these.

27. If voids ratio is 0.67, water content is 0.188 and specific gravity is 2.68, the degree of saturation of the soil, is

a. 25%

b. 40%

c. 60%

d. 75%

28. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

29. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

30. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

31. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

32. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

33. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

34. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

35. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

36. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

37. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

38. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

39. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

40. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

41. The section obtained when the section plane passes through the apex of the cone in such a of it is perpendicular to the base is called .....

a. a triangle

b. a parabola

c. a hyperbola

d. a circle

42. The letter 'H' lebelled in a Drawing Pencil indicates -

a. relative hardness

b. thickness of line

c. both of above

d. none of the above

43. First angle projection method and third angle projection method are used in:

a. axonometric views

b. isometric views

c. orthographic views

d. none of the above

44. Dashed (dotted) line is used for indication

a. edge of the object

b. hidden edge of the object

c. centre line of the object

d. all of the above

45. The ratio obtained by dividing 'quick assests' by current liabilities is called

a. Turnover ratio

b. Acid test ratio

c. Solvency ratio

d. None of these.

46. Pick up the correct statement from the following:

a. Engineering economy is a collection of mathematical techniques which simplify economic comparisons

b. Engineering economy is a decision assistance tool by which one method will be chosen as the most economically one.

c. For understanding the engineering economy, one should be able to classify the basic terminology and fundamental concepts of economy.

d. All of these.

47. Pick up the correct statement from the following:

a. The capital required to get a project started, is called first cost.

b. The costs associated with a new or existing project that remain unaffected by the changes in activity level over the normal range of operation of the project, are called fixed costs.

c. The group of costs that vary proportionately to the changes in the activity level of a new or existing project are called variable costs.

d. All of these

48. The general experience required for nomination of chairperson of NEC is:

a. 10 yrs

b. 15 yrs

c. 20 yrs

d. 25 yrs

49. Engineer shall act in such a manner as to uphold and enhance the honor and dignity of engineering profession and shall act with......for bribery, fraud and corruption.

a. a tolerence

b. a pnnchant

c. a predilection

d. zero tolerance

50. Who is the Chairperson of Construction Material Rate fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer