***Engineer***

1. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

2. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

3. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

4. The property by which a body returns to its original shape after removal of the force, is called

a. plasticity

b. elasticity

c. ductility

d. malleability.

5. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

6. Flint glass

a. has slightly the greater refracting power than crown glass

b. has roughly double refracting power than that of crown

c. and crown glass proportions yield the required focal length and neutralise the dispersion produced by the convex lens at the emergence from the concave

d. all the above.

7. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

8. Closed contours of decreasing values towards their centre, represent

a. a hill

b. a depression

c. a saddle or pass

d. a river bed.

9. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

10. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

11. The method generally preferred to for contouring an undulating area, is

a. chain surveying

b. plane table surveying

c. tacheometrical surveying

d. compass surveying.

12. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

13. Pick up the correct statement from the following:

a. Impurities float on the molten iron as slag

b. The slag contains lime about 45%

c. The molten iron is made to run, in a long channel formed in sand called 'sow'

d. All the above.

14. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

15. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

16. Pick up the correct statement from the following:

a. For mass concrete structures subjected to wetting and drying, the water ratio should be 0.55

b. For thin structures which remain continuously under water, the water-cement ratio by weight should be 0.55

c. For massive concrete structures which remain continuously under water, the water cement ratio by weight should be 0.65

d. All the above.

17. Pick up the correct statement from the following:

a. The silica in excess makes the cement stronger but its setting time also increases

b. The excess amount of alumina weakens the cement

c. The addition of gypsum increases the initial setting time of cement

d. All the above.

18. Pick up the correct statement from the following:

a. The distinct plane of division along which a stone can easily be split, is called natural bed of stone

b. The natural bed of sedimentary rocks is along the planes of stratification

c. The natural bed of igneous rocks is not defined

d. All the above.

19. Pick up the correct statement from the following:

a. The specific gravity of aggregate is important for the determination of the moisture content

b. The absorption and porosity of an aggregate influence the property of the concrete

c. A highly absorptive aggregate reduces the workability of concrete considerably

d. All the above.

20. Pick up the incorrect statement from the following:

a. The process of changing cement paste into hard mass, is known as setting of cement

b. The phenomenon by virtue of which cement does not allow transmission of sound, is known as soundness of cement

c. The heat generated during chemical reaction of cement with water, is known as heat of hydration

d. None of these.

21. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

22. Construction joints are generally provided in concrete

a. retaining walls

b. lining of canals

c. lining of tunnels

d. all the above.

23. For the construction of the retaining structures, the type of concrete mix to be used, is

a. 1 : 2 :4

b. 1 :  : 3

c. 1 : l : 2

d. 1 : 4 : 8.

24. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

25. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

26. The property of a soil which allows it to be deformed rapidly without rupture, elastic rebound and also a volume change, is known

a. porosity

b. plasticity

c. permeability

d. ductility.

27. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

28. The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

29. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

30. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

31. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

32. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

33. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

34. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

35. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

36. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

37. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

38. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

39. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

40. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

41. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

42. Perspective drawing is done for

a. not for civil engineers

b. show details of market products

c. both (a) and (b) of above

d. none of the above

43. The letter 'H' lebelled in a Drawing Pencil indicates -

a. relative hardness

b. thickness of line

c. both of above

d. none of the above

44. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

45. Pick up the correct statement from the following:

a. Uniform series present worth factor =

b. Sinking fund factor =

c. Capital recovery factor =  where letters carry their usual meanings.

d. All of these

46. Pick up the method used for project evaluation and selection in capital budgetting from the following:

a. Internal ratio of return

b. Net present worth

c. Profitability index

d. All the above

47. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

48. What percent of interest is taken for the advance amount paid to the contractor by employer?

a. 1% of advance amount

b. 2% of advance amount

c. 3% of advance amount

d. none of the above

49. The time between ordering of the goods or materials and receiving and placing them into use at the point of need is called

a. order-interval

b. lead time

c. usage time

d. delayed time

50. Engineer shall practices:

a. in favour of client

b. in favour of contractor

c. in favour of him/herself

d. impartial andefficient professional relationship with client, contractor & fellow personnels