***Engineer***

1. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory

2. Pick up the correct statement from the following:

a. The shear stress is zero at the outer fibres and maximum at the neutral axis

b. The bending stress at the outer fibres, is known as principal stress

c. The planes of principal stresses are inclined at 45° to the neutral plane

d. All the above.

3. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

4. In a beam, the neutral plane

a. may be its centre

b. passes through the C.G. of the area of cross-section

c. does not change during deformation

d. none of these.

5. The maximum bending moment due to a moving load on a simply supported beam, occurs

a. at the supports

b. under the load

c. anywhere on the beam

d. none of these.

6. If S is the length of a subchord and R is the radius of simple curve, the angle of deflection between its tangent and sub-chord, in minutes, is equal to

a. 573 R/S

b. 171.9 S/R

c. 1718.9 R/S

d. 1718.9 S/R.

7. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

8. The reduced bearing of a line is N 87° W. Its whole circle bearing is

a. 87°

b. 273°

c. 93°

d. 3°

9. Location of contour gradient for a high way is best set out from

a. ridge down the hill

b. saddle down the hill

c. bottom to the ridge

d. bottom to the saddle.

10. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

11. For high sensivity of the bubble tube

a. a liquid of low surface tension is used

b. the bubble space should be long

c. the bubble tube should not be too narrow

d. all the above.

12. Pick up the correct statement from the following :

a. to locate a gross error in bearing that may exist in controlled theodolite traverse, we may plot the traverse from each end. The traverse station having the same coordinates by each route is the one where the error lies

b. to locate a gross error in bearing, in a controlled traverse, we plot the traverse and the station through which perpendicular to sector of the closing line passes is the station at which the error was made

c. to locate a gross error due to taping in a controlled traverse, we plot the traverse to a convenient scale. The bearing of the closing error will be approximately the same as that of the leg in which the gross error consists

d. all the above.

13. The sequence of refractory materials according to increasing melting points is :

a. Dolomite, Magnesia, Bauxite, Chromite

b. Bauxite, Chromite, Dolomite, Magnesia

c. Magnesia, Bauxite, Dolomite, Chromite.

14. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

15. Plywood is normally available

a. 1 mm thick

b. 2 mm thick

c. 2 to 3 mm thick

d. 3 mm to 4 mm thick.

16. Pick up the correct statement from the following:

a. The bulking of sand is taken into account while volumetric proportioning of the aggregates

b. The dry sand and the sand completely flooded with water, have practically the same volume

c. The expansion and contraction joints are provided if concrete structures exceed 12 m in length

d. All the above.

17. Turpentine oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

18. Pick up the correct statement from the following:

a. Strength and stiffness of corrugated sheets are considerably increased

b. Corrugated sheets are generally gal-venised to protect iron from corrosion by rust

c. Corrugated sheets are generally used on slanting roofs

d. all the above.

19. The surface where two successive placements of concrete meet, is known as

a. Expansion joint

b. Construction joint

c. both (a) and (b)

d. both (6) and (c).

20. Di-calcium silicate (C2S)

a. generates less heat of hydration

b. hardens rapidly

c. provides less ultimate strength to cement

d. has less resistance to sulphate attack.

21. Permissible compressive strength of M 300 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

22. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

23. Workability of concrete is measured by

a. Vicat apparatus test

b. Slump test

c. Minimum void method

d. Talbot Richard test.

24. The property of a soil which allows it to be deformed rapidly without rupture, elastic rebound and also a volume change, is known

a. porosity

b. plasticity

c. permeability

d. ductility.

25. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

26. Hydrometer readings are corrected for:

a. meniscus correction

b. dispersing agent correction

c. meniscus and dispersing agent corrections

d. temperature, meniscus and dispersing agent corrections.

27. If dry density, water density and specific gravity of solids of a given soil sample are 1.6 g/cc, 1.84 g/cc and 2.56 respectively, the porosity of the soil sample, is

a. 0.375

b. 0.370

c. 0.380

d. 0.390

28. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

29. A grillage foundation

a. is provided for heavily loaded isolated columns

b. is treated as spread foundation

c. consists of two sets of perpendicularly placed steel beams

d. all the above.

30. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

31. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

32. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

33. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

34. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

35. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

36. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

37. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

38. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

39. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

40. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

41. The point in space where parallel lines meet are called

a. station point

b. vanishing point

c. origin

d. all of the above

42. The equipment mainly used to transfer the measurements is

a. scale

b. divider

c. compass

d. drawing ping

43. The drawing prepared by a pencil and without use of drawing instrument is called

a. isometric drawing

b. orthographic projection

c. cadastral

d. free hand sketching

44. The solid which is generated by the revolution of a rectangle about one of its sides which remains fixed is called

a. a prism

b. a pyramid

c. a cylinder

d. a cone

45. Pick up the correct statement from the following:

a. The capital required to get a project started is the first cost.

b. The first cost is a single cash flow or a series of cash flows that are made in the beginning of the activity's life span

c. The first cost of purchasing a car is the sum of the down payment, taxes and dealers charges.

d. All of these

46. Pick up the correct statement from the following:

a. Both the numerator and denominator of financial ratios come directly from the balance sheet.

b. Income statement ratios compare one 'flow' item from the income statement to another flow item from the income statement.

c. Income statement ratios compare a flow item from the income statement to another flow item form the income statement

d. All of these

47. he ratio obtained by dividing 'quick assests' by current liabilities is called

a. Turnover ratio

b. Acid test ratio

c. Solvency ratio

d. None of these.

48. The number of members in dispute resolution committee are

a. 2

b. 3

c. 4

d. 5

49. The variation exceeding 15 % of the contract amount is approved by

a. gazetted 2nd class

b. gazetted 1st class

c. deaprtment head

d. none of the above

50. The purpose of bid evaluation is to determine

a. lowest bid

b. responsive bid

c. substantially responsive bid

d. lowest evaluated substantially responsive bid