***Engineer***

1. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula

2. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

3. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

4. The under mentioned type is simple strain

a. compressive strain

b. shear strain

c. volumetric strain

d. all the above.

5. The maximum twisting moment a shaft can resist, is the product of the permissible shear stress and

a. moment of inertia

b. polar moment of inertia

c. polar modulus

d. modulus of rigidly.

6. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

7. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

8. If the radius of a simple curve is R, the length of the chord for calculating offsets by the method of chords produced, should not exceed.

a. R/10

b. R/15

c. R/20

d. R/25.

9. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

10. Surveys which are carried out to depict mountains, rivers, water bodies, wooded areas and other cultural details, are known as

a. city surveys

b. topographical surveys

c. guide map surveys

d. plane surveys.

11. Which one of the following mistakes/errors may be cumulative + or - :

a. bad ranging

b. bad straightening

c. erroneous length of chain

d. sag.

12. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

13. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

14. For filling cracks in masonry structures, the type of bitumen used, is

a. cut-back bitumen

b. bitumen-emulsion

c. blown bitumen

d. plastic bitumen.

15. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

16. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

17. According to ISI, bitumen is classified into

a. 4 grades

b. 6 grades

c. 8 grades.

d. 10 grades.

18. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

19. The mixture of different ingredients of cement, is burnt at

a. 1200°C

b. 1400°C

c. 1600°C

d. 1800°C

20. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

21. The compaction of concrete, improves

a. density

b. strength

c. durability

d. all the above.

22. Curing of pavements, floors, roofs and slabs, is done by

a. ponding method

b. covering surface with bags

c. sprinkling water method

d. shading concrete method.

23. Permissible compressive strength of M 200 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

24. The ultimate Settlement of a soil is directly proportional to:

a. compressive index

b. void ratio

c. both (a) and (b)

d. none of these.

25. Degree of saturation of a natural soil deposit having water content 15%, specific gravity 2.50 and void ratio 0.5, is

a. 50%

b. 60%

c. 75%

d. 80%

26. Pick up the clay soil group which does not swell when wet from the following :

a. Kaolinite group

b. Illite group

c. Vermiculite group

d. Montrorillonite group.

27. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

28. Coulomb's wedge theory assumes that

a. slip surface is the plane which passes through the heel of the wall

b. sliding wedge itself acts as a rigid body and the value of earth pressure is obtained by considering the limiting equilibrium of the wedge

c. position and direction of the resultant earth pressure, are known

d. all the above.

29. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

30. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

31. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

32. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

33. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

34. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

35. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

36. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

37. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

38. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

39. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

40. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

41. In orthographic projection, the line of sight and plane of projection are

a. perpendicular

b. parallel

c. horizontal

d. vertical

42. The polyhedra  having two equal and similar end bases, parallel to each other and are other faces which may be rectangle, or parallelograms is called ......

a. a prism

b. a pyramid

c. a cylinder

d. a cone

43. Engineers prefer to make ......... sketches than to pictorial sketches to clear the shape of an object

a. orthographic

b. perspective

c. auto CAD

d. all of the above

44. Why perspective drawing is done by manufacturer?

a. It is easy to express in the market

b. shape is confirmed

c. exact size is not measured

d. all of the above

45. If a seller recovers his capital along with accumulated compensating interest not in one single lumpsum payment but in periodical equal payments, over time :

a. Capital Recovery Annuity fs availed

b. Present work Annuity is availed

c. Sinking Fund Annuity is availed

d. Sinking Fund Annuity is availed

46. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

47. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

48. Prequilification is not required for any types of firm if the costing to buy an equipment is

a. less than 20 million

b. more than 20 million

c. any amount

d. all of the above

49. Procurement of services means:

a. buying logistics

b. hiring consultant

c. renting house

d. none of the above

50. Who is the chairperson of Central Labour Advisory Board?

a. Secretory of ministry of labour

b. Specialist of ministry of labour

c. Director General, Department of labour

d. Labour minister/ satate minister