***Engineer***

1. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises

2. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

3. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

4. A 8 metre long simply supported rectangular beam which carries a distributed load 45 kg/m. experiences a maximum fibre stress 160 kg/cm2. If the moment of inertia of the beam is 640 cm4, the overall depth of the beam is

a. 10 cm

b. 12 cm

c. 15 cm

d. 16 cm

5. A beam of length L supported on two intermediate rollers carries a uniformly distributed load on its entire length. If sagging B.M. and hogging B.M. of the beam are equal, the length of each overhang, is

a. 0.207 L

b. 0.307 L

c. 0.407 L

d. 0.5 L.

6. Pick up the correct statement from the following

a. the contour lines having the same elevation cannot unite and continue as one line

b. a contour can not end abruptly, but must ultimately close itself not necessarily within the limits of map.

c. the direction of steepest slope at a point on a contour is at right angles to the contour

d. all the above.

7.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

8. If R is the radius of the main curve, θ the angle of deflection, S the shift and L the length of the transition curve, then, total tangent length of the curve, is

a. (R + S) tan θ/2 - L/2

b. (R + S) tan θ/2 + L/2

c. (R - S) tan θ/2 + L/2

d. (R - S) cos θ/2 + L/2

9. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

10. Pick up the correct statement from the following :

a. A level line lies in level surface

b. A horizontal surface is normal to the direction of gravity at only one point

c. A horizontal line is tangential to the level surface

d. All the above.

11. In a lemniscate curve the ratio of the angle between the tangent at the end of the polar ray and the straight, and the angle between the polar ray and the straight, is

a. 3

b. 4

c. 5

d. 3/2

12. The line normal to the plumb line is known as

a. horizontal line

b. level line

c. datum line

d. vertical line.

13. Brass is an alloy of

a. copper and zinc

b. zinc and lead

c. tin and silver

d. zinc and nickel

14. For construction of structures under water, the type of lime used, is

a. hydraulic lime

b. fat lime

c. quick lime

d. pure lime

15. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

16. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

17. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

18. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

19. M 150 grade of concrete approximates

a. 1 : 1 :2 mix

b. 1 : 2 : 4 mix

c. 1 : 1.5 : 3 mix

d. none of these.

20. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

21. Particles of 0.002 mm size are that of

a. clay

b. sand

c. gravel

d. none of these.

22. High temperature

a. increases the strength of concrete

b. decreases the strength of concrete

c. has no effect on the strength of concrete

d. none of these.

23. Workability of concrete for a given water content is good if the aggregates, are

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregates.

24. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

25. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

26. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

27. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

28. The Westergaard analysis is used for

a. sandy soils

b. cohesive soils

c. stratified soils

d. clayey soils.

29. Maximum size of clay particles, is :

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm

d. 0.08 mm

30. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

31. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

32. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

33. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

34. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

35. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

36. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

37. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

38. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

39. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

40. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

41. In third angle projection top view is put

a. above front view

b. below font view

c. side view

d. none of the above

42. Working drawings:

a. must provide complete information to produce the object

b. need not provide complete information

c. should be supplemented by further details TOPS

d. should be supplemented by specifications

43. During drawing stock of T-square is placed

a. adjoining the working edge of the board

b. adjoining the sliding edge of the board

c. surface of the board,

d. none of the above

44. Uses of the T-square, set squares, scale and protractor are combined in the

a. set-square

b. drafting machine

c. compass

d. none of the above

45. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

46. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

47. Pick up the ratio which gives us sufficient information by which to judge the financial condition and performance of the firm, from the following:

a. Financial leverage ratio

b. Activity ratio

c. Portability

d. None of these

48. Code of conduct defines

a. what should be

b. what should not be

c. what had been

d. what will be

49. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

50. What is EPC contract ?

a. Engineering, Planning and costing

b. Evaluation, Planning and consulting

c. Engineering, procurement and Construction

d. none of above