***Engineer***

1. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

2. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero

3. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

4. The equivalent length of a column fixed at one end and free at the other end, is

a. 0.7 l

b. l

c. 2 l

d. 1.5 l.

5. For a given material Young's modulus is 200 GN/m2 and modulus of rigidity is 80 GN/m2. The value of Poisson's ratio is

a. 0.20

b. 0.25

c. 0.30

d. 0.40

6. If L is in kilometres, the curvature correction is

a. 58.2 L2 mm

b. 64.8 L2 mm

c. 74.8 L2 mm

d. 78.4 L2 mm.

7. If the chain line which runs along N-S direction is horizontal and the ground in E-W direction is sloping

a. it is possible to set offsets correctly on east side

b. it is possible to set offsets correctly on east side

c. it is not possible to set offsets correctly on west side

d. it is possible to set offsets correctly on both sides.

8. ABCD is a rectangular plot of land. If the bearing of the side AB is 75°, the bearing of DC is

a. 75°

b. 255°

c. 105°

d. 285°

9. Pick up the correct statement from the following :

a. mistakes arise from inattention, inexperience or carelessness

b. systematic errors persist and have regular effects in the survey performances

c. accidental errors occur inspite of every precaution is taken

d. all the above.

10. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

11. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

12. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

13. A well seasoned timber may contain moisture up to

a. 4 to 6%

b. 6 to 8%

c. 8 to 10%

d. 10 to 12%.

14. Minimum required water cement ratio for a workable concrete, is

a. 0.40

b. 0.50

c. 0.60

d. 1.0.

15. Plastics are compounds of carbon with element

a. hydrogen

b. nitrogen

c. oxygen

d. All of these.

16. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

17. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

18. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

19. A construction joint is provided where

a. bending moment is small

b. shear force is small

c. the member is supported by other member

d. all the above.

20. For the construction of R.C.C. slabs, columns, beams, walls, etc. the grade of concrete mix used, is

a. 1 :  : 3

b. 1 : 2 : 4

c. 1 : 1 : 2

d. 1 : 4 : 8.

21. Shrinkage in concrete can be reduced by using

a. less cement in the concrete

b. proper concrete mix

c. presaturated aggregates

d. all the above.

22. Higher workability of concrete is required if the structure is

a. made with cement concrete

b. thick and reinfored

c. thin and heavily reinforced

d. thick and heavily reinfored.

23. If 1500 g of water is required to have a cement paste 1875 g of normal consistency, the percentage of water is,

a. 25%

b. 30%

c. 35%

d. 40%

24. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

25. The angle of internal friction, is least for

a. angular -grained dense sand

b. round-grained loose sand

c. round-grained loose sand

d. clays.

26. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

27. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

28. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

29. A soil sample has passing 0.075 mm sieve = 60% liquid limit = 65% and plastic limit = 40%. The group index of the soil, is

a. 5

b. 20

c. 40

d. none of these.

30. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

31. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

32. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

33. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

34. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

35. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

36. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

37. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

38. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

39. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

40. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

41. In one plane drawing, which one does not falls under

a. orthographic

b. axonometric

c. oblique

d. dimetric

42. Site plan is a:

a. structural plan

b. architecturel plan

c. section plan

d. none of the above 1

43. The section obtained when the section plane is inclined to the axis of cone, but parallel to one of the generators of a cone is called ......... ......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

44. Free hand sketch is normally used for

a. concept

b. final

c. discussion

d. none of the above

45. Both architect and engineer make use of the cost estimate of the project:

a. for site selection

b. for designing of the project

c. for choosing alternatives

d. All of these

46. Pick up the correct statement from the following:

a. The manifestation of the time value of money is termed as interest.

b. Interest on borrowing = present amount owed - original loan

c. The original investment (or loan) is referred to as principal.

d. All of these.

47. The construction manager uses the estimate of the project

a. to advise the architect/engineer regarding design cost parameter especially in value engineering analysis.

b. to develop bids on the project.

c. to control the project during its construction.

d. All of these

48. Professional code of conduct issued by Nepal Engineering Council has

a. 6 articles

b. 7 articles

c. 8 articles

d. 9 articles

49. Engineer shall not disclose, without...............confidential information concerning the business affairs or technical processes of any present or former client or employer or public body on which the serve:

a. editing

b. getting paid

c. remuneration

d. consent

50. For the registration of general engineer tha amount of fee required is:

a. NRS 2000/-

b. NRS 3000/-

c. NRS 3500/-

d. NRS 4000/-