***Engineer***

1. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

2. Flat spiral springs

a. are supported at outer end

b. are wound by applying a torque

c. are used in clock-work mechanism

d. all the above.

3. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

4. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

5. The slenderness ratio of a vertical column of square cross- section of 10 cm side and 500 cm long, is

a. 17.3

b. 173.2

c. 137.2

d. 13.72

6. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

7. Which of the following introduces an error of about 1 in 1000 if 20 m chain is used

a. one end of the chain 0.9 m off the line

b. one end of chain 0.9 m higher than the other

c. middle of the chain 0.45 m off the line

d. all the above.

8. You have to observe an included angle with better accuracy than what is achievable by a vernier, you will prefer the method of

a. repetition

b. reiteration

c. double observations

d. exactness.

9. Profile levelling is usually done for determining

a. contours of an area

b. capacity of a reservoir

c. elevations along a straight line

d. boundaries of property

10. Setting out a curve by two theodolite method, involves

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. none of these.

11. A back sight

a. is always taken on a point of known elevation or can be computed

b. is added to the known level to obtain the instrument height

c. taken on an inverted staff is treated as negative

d. all the above.

12. During secular variation of magnetic meridian at different places

a. range of oscillations is constant

b. period of oscillation is constant

c. range and period of oscillation both vary

d. period of oscillation only varies.

13. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

14. Pick up the correct statement from the following:

a. Styrene resin is light in weight

b. Styrene resin transmits ultraviolet waves of light

c. Styrene resin is used to manufacture utensils which are unaffected by chemicals

d. All the above.

15. Apiece of sawn timber whose cross-sectional dimensions exceed 5 cm, in one direction and 20 cm in the other direction, is called a

a. cant

b. deal

c. baulk

d. strip.

16. The lime which contains high percentage of calcium oxide, is generally called

a. fat lime

b. rich lime

c. white lime

d. All of these.

17. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

18. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

19. Permissible compressive strength of M 150 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

20. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

21. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

22. Pozzolanic properties exist in

a. fly ash

b. pumicite

c. diatomaceous clay

d. all the above.

23. The datum temperature for maturity by Plowman, is

a. 23°C

b. 0°

c. - 5.6°C

d. - 11.7°C

24. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

25. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

26. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

27. The ratio of the undrained strength in the undrained state to the undrained strength, at the same water content, in the remoulded state, is called the sensivity of the clay. Its value for quick clays is

a. 8

b. 12

c. 16

d. 20

28. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

29. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

30. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

31. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

32. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

33. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

34. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

35. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

36. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

37. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

38. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

39. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

40. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

41. In isometric view a circle is seen as :

a. a parabola

b. a hyperbola

c. a cone

d. an ellipse

42. The set of technical drawings used during the manufacturing phase of product including cvili construction, are known as

a. structural drawing

b. as built drawing

c. structural drawing

d. working drawing

43. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

44. The edge of the board on which T-square is made to slide is called

a. working edge

b. drawing line

c. border line

d. deader line

45. The financial analysis helps to judge:

a. The operational efficiency of the firm

b. The financial position of the firm.

c. Both (a) and (b)

d. Neither (a) nor (b)

46. Ratio analysis of a construction firm is used for analysis by :

a. firm's management

b. Banks of the firm

c. financial analysts

d. All of these.

47. Pick up the correct statement from the following:

a. The ratios which show profitability in relation to sales and those which show profitability in relation to investment, are called profitability ratios.

b. The ratio of gross profit and net sales, is called profitability in relation to sales ratio.

c. The ratio of net profit after taxes to total assests is known as profitability in relation to investment ratio

d. All of these

48. The minimum qulification & experience required for the registration of foreigner engineers is

a. bacheclor in engg. and 5 years experience

b. bachelor in engg. and 10yrs experience

c. master in engg. and 5 yrs experience

d. master in engg. and 10yrs experience

49. Time extension may need to be granted due to

a. causes by force majeure

b. causes by purchaser

c. both a) and b) of above

d. none of the above

50. Any Non-Nepali Engineer desiring to work in Nepal may apply for registration under:

a. Category A

b. Category B

c. Category C

d. Category D