***Engineer***

1.  is the equation of Euler's crippling load if

a. both at the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

2. A rolled steel joist is simply supported at its ends and carries a uniformly distributed load which causes a maximum deflection of 10 mm and slope at the ends of 0.002 radian. The length of the joist will be,

a. 12 m

b. 14 m

c. 16 m

d. 18 m

3. Gradually applied static loads do not change with time their

a. magnitude

b. direction

c. point of application

d. all the above.

4. The ratio of strengths of solid to hollow shafts, both having outside diameter D and hollow having inside diameter D/2, in torsion, is

a. 1/2

b. 1/16

c. 15/16

d. 3/8.

5. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

6. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

7. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

8. Pick up the correct statement from the following :

a. to locate a gross error in bearing that may exist in controlled theodolite traverse, we may plot the traverse from each end. The traverse station having the same coordinates by each route is the one where the error lies

b. to locate a gross error in bearing, in a controlled traverse, we plot the traverse and the station through which perpendicular to sector of the closing line passes is the station at which the error was made

c. to locate a gross error due to taping in a controlled traverse, we plot the traverse to a convenient scale. The bearing of the closing error will be approximately the same as that of the leg in which the gross error consists

d. all the above.

9. The vertical angle between longitudinal axis of a freely suspended magnetic needle and a horizontal line at its pivot, is known

a. declination

b. azimuth

c. dip

d. bearing.

10. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

11. The orthographical projection of a traverse leg upon the reference meridian, is known as

a. departure of leg

b. latitude to the leg

c. co-ordinate of the leg

d. bearing of the leg.

12. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

13. Pick up the correct statement from the following:

a. The plastic sheets are made by the calendering process in which the plastic material is allowed to pass between cylindrical rollers

b. The application of thermo-setting resins on sheets of paper, is called laminating process

c. The plastic articles made by placing raw material in the desired moulds, is known as moulding process

d. All the above.

14. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

15. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

16. The size of modular bricks, is

a. 10 x 10 x 9 cm

b. 19 x 9 x 9 cm

c. 22.5 x 10 x 8.5 cm

d. 22.5 x 8.0 x 9 cm.

17. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

18. Plastic asphalt is

a. used as a water proofing layer over roof

b. a mixture of cement and asphalt

c. a natural asphalt

d. a refinery product.

19. Pozzolanic properties exist in

a. fly ash

b. pumicite

c. diatomaceous clay

d. all the above.

20. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

21. Vicat apparatus is used for

a. consistency test

b. test for setting time

c. test for tensile strength

d. none of these.

22. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

23. Pick up the correct statement from the following:

a. Water and aggregates are measured in litres

b. The finished concrete is measured in cubic metres

c. 20 bags of cement make one tonne

d. All the above.

24. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

25. A triaxial shear test is prefered to direct shear test, because

a. it can be performed under all three driange conditions with complete control

b. precise measurement of pore pressure and change in volume during test, is not posible

c. stress distribution on the failure plane, is non uniform

d. none of these.

26. Number of piles required to support a column, is

a. 1

b. 2

c. 3

d. 4

27. Tergazhi's theory of one dimensional consolidation assumes

a. coefficient of permeability is constant

b. excess pore water drains out only in the vertical direction

c. time lag in consolidation is due entirely to permeability

d. all the above.

28. Pick up the correct statement from the following:

a. Failure plane carries maximum shear stress

b. Failure plane does not carry maximum shear stress

c. Failure plane carries shear stress equal to maximum shear stress

d. None of these.

29. Pick up the clay soil group which does not swell when wet from the following :

a. Kaolinite group

b. Illite group

c. Vermiculite group

d. Montrorillonite group.

30. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

31. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

32. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

33. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

34. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

35. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

36. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

37. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

38. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

39. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

40. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area

41. As per IŞI recommended size of drawing board is/are

a. B0 (1000x 1500m)

b. B1 (700x 1000m)

c. B2 (500x700m)

d. all of the above

42. To draw or measure angles .......... is used.

a. set-square

b. T-square

c. protractor

d. all of the above

43. The lines which are drawn to represent visible edges and surface boundaries of objects are called.

a. outlines

b. principle lines

c. both (a) and (b) of above

d. none of the above

44. Size of A4 size paper is

a. 148 x 210 mm

b. 210 x 297 mm

c. 297 x 210 mm

d. 320 X 460 mm

45. Pick up the correct statement from the following:

a. Both the numerator and denominator of financial ratios come directly from the balance sheet.

b. Income statement ratios compare one 'flow' item from the income statement to another flow item from the income statement.

c. Income statement ratios compare a flow item from the income statement to another flow item form the income statement

d. All of these

46. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

47. The more critical (or severe) test of the firm's liquidity can be judged by :

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

48. To work for survey, design estimate & supervision of a road project, the consultant has to submit

a. technical proposal

b. financial proposal

c. technical and financial proposal

d. none of the above

49. Who is the Chairperson of Construction Material Rate fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

50. What is the provision in Engineer Council Act and Regulation in Nepal for the punishment na person practices Engineering work without registering into the Nepal Engineering Council?

a. Rs 1000 find or imprisonment of 1 month or both

b. Rs 5000 fine

c. Rs10000 fine

d. Rs3000 fine or Imprisonment of 3 month of both