***Engineer***

1. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

2. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

3. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

4. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

5. For a simply supported beam of length L, the bending moment M is described as M = a (x - x3/L2), 0 ≤ x < L; where a is a constant. The shear force will be zero at

a. the supports

b. x = L/2

c. x = L/3

d. x = L/3

6. For a closed traverse the omitted measurements may be calculated

a. bearing of one side only

b. both length and bearing of one side

c. length or bearing of adjacent side

d. all the above.

7. Pick up the correct statement from the following :

a. The directions of plumb lines suspended at different points in a survey are not strictly parallel

b. In surveys of small extent, the effect of curvature may be ignored and the level surface of the earth is assumed as horizontal

c. In surveys of large extent, the effect of curvature of the earth must be considered

d. All the above.

8. While surveying a plot of land by plane tabling, the field observations

a. and plotting proceed simultaneously

b. and plotting do not proceed simultaneously

c. and recorded in field books to be plotted later

d. all the above.

9. While working on a plane table, the correct rule is :

a. Draw continuous lines from all instrument stations

b. Draw short rays sufficient to contain the points sought

c. Intersection should be obtained by actually drawing second rays

d. Take maximum number of sights as possible from each station to distant objects.

10. One of the tacheometric constants is additive, the other constant, is

a. subtractive constant

b. multiplying constant

c. dividing constant

d. indicative constant.

11. If the angular measurements of a traverse are more precise than its linear measurements, balancing of the traverse, is done by

a. Bowditch's rule

b. Transit rule

c. Empirical rule

d. all of the above.

12. The line of sight is kept as high above ground surface as possible to minimise the error in the observed angles due to

a. shimmering

b. horizontal refraction

c. vertical refraction

d. both shimmering and horizontal refraction.

13. Percentage content of silica in window glass, is

a. 40 to 45

b. 50 to 55

c. 60 to 65

d. 70 to 75.

14. Resins are

a. soluble in spirit

b. used in varnishes

c. left behind on evaporation of oil

d. all the above.

15. The fire clay contains pure

a. lime

b. oxide of iron

c. hydrated aluminium silicate

d. magnesium.

16. Cast iron

a. is manufactured in required shapes

b. may contain 2 to 5 per cent of carbon with other impurities

c. is remelted in a cupola furnace

d. all the above.

17. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

18. Bitumen emulsion is

a. a liquid containing bitumen in suspension

b. a paint

c. used as anti-corrosive paint

d. all the above.

19. To ensure constant moisture content in aggregates

a. height of each aggregate pile should not exceed 1.50 m

b. aggregate pile should be left for 24 hours before aggregates are used

c. conical heaps of aggregates should be avoided to prevent moisture variation

d. all the above.

20. Pick up the correct statement from the following:

a. Angular and rough aggregates reduce the workability of the concrete

b. Large size aggregates increase the workability due to lesser surface area

c. The slump of the concrete mix decreases due to an increase in temperature

d. All the above.

21. The dimensions of a 35 litre forma for measuring aggregates by volume, are :

a. length 39 cm, breadth 25 cm, height 32 cm

b. length 27 cm, breadth 27 cm, height 48 cm

c. length 220 cm, breadth 25 cm, height 40 cm

d. none of these.

22. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

23. Pick up the correct statement from the following:

a. According to the petrological characteristics, concrete aggregates are classified as heavy weight, normal weight and light weight

b. According to the shape of the particles, concrete aggregates are classified as rounded irregular, angular and flaky

c. According to the surface texture of the particles, the concrete aggregates are classified as glassy, smooth, granular, rough, crystalline, honey combed and porous

d. All the above.

24. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

25. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the degree of saturation, is

a. 95.4%

b. 95.5%

c. 95.6%

d. 95.7%

26. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

27. The shear strength of a soil

a. is proportional to the cohesion of the soil

b. is generally known as the strength of the soil

c. is proportional to the tangent of the angle of internal friction

d. all the above.

28. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

29. Coulomb's wedge theory assumes that

a. slip surface is the plane which passes through the heel of the wall

b. sliding wedge itself acts as a rigid body and the value of earth pressure is obtained by considering the limiting equilibrium of the wedge

c. position and direction of the resultant earth pressure, are known

d. all the above.

30. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

31. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

32. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

33. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

34. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

35. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

36. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

37. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

38. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

39. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

40. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

41. The section obtained when the section plane is inclined to the axis of cone and cuts all the generators on either side of the apex, is called .......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

42. French curve is used to draw

a. regular curve

b. circle

c. irregular curve

d. none of the above

43. Bisecting a line means, divide a line into..... equal part.

a. two

b. three

c. four

d. none of the above

44. In general structural drawings would typically include following information:

a. north point

b. plan, section and elevation

c. notes on specification

d. all of the above

45. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. The ratio obtained by dividing 'quick assests' by current liabilities is called

a. Turnover ratio

b. Acid test ratio

c. Solvency ratio

d. None of these.

47. Pick up the correct statement from the following:

a. The ability of a company to meet obligations which are likely to mature in short term, is called liquidity.

b. The liquidity ratio may be defined as a relationship of current liabilities and current assests and advances.

c. The liquidity ratios are used to indicate the financial position of the firm.

d. All of these

48. Labour act was introduced /founded in

a. 2038

b. 2048

c. 2043

d. 2053

49. Engineering shall act in such a manner as to uphold and enhance the honor, and dignity of the engineering profession and shall act with ..... for bribery, fraud, and corruption.

a. a tolerance

b. a penchant

c. zero tolerance

d. a predilection

50. As per the prevalling Public Procurement Acts & Regulations of Nepal, provision of price adjustments is normally to inculpated in civil works contract having project duration ?

a. more than 12 months

b. more than 15 months

c. more than 18 months

d. more than 24 months