***Engineer***

1. m1 and m2 are the members of two individual simple trusses of a compound truss. The compound truss will be rigid and determinate if

a. m = m1 + m2

b. m = m1 + m2 + 1

c. m = m1 + m2 + 2

d. m = m1 + m2 + 3.

2. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

3. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

4. The ratio of the tensile stress developed in the wall of a boiler in the circumferential direction to the tensile stress in the axial direction, is

a. 4

b. 3

c. 2

d. 1

5. When loads are applied proportionately to a frame structure containing its members in one plane, the structure is called

a. grid frame

b. plane frame

c. space frame

d. truss frame.

6. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

7. Which one of the following procedures for getting accurate orientation is the most distinctive feature of the art of plane tabling

a. radiation

b. intersection

c. traversing

d. resection.

8. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

9. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

10. The desired sensitivity of a bubble tube with 2 mm divisions is 30". The radius of the bubble tube should be

a. 13.75 m

b. 3.44 m

c. 1375 m

d. none of these.

11. If deflection angles are measured in a closed traverse, the difference between the sum of the right-hand and that of the left hand angles should be equal to

a. 0°

b. 90°

c. 180°

d. 360°

12. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

13. Plastic bitumen is generally used for

a. road pavements

b. expansion joints

c. crack fillings

d. none to these.

14. Pick up the correct statement from the following:

a. Formaldehyde is prepared synthetically from methane

b. The melamine when reacted with formaldehyde forms the melamine-formaldehyde resin

c. The plastics made from melamine formaldehyde resin, are used for electrical insulators

d. All the above.

15. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

16. The thermosetting plastic

a. becomes rigid when moulded at suitable pressure and temperature

b. at 127°C to 177°C permanently set and further application of heat does not soften it

c. charrs at 343°C

d. All the above.

17. The operation of removal of impurities or clay adhering to iron ores, is known as

a. dressing

b. calcination

c. roasting

d. smelting.

18. In paints, the pigment is responsible for

a. colour

b. smoothness

c. glassy face

d. none of these.

19. The size of fine aggregates does not exceed

a. 3.00 mm

b. 3.75 mm

c. 4.75 mm

d. 5.75 mm

20. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

21. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

22. Pick up the correct statement from the following:

a. Concrete using sand stones, cracks due to excessive shrinkage

b. Very hard and close grained crystallined lime stones are suitable aggregates but provide low strength

c. Broken bricks produce a concrete having good fire resisting qualities

d. All the above.

23. Batching error means inaccuracy in the quantity of

a. aggregates

b. cement

c. water

d. all the above.

24. A moist soil sample of volume 60 cc. weighs 108 g and its dried weight is 86.4 g. If its absolute density is 2.52, the degree of saturation is

a. 54%

b. 64%

c. 74%

d. 84%

25. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

26. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

27. The ultimate bearing capacity of a soil, is

a. total load on the bearing area

b. safe load on the bearing area

c. load at which soil fails

d. load at which soil consolidates.

28. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

29. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

30. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

31. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

32. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

33. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

34. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

35. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

36. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

37. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

38. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

39. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

40. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

41. The polyhedra  having two equal and similar end bases, parallel to each other and are other faces which may be rectangle, or parallelograms is called ......

a. a prism

b. a pyramid

c. a cylinder

d. a cone

42. The area occupied by Ao (drawing sheet) is ........... times the area occupied by A1 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

43. In case of unidirectional method of dimensioning, they can be read from

a. bottom side of drawing sheet only

b. right side of drawing sheet only

c. top side of drawing sheet only

d. left side of drawing sheet only

44. In orthographic projection, the line of sight and plane of projection are

a. perpendicular

b. parallel

c. horizontal

d. vertical

45. Both architect and engineer make use of the cost estimate of the project:

a. for site selection

b. for designing of the project

c. for choosing alternatives

d. All of these

46. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

47. Pick up the correct statement from the following:

a. The ability of a company to meet obligations which are likely to mature in short term, is called liquidity.

b. The liquidity ratio may be defined as a relationship of current liabilities and current assests and advances.

c. The liquidity ratios are used to indicate the financial position of the firm.

d. All of these

48. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience

49. For procurement of works up to 2 million, buy which of the following method is to be considered?

a. by sealed tender

b. by direct negotiation

c. by bidding

d. by direct labour or direct negotiation

50. As per the public Procurement Regulation, 2064 of Nepal, what is the maximum age for a technical proposal for consulting services ?

a. 90%

b. 80%

c. 70%

d. 60%