***Sub Engineer***

1. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

2. Keeping the instrument height as 1  m, length of staff 4 m, the up gradient of the ground 1 in 10, the sight on the up slope must be less than

a. 25 cm

b. 20 m

c. 45 m

d. 10 m

3. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

4. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

5. During smelting process, the combination of fuel in the furnace

a. forms carbon dioxide

b. carbon dioxide with carbon forms carbon mono-oxide

c. carbon mono-oxide reacts with Fe2O3 to form iron and liberates CO2

d. all the above.

6. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

7. The specific gravity of marble, is

a. 2.60

b. 2.66

c. 2.72

d. 3.00.

8. Pick up the correct statement from the following:

a. Air bubbles in casting produce a dull sound by tapping their surfaces lightly with a hammer

b. Cupola furnace is used for the manufacture of cast iron

c. Red short iron is of no value for welding purpose

d. All the above.

9. For calculating the permissible stress  is the emprical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

10. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

11. While testing a cast iron beam (2.5 cm x 2.5 cm) in section and a metre long simply supported at the ends failed when a 100 kg weight is applied at the centre. The maximum stress induced is :

a. 960 kg/cm2

b. 980 kg/cm2

c. 1000 kg/cm2

d. 1200 kg/cm2.

12. The maximum resistance against rotation, is offered by the weld at a point

a. most distant

b. least distant

c. at either end

d. centrally located.

13. The discharge through an internal mouth piece is more if its length is

a. <  diameter

b. < diameter

c. ≥ diameter

d. none of these.

14. The discharge formula Q = Cd 2gH x A is used for rectangular

a. small orifices only

b. large orifices only

c. small and large orifices only

d. for all types of orifices.

15. Chezy's formula is used to determine

a. head loss due to friction in pipe

b. velocity of flow in pipe

c. velocity of flow in open channels

d. none of these.

16. For solving the problems is hydraulic engineering, the velocity used is

a. velocity at the centre of pipe section

b. average velocity of flow over a section

c. mean of the velocities at the centre and that along the pipe surface

d. none of these.

17. Through a point in a loaded soil, the principal stress is maximum on

a. minor principal plane

b. intermediate principal plane

c. major principal plane

d. none of these.

18. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

19. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

20. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

21. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

22. The angle of internal friction of soil mass is the angle whose

a. tangent is equal to the rate of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

b. sine is equal to the ratio of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

c. cosine is equal to the ratio of the maximum resistance sliding on any internal inclined plane to the normal pressure acting on the plane

d. none of these.

23. The thickness of base slab of a retaining wall generally provided, is

a. one-third of the width of the stem at the bottom

b. one fourth of the width of the steam at the bottom

c. width of the stem at the bottom

d. twice the width of the steam at the bottom.

24. The spacing of transverse reinforcement of column is decided by the following consideration.

a. The least lateral dimension of the column

b. Sixteen times the diameter of the smallest longitudinal reinforcing rods in the column

c. Forty-eight times the diameter of transverse reinforcement

d. All the above.

25. The stone whose crushing strength is least, is

a. chalk

b. marble

c. slate

d. sand stone.

26. For plastering the exposed brick walls, the cement sand mortar should be

a. 1:3

b. 1:4

c. 1:6

d. 1:8.

27. An ordinary Portland cement when tested for its fineness, should not leave any residue on I.S. seive No. 9, more than

a. 10%

b. 15%

c. 20%

d. 25%

28. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

29. A circular gravity aqueduct is not generally preferred to because of

a. its maximum hydraulic mean depth

b. maximum area per unit of wetted perimeter

c. minimum cost of construction

d. its proper support on the ground.

30. Air valves are generally provided in pressure pipes of water supply

a. at pipe junctions

b. at summits

c. at low points

d. near service pipes.

31. The sewerage system consists of

a. lateral sewer

b. branch sewer

c. main sewer

d. all of these.

32. Pick up the correct statement from the following :

a. In acidic sewage, the pH value is less than 7

b. In alkaline sewage, the pH value is more than 7

c. Fresh sewage is generally alkaline

d. All the above.

33. For smooth entry of water in a canal, the angle between head regulator and water is generally kept

a. 80°

b. 90°

c. 110°

d. 120°

34. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

35. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

36. The crest level of a canal diversion head work, depends upon

a. discharge perimeters

b. pond level

c. all the above

d. none these.

37. On concrete roads, the camber generally provided, is

a. 1 in 30 to 1 in 48

b. 1 in 36 to 1 in 48

c. 1 in 48 to 1 in 60

d. 1 in 60 to 1 in 72

38. At a road junction, 16 cross conflict points are severe, if

a. both are one-way roads

b. both are two-way roads

c. one is two-way road and other is one-way road

d. none of these.

39. Roughness index of roads, is expressed as

a. size of the stone on the pavement

b. number of patches on the pavement

c. cumulative deformation of surface per horizontal distance

d. type of the road surface.

40. To indicate proper control of consistency of a freshly mixed concrete for pavement construction, the slump should be between

a. 4 to 6 cm

b. 5 to 7 cm

c. 7 to 10 cm

d. 10 to 12 cm

41. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

42. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

43. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

44. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

45. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

46. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

47. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

48. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

49. Total correction for elevation, temperature and gradient for a runway should not be more than

a. 20%

b. 25%

c. 30%

d. 35%

50. A gradient of + 0.08% is followed by a gradient of - 0.07%. If the permissible rate of change of grade is 0.003 per 30 metres, the length of the transition curve, is

a. 150 m

b. 140 m

c. 160 m

d. 175 m