***Sub Engineer***

1. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

2. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

3. The bearing of C from A is N 30° E and from B, 50 metres east of A, is N 60° W. The departure of C from A is

a. 50 m

b. 50 3 m

c. 25 3 m

d. 25 m

4. While measuring a chain line between two stations A and B intervened by a raised ground

a. vision gets obstructed

b. chaining gets obstructed

c. both vision and chaining get obstructed

d. all the above.

5. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

6. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

7. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

8. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

9. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

10. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

11. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

12. Shear force for a cantilever carrying a uniformly distributed load over its length, is

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

13. The flow is called rotational if its velocity normal to the plane of area is equal to

a. angular velocity vector

b. twice the angular velocity vector

c. thrice the angular velocity vector

d. none of these.

14. In a two dimensional flow if the components of the velocity are u = ax ; v = by, the point where no motion occurs, is known as

a. neutral point

b. stagnation point

c. stationary point

d. none of these.

15. The acceleration f required to accelerate a rectangular tank containing water horizontally so that the slope of its free surface is 45°, is

a. g

b. 2 g

c. 2.5 g

d. 3 g.

16. Water belongs to

a. Newtonian fluids

b. non-Newtonian fluids

c. compressible fluid

d. none of these.

17. Flow net is used for the determination of

a. hydrostatic pressure

b. seepage pressure

c. exit gradient

d. all the above.

18. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

19. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

20. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

21. As the percentage of steel increases

a. depth of neutral axis increases

b. lever arm increases

c. lever arm decreases

d. none of these.

22. If the average bending stress is 6 kg/cm2 for M 150 grade concrete, the length of embedment of a bar of diameter d according to I.S. 456 specifications, is

a. 38 d

b. 48 d

c. 58 d

d. 95 d

23. A reinforced concrete cantilever beam is 3.6 m long, 25 cm wide and has its lever arm 40 cm. It carries a load of 1200 kg at its free end and vertical stirrups can carry 1800 kg. Assuming concrete to carry one-third of the diagonal tension and ignoring the weight of the beam, the number of shear stirrups required, is

a. 35

b. 40

c. 45

d. 50

24. A continuous beam shall be deemed to be a deep beam if the ratio of effective span to overall depth, is

a. 2.5

b. 2.0

c. less than 2

d. less than 2.5

25. The stone whose crushing strength is least, is

a. chalk

b. marble

c. slate

d. sand stone.

26. Under reamed piles are generally used for

a. factory buildinp

b. transmission linetowers

c. tall structures.

d. All the above.

27. Water cement ratio is generally expressed in volume of water required per

a. 20 kg

b. 30 kg

c. 40 kg

d. 50 kg

28. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

29. Pick up the correct statement from the following :

a. Detention period for plain sedimentation tanks ranges between 4 to 8 hours

b. Detention period for sedimentation tanks, using coagulants usually ranges between 2 to 4 hours

c. The horizontally flow velocity in sedimentation tanks, is generally limited to 0.3 m/minute

d. All the above.

30. To detect the turbidity of the order of 0 to 1000 P.P.M. the instrument used is

a. turbidimeter

b. Jackson turbidimeter

c. Baylis turbidimeter

d. Hallige turbidimeter.

31. The coagulant which is generally not used for treating the sewage, is

a. alum

b. ferric chloride

c. ferric sulphate

d. chlorinated copperas.

32. The gradient of sewers depends upon

a. velocity of flow

b. diameter of the sewer

c. discharge

d. all the above.

33. A minimum of 90 cm free board is provided if the discharge in the canal is between

a. 30 to 33 cumecs

b. 30 to 60 cumecs

c. Over 60 cumecs

d. Over 100 cumecs

34. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

35. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

36. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

37. Roadway width for a National highways and State highways (two-lanes) is

a. 12 m

b. 9 m

c. 9.5 m

d. 15 m

38. In scanty rainfall regions, the camber provided will be

a. nil

b. flatter

c. steeper

d. None of these.

39. Super-elevation on roads in snow bound areas, should generally not exceed

a. 12%

b. 10%

c. 7%

d. 6%

40. Pick up the correct statement from the following:

a. Seasonal cycle of traffic volume during April and November, is usually near the annual average

b. Mid-winter seasonal cycle of traffic is least

c. Mid-summer seasonal cycle of traffic is highest

d. All the above.

41. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

42. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

43. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

44. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

45. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

46. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

47. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

48. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

49. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.