***Sub Engineer***

1. In tangential tacheometry, an ordinary level staff is used

a. leaning towards the instrument for inclined sights upward

b. leaning away from the instrument for inclined sights downwards

c. vertical in all cases

d. none of these.

2. Number of links per metre length of a chain are

a. 5

b. 8

c. 10

d. 20

3. The probable error of the adjusted bearing at the middle is

a.  rn

b.  rn

c.  rn

d.  rn.

4. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

5. Plywood is made from

a. common timber

b. bamboo fibre

c. teak wood only

d. asbestos sheets.

6. Seasoning is

a. a process of removing sap

b. creosoting

c. painting with sodium silicate

d. coating with tar.

7. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

8. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

9. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

10. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

11. Ties are load carrying members of a frame, which are subjected to

a. transverse loads

b. axial tension loads

c. axial compressive loads

d. torsional loads.

12. An arch may be subjected to

a. bending moment and shear force

b. bending moment and axial force

c. shear force and thrust

d. thrust, shear force and bending moment.

13. If the Mach number for a fluid flow is less than 1, the flow is

a. sonic

b. supersonic

c. sub-sonic

d. none of these.

14. A syphon is used

a. to connect water reservoirs at different levels intervened by a hill

b. to supply water to a town from higher level to lower level

c. to fill up a tank with water at higher level from a lower level

d. none of these.

15. For exerting a pressure of 4.8 kg/cm2, the depth of oil (specific gravity 0.8), should be

a. 41 cm

b. 56 cm

c. 60 cm

d. 76 cm.

16. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

17. If G is specific gravity of sand particles, e is porosity, the critically hydraulic gradient

a. ic =

b. ic =

c. ic =

d. ic =

18. If water content of a soil is 40%, G is 2.70 and void ratio is 1.35, the degree of saturation is

a. 75%

b. 80%

c. 85%

d. 90%

19. The fluid generally used for grouting is

a. clay suspension

b. sodium silicate

c. bitumen emulsion

d. all the above.

20. Terzaghi's analysis assumes :

a. soil is homogeneous and isotropic

b. elastic zone has straight boundaries inclined at ψ = φ to the horizontal and plastic zones fully developed

c. failure zones do not extend above the horizontal plane through the base of the footing

d. all the above.

21. Pick up the true statement from the following:

a. In the absence of beams, it is easier to install piping

b. In the absence of beams, it is easier to paint

c. A flat slab is capable to withstand concentrated loads

d. All the above.

22. The amount of reinformcement for main bars in a slab, is based upon

a. minimum bending moment

b. maximum bending moment

c. maximum shear force

d. minimum shear force.

23. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

24. A T-beam behaves as a rectangular beam of a width equal to its flange if its neutral axis

a. remains within the flange

b. remains below the slab

c. coincides the geometrical centre of the beam

d. none of these.

25. The maximum permissible differential settlement, in case of foundations in clayey soil, is usually limited to

a. 20 mm

b. 30 mm

c. 40 mm

d. 50 mm.

26. The formWork including the props can be removed from beams, only after

a. 3 day

b. 7 days

c. 14 days

d. 21 days.

27. Saw dust can be rendered chemically inert by boiling it in water containing

a. ferrous sulphate

b. potassium chloride

c. ammonia

d. nitric acid

28. To prevent segregation, the maximum height for placing concrete, is

a. 100 cm

b. 125 cm

c. 150 cm

d. 200 cm

29. Pick up the correct statement from the following :

a. Even if ports are closed, the wet intake tower will have water filled up to reservoir level

b. The dry intake towers are generally constructed with heavier construction to resist buoyant forces

c. No buoyant force acts on wet intake towers

d. All the above.

30. A slow sand filter is cleaned if its filter head is higher than

a. 20 cm to 40 cm

b. 40 cm to 70 cm

c. 70 cm to 120 cm

d. 120 cm to 150 cm.

31. Bio-chemical oxygen demand (BOD) for the first 20 days in generally referred to

a. initial demand

b. first stage demand

c. carbonaceous demand

d. all of these.

32. Pick up the correct statement from the following :

a. Rational method of estimating peak run off, may be used precisely for areas less than 50 hectares

b. The period after which the entire area starts contributing to the run off, is called the time of concentration

c. The time required by water to reach the outlet of a catchment from the most remote point, is called time of concentration

d. All the above.

33. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

34. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

35. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

36. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

37. The width of road pavements, depends upon

a. width of traffic lane

b. number of traffic

c. width of median strip

d. all the above.

38. Passing zones are generally not provided on

a. summit curves

b. horizontal curves

c. two lane highways

d. all the above.

39. The top height of a route marker above crown level is

a. 1.50 m

b. 1.75 m

c. 2.00 m

d. 2.25 m

40. For clear distinct vision, images of obstructions should fall on the retina with a cone of

a. 3°

b. 4°

c. 5°

d. 6°

41. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

42. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

43. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

44. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

45. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

46. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

47. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

48. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

49. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m

50. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.