***Sub Engineer***

1. In a telescope the object glass of focal length 14 cm, is located at 20 cm from the diaphragm. The focussing lens is midway between them when a staff 16.50 m away is focussed. The focal length of the focussing lens, is

a. 5.24 cm

b. 6.24 cm

c. 7.24 cm

d. 8.24 cm

2. A relatively fixed point of known elevation above datum, is called

a. bench mark

b. datum point

c. reduced level

d. reference point.

3. Pick up the correct statement from the following :

a. it is difficult to eliminate an error completely at first trial

b. instability of the instrument makes it almost impossible to adjust it satisfactorily

c. adjustment screws must be left bearing firmly but should never be forced

d. all the above.

4. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

5. Pick up the correct statement from the following:

a. The average crushing strength of hand moulded bricks is 6000 t/m2

b. The average tensile strength of hand moulded brick is 200 t/m2

c. The average shearing strength of hand moulded brick is 600 t/m2

d. All the above.

6. A volatile substance added to a paint to make its application easy and smooth, is known as

a. base

b. solvent

c. vehicle

d. none to these.

7. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

8. The main ingredient of a good quality brick earth, is

a. magnesia

b. lime

c. silica

d. alumina.

9. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

10. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3 M

d. depth d ∝

11. In a three hinged arch, the third hinge is generally kept at

a. crown of the arch

b. midpoint of the crown and left support hinge

c. midpoint of the crown and right support hinge

d. none of these.

12. A cylinder is said to be thin if the ratio of its thickness and diameter, is less than

a. 1/20

b. 1/15

c. 1/10

d. 1/5

13. Bernoulli's equation assumes that

a. fluid is homogeneous

b. flow is steady

c. flow is along the stream line

d. all the above.

14. Unit of kinematic viscosity is

a. m2/sec

b. Newton sec/m2

c. Newton sec/m3

d. Kg sec/m2.

15. Flow in pipes is laminar if Reynold number is

a. less than 2100

b. more than 3000

c. between 2100 and 3000

d. none of these.

16. A cylindrical vessel 40 cm high is revolved about its vertical axis so that the water touches the bottom when it just spills out. If the radius of the cylinder is 5 cm, the angular velocity of rotation, is

a. 2 rad/sec

b. 3 rad/sec

c. 4 rad/sec

d. 5 rad/sec

17. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

18. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

19. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

20. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

21. The thickness of the topping of a ribbed slab, varies between

a. 5 cm to 8 cm

b. 8 cm to 10 cm

c. 12 cm to 15 cm

d. 12 cm to 18 cm

22. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

23. In a singly reinforced beam, if the permissible stress in concrete reaches earlier than that in steel, the beam section is called

a. under-reinforced section

b. over reinforced section

c. economic section

d. critical section.

24. For stairs spanning horizontally, the minimum waist provided is

a. 6 cm

b. 8 cm

c. 10 cm

d. 12 cm.

25. A floor constructed with 3 mm marble chips, is known

a. mosaic floor

b. terrazo floor

c. chips floor

d. marble floor.

26. Gravels

a. vary in size between 2 to 20 mm

b. never swell when they come into contact with water

c. seldom shrink when dried

d. all the above.

27. For construction of structures in sea water, the cement generally preferred to, is

a. Portland-pozzolana cement

b. quick setting cement

c. low heat Portland cement

d. rapid hardening cement

28. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

29. Rapid gravity filters

a. make use of coarser sand with effective size as 0.5 mm

b. yield as high as 30 times the yield of slow sand filters

c. are fed with coagulation treated water

d. all the above.

30. Pick up the correct statement from the following :

a. The pH value of neutral water is 7

b. The maximum acidity is obtained when pH value is zero

c. The maximum alkalinity is obtained when pH value is 14.

d. All the above.

31. The minimum diameter of sewer to be adopted is

a. 10 cm

b. 12.5 cm

c. 15 cm

d. 25 cm.

32. For estimating the peak run off the rational formula Q = 0.0278 KpA was evolved by

a. Kinchling

b. Lloyd Davis

c. Frubling

d. all the above.

33. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

34. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

35. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

36. In a Sarda type fall, the rectangular crest, may be used for discharge upto

a. 10 cumecs

b. 14 cumecs

c. 20 cumecs

d. 25 cumecs.

37. For the administration of road transport, a Motor Vehicle Act was enacted in

a. 1934

b. 1939

c. 1947

d. 1950

38. Increase in traffic volume, due to increase in transport vehicles, is known as

a. normal traffic growth

b. generated traffic growth

c. current traffic

d. none of these.

39. At a road junction, 7 cross conflict points are severe if

a. both are one-way roads

b. both are two-way roads

c. one is two-way road and other is one-way road

d. none of these.

40. The shape of a vertical curve, is

a. parabolic

b. elliptical

c. circular

d. spiral

41. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

44. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

45. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

46. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

47. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

48. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

49. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.

50. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.