***Sub Engineer***

1. In a perfect prismatic compass

a. ends of the needle and pivot are in same vertical and horizontal planes

b. pivot is vertically over the centre of the graduated circle

c. needle is always kept sensitive

d. all the above.

2. The diaphragm of a stadia theodolite is fitted with two additional

a. horizontal hairs

b. vertical hairs

c. horizontal and two vertical hairs

d. none of these.

3. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

4. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

5. The colour of statuary marble used for sculptor's work, is

a. blue

b. white

c. green

d. yellow

6. Apiece of sawn timber whose cross-sectional dimensions exceed 5 cm, in one direction and 20 cm in the other direction, is called a

a. cant

b. deal

c. baulk

d. strip.

7. Mastic asphalt is generally used for

a. damp proof course

b. water proof layer

c. partition walls

d. both (a) and (b).

8. For light and ornamental casting, the most unsuitable pig iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

9. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

10. In plastic analysis, the shape factor for a triangular section, is

a. 1.5

b. 1.34

c. 2.34

d. 2.5

11. The law which states, "within elastic limits strain produced is proportional to the stress producing it", is known as

a. Stress law

b. Hooke's law

c. Poisson's law

d. none of these.

12. The principal stresses at a point are 100, 100 and-200 kgf/cm2, the octo hedral shear stress at the point is :

a. 100 2 kg/cm2

b. 200 2 kg/cm2

c. 300 2 kg/cm2

d. 400 2 kg/cm2

13. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

14. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

15. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

16. In a fluid flow a particle may posses

a. kinetic energy

b. pressure energy

c. initial energy

d. all the above.

17. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

18. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

19. The general relationship between specific gravity (G), weight of water (γω), degree of saturation (Sr), void ratio (e) and bulk density (γ), is

a. γ =

b. γ =

c. γ =

d. γ =

20. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

21. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

22. The diameter of main bars in R.C.C. columns, shall not be less than

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

23. An R.C.C. beam of 6 m span is 30 cm wide and has a lever arm of 55 cm. If it carries a U.D.L. of 12 t per m and allowable shear stress is 5 kg/cm2, the beam

a. is safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined bars

d. needs revision of section

24. If the shear stress in a R.C.C. beam is

a. equal or less than 5 kg/cm2, no shear reinforcement is provided

b. greater than 4 kg/cm2, but less than 20 kg/cm2, shear reinforcement is provided

c. greater than 20 kg/cm2, the size of the section is changed

d. all the above.

25. The service area in a building means the area occupied by

a. stairs

b. toilets

c. light and shafts

d. all the above.

26. Pick up the incorrect statement from the following :

a. Cement is added to lime mortar to increase its hydraulic properties only

b. Lime surkhi mortar is used for pointing the walls

c. Lime should be slaked before preparing lime mortar

d. High early strength concrete is generally used in cold weather.

27. Log Angles machine is used to test the aggregate for

a. impact value

b. abrasion resistance

c. water absorption

d. none of these.

28. Pick up the incorrect statement from the following:

a. The axis of the mould is perpendicular to the end faces

b. The internal surface of the mould is kept clean and free from set cement

c. The mould is in the form of a frustum of hexagonal pyramid

d. None of these.

29. A strainer type well sunk through three pervious layers intervened by three impervious aquicludes, draws water from

a. top most pervious layer

b. central pervious layer

c. lowest pervious layer

d. all the pervious layers.

30. A pressure conduit carrying water beneath a stream or a canal, is known as

a. sag

b. depressed pipe

c. inverted syphon

d. all the above.

31. The algae dies out in the zone of

a. degradation

b. active decomposition

c. recovery

d. cleaner water.

32. The gradient of sewers depends upon

a. velocity of flow

b. diameter of the sewer

c. discharge

d. all the above.

33. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

34. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

35. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

36. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

37. Minimum thickness of a layer of fine sand required to cut off capillary rise of water completely, should be

a. 52 cm

b. 64 cm

c. 76 cm

d. 80 cm

38. Minimum stopping distance for moving vehicles on road with a design speed of 80 km/hour, is

a. 100 m

b. 120 m

c. 150 m

d. 200 m

39. A gradient along which the vehicle does not require any tractive effort to maintain a specified speed, is known as

a. ruling gradient

b. pushing gradient

c. floating gradient

d. minimum gradient

40. An exceptional grade may be provided upto 1 in 12 along hill roads, if the length does not exceed

a. 60 m per km

b. 75 m per km

c. 90 m per km

d. 100 m per km

41. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

42. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

43. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

44. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

45. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

46. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

47. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

48. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

49. According to I.C.A.O. all markings on the runways are painted white and on taxiways

a. black

b. red

c. yellow

d. green

50. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.