***Sub Engineer***

1. While working on a plane table, the correct rule is :

a. Draw continuous lines from all instrument stations

b. Draw short rays sufficient to contain the points sought

c. Intersection should be obtained by actually drawing second rays

d. Take maximum number of sights as possible from each station to distant objects.

2. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

3. If the length of a chain line along a slope of θ° is l, the required slope correction is

a. 2l cos2 θ/2

b. 2l sin2 θ/2

c. l tan2 θ/2

d. l cos2 θ/2.

4. The diaphragm of a stadia theodolite is fitted with two additional

a. horizontal hairs

b. vertical hairs

c. horizontal and two vertical hairs

d. none of these.

5. Portland cement manufactured from pure white chalk and clay but free from iron-oxide, is known as

a. quick setting cement

b. rapid hardening cement

c. white cement

d. low heat Portland cement.

6. Gniess is obtained from

a. igneous rocks

b. metamorphic rocks

c. sedimentary rocks

d. sedimentary metamorphic rocks.

7. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

8. Bitumen in

a. solid state, is called asphalt

b. semi fluid state, is called mineral tar

c. fluid state, is called petroleum

d. all the above.

9. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

10. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

11. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

12. Stress in a beam due to simple bending, is

a. directly proportional

b. inversely proportional

c. curvilinearly related

d. none of these.

13. From a nozzle exposed to atmosphere, the liquid jet traverses

a. a straight line

b. a circular path

c. an elliptical path

d. a parabolic path.

14. Water belongs to

a. Newtonian fluids

b. non-Newtonian fluids

c. compressible fluid

d. none of these.

15. Hydraulic coefficient of an orifice means the coefficient of

a. velocity

b. contraction

c. resistance

d. all the above.

16. Discharge with velocity of approach, over a rectangular weir is

a.  Cd x L x 2g [(H + ha)3/2 + ha3/2]

b.  Cd x L x 2g [(H + ha)3/2 - ha3/2]

c.  Cd x L x 2g [(H - ha)3/2 + ha3/2]

d.  Cd x L x 2g [(H - ha)3/2 - ha3/2]

17. Water content of a soil sample is the difference of the weight of the given sample at the given temperature and the weight determined after drying it for 24 hours at temperature ranging from

a. 90° to 95°C

b. 95° to 100°C

c. 103° to 105°C

d. 105° to 110°C

18. Compression of soil occurs rapidly if voids are filled with

a. air

b. water

c. partly with air and partly with water

d. none of these.

19. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

20. The total weight of a pycnometer with water and oven dried soil 20 (g) is 1600 g. The pycnometer filled with water alone weighs 1500 g. The specific gravity of the soil, is

a. 1.0

b. 1.5

c. 2.0

d. 2.5

21. Pick up the incorrect statement from the following:

a. In the stem of a retaining wall, reinforcement is provided near the earth side

b. In the toe slab of a retaining wall, rein forcement is provided at the bottom of the slab

c. In the heel slab of a retaining wall, rein forcement is provided at the top of the slab

d. None of these.

22. The steel generally used in R.C.C. work, is

a. stainless

b. mildsteel

c. high carbon steel

d. high tension steel.

23. A column is regarded as long column if the ratio of its effective length and lateral dimension, exceeds

a. 15

b. 20

c. 25

d. 30

24. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

25. During percussion drilling

a. ground water observations are hindered due to entry of the slurry in the soil below the bottom of the hole

b. caving or mixing of strata are caused in soft soils or cohesionless soils

c. the soil to a considerable depth below the bottom of the hole gets disturbed

d. all the above.

26. The brick laid with its length parallel to the face of a wall, is a known as

a. header

b. stretcher

c. closer

d. none of these.

27. Ordinary Portland cement is manufactured from

a. lime stone and clay

b. gypsum and lime

c. pozzolana

d. lime, pozzolana and clay.

28. Pick up the incorrect statement from the following:

a. A rich mix of concrete possesses higher strength than that a lean mix of desired workability with excessive equantity of water

b. The strength of concrete decreases as the water cement ratio increases

c. If the water cement ratio is less than 0.45, the concrete is not workable and causes honey-combed structure

d. Good compaction by mechanical vibrtions, increases the strength of concrete

29. Rapid gravity filter can only remove turbidity of water upto

a. 25 to 30 gm/litre

b. 30 to 35 cm/litre

c. 35 to 40 gm/litre

d. 50 gm/litre.

30. Demand for public uses in a city, does not include water required for

a. watering of public parks

b. watering of public gardens

c. sprickling on roads

d. drinking purposes.

31. In case of Imhoff tanks,

a. detention period is 2 hours

b. the velocity of flow is restricted to 0.30 m/minute

c. surface loading is limited to 30, 000 litres/m of plan area per day

d. All the above.

32. The layers of vegetable wastes and night soil alternatively piled above the ground to form a mound, is called

a. a heap

b. plateu

c. windrow

d. none of these.

33. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

34. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

35. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

36. The crest level of a canal diversion head work, depends upon

a. discharge perimeters

b. pond level

c. all the above

d. none these.

37. Minimum stopping distance for moving vehicles on road with a design speed of 80 km/hour, is

a. 100 m

b. 120 m

c. 150 m

d. 200 m

38. Interior thickness of concrete road slab for design wheel load 6300 kg and permissible flexural stress 21 kg/cm2, is

a. 25.5 cm

b. 34.0 cm

c. 42.5 cm

d. 50.0 cm

39. The minimum width of the pavement of a National Highway should be

a. 5.7 m

b. 6.7 m

c. 7.7 m

d. 8.0 m

40. Shoulders for high traffic volume roads, should

a. be stable throughout the year to be used by vehicles in the case of emergency

b. support and protect the ends of carriage ways

c. not allow entrance of water to sub-grade

d. all the above.

41. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

42. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

43. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

44. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

45. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

46. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

47. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

48. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

49. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.