***Sub Engineer***

1. The slope correction may be ignored if

a. the slope of the ground is less than 3°

b. to slope of the ground is say 1 in 19

c. both (a) and (b)

d. neither (a) nor (b)

2. An internal focussing type surveying telescope, may be focussed by the movement of

a. objective glass of the telescope

b. convex-lens in the telescope

c. concave lens in the telescope

d. plano-convex lens in the telescope.

3. Diopter is the power of a lens having a focal length of

a. 50 cm

b. 75 cm

c. 100 cm

d. 125 cm

4. When the bubble of the level tube of a level, remains central

a. line of sight is horizontal

b. axis of the telescope is horizontal

c. line of collimation is horizontal

d. geometrical axis of the telescope is horizontal.

5. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

6. Pick up the hypabyssal rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

7. Pick up the correct statement from the following:

a. Plastics have generally low melting point

b. The coefficient of thermal expansion of plastics is about three times than that of steel

c. The acoustical boards prepared by impregnating fibre-glass with phenolic resins has absorption coefficient of about 0.67

d. All the above.

8. The material generally not used as extender in paints, is

a. powdered silica

b. gypsum

c. talc

d. zinc white.

9. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

10.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. Pick up the correct assumption of the theory of simple bending

a. Transverse section of a beam remains plane before and after bending

b. The material of the beam is homogeneous and isotropic

c. The resultant pull or thrust on transverse section of a beam is zero

d. All the above.

12. The deflection of any rectangular beam simply supported, is

a. inversely proportional to its width

b. inversely proportional to the cube of its depth

c. directly proportional to the cube of its length

d. none of these.

13. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

14. When no air is left below the nappe and water stream adheres to the down stream face of the weir, it is known as

a. free nappe

b. depressed nappe

c. clinging nappe

d. none of these.

15. The following is not a laminar flow

a. Flow in beds in ground water

b. Rise of water in plants through roots

c. Flow of blood in the arteries of human body

d. Flow in water pipe lines.

16. The formula Q = m x L x 2g x H3/2 where  was suggested by :

a. Bazin

b. Francis

c. Cipolletti

d. None of these.

17. A stratum of clay 2 m thick will get consolidated 80% in 10 years. For the 80% consolidation of 8 m thick stratum of the same clay, the time required is

a. 120 years

b. 140 years

c. 160 years

d. 180 years.

18. Pick up the correct statement from the following:

a. If the ratio of depth to width is more than 2, it is deep foundation

b. If the ratio of the length to width is between 1 and 2, it is spread foundation

c. If the length is large as compared to width, it is a strip foundation

d. All the above.

19. On wetting, cohesive soils,

a. loose permeability

b. gain shear strength

c. loose elasticity

d. decrease their shear strength.

20. The compression resulting from a long term static load and consequent escape of pore water, is known as

a. compaction

b. consolidation

c. swelling

d. none of these.

21. According to I.S.: 456, 1978 the thickness of reinforced concrete footing on piles at its edges, is kept less than

a. 30 cm

b. 40 cm

c. 50 cm

d. 75 cm

22. The system in which high tensile alloy steel bars (silica manganese steel) are used as prestressing tendons, is known as

a. Freyssinet system

b. Magnel-Blaton system

c. C.C.L. standard system

d. Lee-McCall system.

23. In a simply supported slab the minimum spacing of distribution reinforcement, should be four times the effective thickness of the slab or

a. 30 cm

b. 40 cm

c. 50 cm

d. 60 cm

24. Pick up the true statement from the following:

a. In the absence of beams, it is easier to install piping

b. In the absence of beams, it is easier to paint

c. A flat slab is capable to withstand concentrated loads

d. All the above.

25. In English garden wall bond

a. one course of headers to three or five course of stretchers

b. queen closer in provided in each heading course

c. the middle course of stretchers is started with a header to give proper vertical joints

d. all the above.

26. The vertical sides of a door and window openings provided in a wall, are known as

a. verticals

b. reveals

c. jambs

d. none of these.

27. Pick up the correct statement from the following:

a. Water cement paste hardens due to hydration

b. During hardening cement binds the aggregates together

c. Cement provides strength, durability and water tightness to the concrete

d. All the above.

28. To obtain cement dry powder, lime stones and shales or their slurry, is burnt in a rotary kiln at a temperature between

a. 1200° and 1300°C

b. 1300° and 1400°C

c. 1400° and 1500°C

d. 1500° and 1600°C

29. The joint used for joining the plain ends of cast iron pipes, is

a. socket and spigot joint .

b. dresser coupling joint

c. flexible joint

d. expansion joint.

30. To control the growth of algae in reservoirs, the compound which is used, is

a. copper sulphate

b. lime solution

c. alum solution

d. all the above.

31. Hume steel pipes are

a. R.C.C. pipes

b. steel shell coated from inside with cement mortar

c. steel shell coated from outside with cement mortar

d. both (c) and (d).

32. If D is the diameter of upper circular portion, the overall depth of a standard egg shaped section, is

a. 1.25 D

b. 1.5 D

c. 1.75 D

d. 2 D.

33. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

34. F.S.L. of a canal at its head with respect to parent channel is kept

a. at the same level

b. 15 cm lower

c. 15 cm higher

d. none of these.

35. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

36. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

37. If C is basic capacity per lane, V is velocity in km/hour, S is stopping distance pluslength of the vehicles in metres, the formula  is applicable to

a. district roads

b. two lane roads

c. two lane roads in one direction

d. none of these.

38. Minimum permissible speed on high speed roads, is decided on the basis of

a. 15 percentile cumulative frequency

b. 20 percentile cumulative frequency

c. 30 percentile cumulative frequency

d. 40 percentile cumulative frequency.

39. If L is the length of the transition curves provided on either side of a circular curve of radius R, the maximum angle of deflection with tangent for the junctions of the transition curve and circular curve, is

a. L/2R

b. L/3R

c. L/4R

d. none of these.

40. To prevent movement of moisture from subgrade to road pavement at the same level as that of water-table, thickness of a cut off layer of coarse sand, is

a. 15 cm

b. 20 cm

c. 30 cm

d. 45 cm

41. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

42. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

43. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

44. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

45. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

46. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

47. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

48. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

49. For Class A Air port the difference of reduced levels of higher and lower edges of the conical surface, is

a. 25 m

b. 50 m

c. 75 m

d. 100 m

50. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.