***Sub Engineer***

1. Profile levelling is usually done for determining

a. contours of an area

b. capacity of a reservoir

c. elevations along a straight line

d. boundaries of property

2. The main plate of a transit is divided into 1080 equal divisions. 60 divisions of the vernier coincide exactly with 59 divisions of the main plate. The transit can read angles accurate upto

a. 10"

b. 15"

c. 20"

d. 30"

3. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

4. ABCD is a rectangular plot of land. If the bearing of the side AB is 75°, the bearing of DC is

a. 75°

b. 255°

c. 105°

d. 285°

5. Porcelain is used as :

a. electric insulators

b. storage vessels

c. reactor chambers

d. All of the above.

6. Refractory bricks resist

a. high temperature

b. chemical action

c. dampness

d. all the above.

7. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

8. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

9. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3 M

d. depth d ∝

10. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

11. Every material obeys the Hooke's law within its

a. elastic limit

b. plastic point

c. limit of proportionality

d. none of these.

12. Pick up the correct statement from the following :

a. A ductile material has large plastic zone

b. A brittle material has no plastic zone

c. A rigid material has no plastic zone

d. All the above.

13. An independent mass of a fluid does not posses

a. elevation energy

b. kinetic energy

c. pressure energy

d. none of these.

14. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

15. The radius of gyration of the water line of a floating ship is 4 m and its metacentric height is 72.5 cm. The period of oscillation of the ship, is

a. 2π

b. 3π

c. 4π

d. π/2

16. Frictional loss of head includes the loss of energy due to

a. viscosity

b. turbulence

c. both (a) and (b)

d. none of these.

17. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

18. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

19. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

20. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

21. If the tendon is placed at an eccentricity e below the centroidal axis of the lon-gitudial axis of a rectangular beam (sectional modulus Z and stressed load P in tendon) the stress at the extreme top edge

a. is increased by

b. is increased by

c. is decreased by

d. remains unchanged.

22. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

23. If the maximum shear stress at the end of a simply supported R.C.C. beam of 16 m effective span is 10 kg/cm2, the length of the beam having nominal reinforcement, is

a. 4 cm

b. 6 m

c. 8 m

d. 10 m

24. For stairs spanning horizontally, the minimum waist provided is

a. 6 cm

b. 8 cm

c. 10 cm

d. 12 cm.

25. Which of the following metal sheets is most effective in preventing dampness ?

a. Copper sheets

b. lead sheets

c. aluminium sheets

d. all the above.

26. Pick up the consideration to the taken while designing a hospital from the following :

a. the operation theatre unit to be detached as it requires sterlized zone but near the ward for the patients and doctor

b. the mortuary should be detached from the main circulation with a postmortem room

c. causalty unit should be provided a separate entrance

d. all the above.

27. Separation of water or water sand cement from a freshly concrete, is known

a. bleeding

b. creeping

c. segregation

d. flooding

28. Concrete is unsuitable for compaction by a vibrator if it is

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

29. While determining the yield of open wells by the pumping test

a. velocity of recharging water, increases with depression head

b. depression head resulting at critical velocity, is called critical depression head

c. working head is generally limited to  rd of the critical depression head

d. maximum safe yield of an open well, is expected at critical depression head.

30. Mostly used coagulant, is

a. chlorine

b. alum

c. lime

d. bleaching powder.

31. Oxidation process results in the formation of

a. carbon dioxide

b. nitrates

c. sulphates

d. all the above.

32. To maintain aerobic biological activity, the moisture content of the compost mass should be about

a. 45%

b. 50%

c. 55%

d. 60%.

33. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

34. For standing crops in undulating sandy fields, the best method of irrigation, is

a. sprinkler irrigation

b. free flooding

c. check method

d. furrow method

35. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

36. Solution of Laplacian equation in three dimensions  of water in a syphon, is done by

a. analytical method

b. Khosla's method

c. method of relaxation

d. Unwin's method.

37. The type of curves generally provided on highways, is

a. critical curve

b. transition curve

c. vertical curve

d. all the above.

38. On a pavement with parabolic camber, the angle of inclination of the vehicles will be

a. less at the edges

b. more at the edges

c. same at the edges as well as at the crown

d. none of these.

39. In an ideal transition curve, the radius of curvature

a. is constant

b. at any point is directly proportional to its distance from the point of commencement

c. is inversely proportional to the radius of main curve

d. is directly proportional to the radius of main curve

40. In cement concrete pavements, tensile stress is due to :

a. bending or deflection under wheel loads

b. difference in temperature of the top and bottom of pavement

c. contraction of slab during falling temperature

d. all the above.

41. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

42. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

43. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

44. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

45. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

46. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

47. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

48. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m

50. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.