***Sub Engineer***

1. The curve composed of two arcs of different radii having their centres on the opposite side of the curve, is known

a. a simple curve

b. a compound curve

c. a reverse curve

d. a vertical curve.

2. The combined effect of curvature and refraction over a distance L kilometres is

a. 67.2 L2 mm

b. 76.3 L2 mm

c. 64.5 L2 mm

d. none of these.

3. The slope correction for a 3° slope for a length of 100 m, is

a. -0.11 m

b. -0.12 m

c. -1.87 m

d. -0.137 m

4. While setting a plane table at a station it was found that the error in centering was 30 cm away from the ray of length 40 m drawn from the station. If the scale of the plan is 1 cm = 2 cm, the displacement of the end of the ray in plan from the true position will be

a. 0.02 cm

b. 0.15 cm

c. 02 cm

d. 0.1 cm

5. Lacquer is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

6. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

7. The steel which contains fissures and cavities, is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

8. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

9. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

10. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

11. If all the dimensions of a bar are increased in the proportion n : 1, the proportion with which the maximum stress produced in the prismatic bar by its own weight, will increase in the ratio

a. n : 1

b. 1 :

c.  : 1

d. 1 : n.

12. The ratio of the tensile stress developed in the wall of a boiler in the circumferential direction to the tensile stress in the axial direction, is

a. 4

b. 3

c. 2

d. 1

13. The dimensions MLT-2 refers to

a. specific weight

b. force

c. discharge

d. none of these.

14. Barometres are used to measure

a. difference in pressure at two points

b. atmospheric pressure

c. very low pressure

d. very high pressure.

15. In steady flow, which one of the following changes with time

a. velocity

b. pressure

c. density

d. none of these.

16. Practical fluids possess

a. viscosity

b. surface tension

c. compressibility

d. all the above.

17. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

18. The effective size of particles of soil is denoted by

a. D10

b. D20

c. D30

d. D60

19. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

20. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

21. The Young's modulus of elasticity of steel, is

a. 150 KN/mm2

b. 200 KN/mm2

c. 250 KN/mm2

d. 275 KN/mm2

22. According to load factor method, the permissible load W on a short column reinforced with longitudinal bars and lateral stirrups, is

a. Stress in concrete x area of concrete

b. Stress in steel x area of steel

c. Stress in concrete x area of concrete + Stress in steel x area of steel

d. None of these.

23. For a ribbed slab

a. clear spacing between ribs shall not be greater than 4.5 cm

b. width of the rib shall not be less than 7.5 cm

c. overall depth of the slab shall not exceed four times the breadth of the rib

d. all the above.

24. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

25. The minimum width of a stair in residential buildings, is

a. 70 cm

b. 85 cm

c. 100 cm

d. 120 cm.

26. While investigating the site, a thick layer of fairly firm clay over a deep layer of soft clay is encountered. In such a situation, the following type of foundation is useful :

a. pile formation

b. raft foundation

c. grillage foundation

d. none of these.

27. The grade of concrete not recommended by I.S. : 456, is

a. M 200

b. M 300

c. M 400

d. M 500

28. Addition of pozzolana to cement causes

a. loss of heat of hydration

b. reduction in bleeding

c. increase in curing time

d. all the above.

29. Pickup the incorrect statement from the following :

a. The invert of pressure conduit is independent of the grade of the hydraulic gradient line

b. The pressure conduits may be taken uphill upto a maximum height of 8.3 m

c. Aqueducts and tunnels sections are generally kept circular

d. None of these.

30. The diameter of pipes in bath rooms and lavatories in domestic water supply, is

a. 6 mm

b. 12 mm

c. 18 mm

d. 24 mm.

31. For laying a sewer line in a trench of 2 m width, an offset line is marked on the ground parallel to the given centre line at a distance of

a. 120 cm

b. 140 cm

c. 160 cm

d. 180 cm.

32. Which one of the following resists hydrogen sulphide corrosion

a. R.C.C.

b. Glazed stone wares

c. Asbestos cement

d. Glazed ware.

33. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

34. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

35. According to Bligh's creep theory, percolating water flows along

a. straight path under the foundation of the dam

b. circular path under the foundation of the dam

c. the outline of the base of the foundation of the dam

d. none of these.

36. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

37. Minimum stopping distance for moving vehicles on road with a design speed of 80 km/hour, is

a. 100 m

b. 120 m

c. 150 m

d. 200 m

38. If the rate of change of the super-elevation along a curved portion of a 7 metre wide road is 1 in 150 and the maximum super-elevation allowed is 1 in 15, the maximum length of the transition curve to be provided at either end, is

a. 70 m

b. 75 m

c. 80 m

d. 90 m

39. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

40. If cross slope of a country is greater than 60%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

41. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

42. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

43. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

44. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

45. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

46. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

47. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

48. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

49. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.

50. The threshold markings are

a. 1 m clear space between adjacents

b. 45 m in length

c. placed symmetrically on either side of the runway centre line

d. all the above.