***Sub Engineer***

1. True meridian of different places

a. converge from the south pole to the north pole

b. converge from the north pole to the south pole

c. converge from the equator to the poles

d. run parallel to each other.

2. For taking offsets with an optical square on the right hand side of the chain line, it is held

a. by right hand upside down

b. by left hand upright

c. by right hand upright

d. by left hand up side down.

3. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

4. The radius of a simple circular curve is 300 m and length of its specified chord is 30 m. The degree of the curve is

a. 5.73°

b. 5.37°

c. 3.57°

d. 3.75°.

5. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

6. Age of a tree may be ascertained by

a. radius of its stem

b. circumference of its stem

c. number of branches

d. number of annual rings.

7. A 1st class brick immersed in water for 24 hours, should not absorb water (by weight) more than

a. 15%

b. 20%

c. 25%

d. 5%.

8. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

9. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

10. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

11. When two plates butt together and are riveted with two cover plates with two rows of rivets, the joint is known as

a. lap joint

b. butt join

c. single riveted single cover butt joint

d. double riveted double cover butt joint.

12. The stress at which extension of a material takes place more quickly as compared to the increase in load, is called

a. elastic point

b. plastic point

c. breaking point

d. yielding point.

13. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

14. For exerting a pressure of 4.8 kg/cm2, the depth of oil (specific gravity 0.8), should be

a. 41 cm

b. 56 cm

c. 60 cm

d. 76 cm.

15. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

16. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

17. A suspended particle falls through a height H cm in water in t minutes. If the viscosity of water is η and specific gravity of the particle is G, the diameter of the particle is (where M is a constant factor)

a. 103M

b. 104M

c. 105M

d. 102M

18. In a liquid limit test, the moisture content at 10 blows was 70% and that at 100 blows was 20%. The liquid limit of the soil, is

a. 35%

b. 50%

c. 65%

d. none of these.

19. Plasticity index is defined as the range of water content between

a. liquid and plastic limit

b. plastic limit and semi solid limit

c. semi-solid limit and liquid limit

d. liquid limit and solid limit.

20. A clay subjected to pressure in excess to its present over-burden, is said to be

a. pre-compressed

b. pre-consolidated

c. over-consolidated

d. all the above.

21. For normal cases, stiffness of a simply supported beam is satisfied if the ratio of its span to its overall depth does not exceed

a. 15

b. 20

c. 25

d. 30

22. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

23. A singly reinforced concrete beam of 25 cm width and 70 cm effective depth is provided with 18.75 cm2 steel. If the modular ratio (m) is 15, the depth of the neutral axis, is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

24. As per I.S. 456 - 1978, the pH value of water shall be

a. less than 6

b. equal to 6

c. not less than 6

d. equal to 7

25. The type of stone masonry in which stones of same height are laid in layers, is called

a. random rubble masonry

b. course rubble masonry

c. uncoursed rubble masonry

d. ashlar masonry.

26. Pick up the correct statements from the following :

a. cracks appear on the plastered surface in the form of hair cracks

b. in brick work, the efflorescence is removed by applying a solution of zinc sulphate and water

c. excessive thermal variations in the backing or plaster causes the plaster to fall

d. all the above.

27. For compacting plain concrete road surface of thickness less than 20 cm, we use

a. internal vibrator

b. screed vibrator

c. form vibrator

d. none of these.

28. For the construction of thin R.C.C. structures, the type of cement to be avoided, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. sulphate resisting cement.

29. Aqueducts are generally designed

a. circular

b. rectangular

c. horse shoe section

d. all the above.

30. The main draw-back of centrifugal pump, is

a. discharge from pump varies with the load of water

b. for high heads, efficiency is low up to 50%

c. the suction lift is limited to 6 m

d. all the above.

31. The sewage discharge in a detritus tank of a treatment plant is 576 litres/sec with flow velocity of 0.2 m/sec. If the ratio of width to depth is 2, the depth is

a. 100 cm

b. 110 cm

c. 120 cm

d. 150 cm.

32. The temperature affects the

a. biological activity of bacterias in sewage

b. solubility of gases in sewage

c. viscosity of sewage

d. all the above.

33. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

34. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

35. The top of the capillary zone

a. lies below the water table at every point

b. lies above the water table at every point

c. coincides the water table at every point

d. none of these.

36. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

37. Maximum number of passenger cars that can pass a given point on a road during one hour under the most ideal road way and traffic conditions, is known as

a. traffic density

b. basic capacity of traffic lane

c. possible capacity of traffic lane

d. all the above.

38. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

39. The absolute minimum radius of horizontal curve for a design speed 60 km ph is

a. 131 m

b. 210 m

c. 360 m

d. none of these.

40. When each particle of aggregates is thinly coated with cement paste, a heterogeneous solid is formed, which is known as

a. hydration

b. gel

c. concrete

d. none of these.

41. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

42. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

43. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

44. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

45. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

46. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

47. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

48. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

49. The depressions and undulations in the pavement, are caused due to

a. impact of heavy wheel loads

b. punching effect

c. flexible gears

d. all the above.

50. Total correction for elevation, temperature and gradient for a runway should not be more than

a. 20%

b. 25%

c. 30%

d. 35%