***Sub Engineer***

1. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

2. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

3. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

4. Pick up the correct statement from the following :

a. a back sight is the first sight taken after setting up the instrument in any position

b. the first sight on each change point is a fore sight

c. the second sight on each change point is a back sight

d. all the above.

5. Pick up the correct statement from the following:

a. The commercial plywoods are available upto 150 cm wide and upto 300 cm long

b. The plywoods possess uniform tensile strength in all directions

c. The plywoods are light in weight

d. All the above.

6. Knots in timber are

a. defects caused by crushing fibres

b. splits radiating from the centre

c. speckled strains

d. signs of branches cut off.

7. Cast iron

a. is manufactured in required shapes

b. may contain 2 to 5 per cent of carbon with other impurities

c. is remelted in a cupola furnace

d. all the above.

8. Upto a maximum of 72% of iron, is available in

a. Magnetite

b. Red haemetite

c. Limonite

d. Siderite

9. Pick up the correct statement from the following:

a. In a loaded beam, the moment at which the entire section of the beam becomes fully plastic, is called plastic moment

b. In a fully plastic stage of the beam, the neutral axis divides the section in two sections of equal area

c. The ratio of plastic moment to the yield moment, is called shape factor

d. All the above.

10. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1: 1 ½: 2

b. 2: 1 ½: 1

c. 1 : 1 : 2

d. none of these.

11. The shear force on a simply supported beam is proportional to

a. sum of the forces

b. sum of the transverse forces

c. algebraic sum of the transverse forces of the section

d. curvature of the neutral axis.

12. At either end of a plane frame, maximum number of possible transverse shear forces, are

a. one

b. two

c. three

d. four

13. A piezometer opening in pipes measures

a. velocity head

b. static pressure

c. total pressure

d. negative static pressure.

14. The length of hydraulic jump is roughly

a. 2 to 3 times its height

b. 3 to 5 times its height

c. 5 to 7 times its height

d. None of these.

15. The continuity equation

a. expresses the relationship between work and energy

b. relates the momentum per unit volume between two points on a stream line

c. relates mass rate of flow along a stream line

d. requires that Newton's second law of motion be satisfied at every point in fluid.

16. A rectangular channel 6 m wide and 3 m deep and having a bed slope as 1 in 2000 is running full. If Chezy's constant C = 54.8, pick up the correct specification of the channel from the following :

a. hydraulic mean depth = 1.5 m

b. Velocity of flow = 1.5 m/sec

c. Rate of flow = 27 m3/sec

d. All the above.

17. Plasticity index is defined as the range of water content between

a. liquid and plastic limit

b. plastic limit and semi solid limit

c. semi-solid limit and liquid limit

d. liquid limit and solid limit.

18. Buoyant unit weight equals the saturated density

a. multiplied by unit weight of water

b. divided by unit weight of water

c. plus unit weight of water

d. minus unit weight of water.

19. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

20. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

21. A ribbed slab is provided for

a. a plain ceiling

b. thermal insulation

c. acoustic insulation

d. all the above.

22. In testing a pile by load test, pile platform is loaded with one and half times the design load and a maximum settlement is noted. The load is gradually removed and the consequent rebound is measured. For a safe pile, the net settlement (i.e. total settlement minus rebound) per tonne of test load should not exceed

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm

23. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

24. Distribution of shear intensity over a rectangular section of a beam, follows :

a. a straight line

b. a parabolic curve

c. an elliptical curve

d. none of these.

25. In soft clay of low bearing capacity, the type of steel pile generally used, is

a. screw pile

b. disc pile

c. pipe pile

d. raking pile.

26. Pick up the correct statement from the following :

a. inclined borings are made for taking samples under existing structures

b. inclined borings are occasionally used instead of vertical holes.

c. the spacing of inclined borings is kept such that one bore hole is vertically above the bottom of an adjacent bore hole.

d. all the above.

27. The following proportion of the ingredients of concrete mix, is not in conformation to arbitrary method of proportioning

a. 1 : 2 : 4

b. 1 : 3 : 6

c. 1 : 2 : 8

d. 1 : 4 : 10

28. Sand generally contains salt if it is obtained from:

a. river beds

b. sea beds

c. all the above

d. none of these.

29. For determining the velocity of flow of underground water, the most commonly used non-empirical formula is

a. Darcy's formula

b. Slichter's formula

c. Hazen's formula

d. Lacy's formula.

30. For complete stabilisation of organic matter in polluted water, it takes

a. 10 days

b. 20 days

c. 30 days

d. 40 days.

31. Imhoff cone is used to measure

a. total organic solids

b. total solids

c. total in organic solids

d. settleable solids.

32. If the side of a square sewer is 1000 mm, the diameter of a hydraulically equivalent circular section, is

a. 1045 mm

b. 1065 mm

c. 1075 mm

d. 1095 mm.

33. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

34. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

35. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

36. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

37. Customers prefer parking of their vehicles at

a. 85° to aisles

b. 80° to aisles

c. 75° to aisles

d. 60° to aisles

38. While designing hair pin bends on higways, the minimum

a. gradient is 1 in 40

b. gradient is 1 in 200

c. superelevations is 1 in 10

d. All the above.

39. Curves in the same direction separated by short tangents, are called

a. compound curves

b. transition curves

c. broken-back curves.

d. None of these.

40. Full amount of extra width of a pavement on the curve, is provided at

a. beginning of the transition curve

b. centre of the transition curve

c. beginning of the circular curve

d. centre of the circular curve.

41. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

42. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

43. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

44. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

45. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

46. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

47. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

48. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

49. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.

50. International Civil Aviation Organisation (I.C.A.O.) was set up at Montreal (Canada), in

a. 1929

b. 1939

c. 1947

d. 1950